RBdualVGA v0.92 	 Copyright 1996 Ralf Brown

[Note: this documentation is very preliminary, and the software is a

pre-release version]

What is RBdualVGA?  It is a set of programs which permit the use of two

color SuperVGA displays on a single PC (which is ordinarily not supported).

Contents

========

System Requirements

Files

Installation

Limitations

Known Problems

Source Code

Contact Information

System Requirements

===================

DOS 3.0 or higher

DESQview 2.26 or higher (if using DESQview)

TWO PCI-bus SuperVGA cards; they should be as similar as possible

1100 bytes memory for TSR

Files

=====

Permission is granted to redistribute this package, provided that all files

are distributed together, unmodified.

Files which comprise RBdualVGA:

DUALVGA.DOC        this documentation

INITDUAL.SYS	     initialize second monitor at boot time

DUALVGA.COM        screen blanker and switcher TSR

MONITOR2.COM       run specified program on second monitor

DO-PIF.DVP	     sample DESQview .DVP

DO-SCRIP.SCR	     sample DESQview Learn script

Installation

============

1. Add INITDUAL to your CONFIG.SYS by inserting a line like

DEVICE=C:\INITDUAL.SYS

This DEVICE= line should be as early in CONFIG.SYS as possible, and

*must* precede any memory manager which remaps ROMs (i.e. QEMM with

Stealth enabled).	If you are using DOSMAX or QEMM's DOS-UP, the call

to INITDUAL should follow the DOSDATA.SYS line to avoid being

executed twice.

INITDUAL.EXE runs as a device driver, but does not remain resident, and

therefore does not consume any memory.

A sample CONFIG.SYS fragment:

DEVICE=C:\QEMM\DOSDATA.SYS

DEVICE=C:\BOOT\INITDUAL.SYS

DEVICE=C:\QEMM\QEMM386.SYS RAM ST:M XBDA:H R:1 DMA=64 SRBP:N

device=c:\qemm\dos-up.sys @c:\qemm\dos-up.dat

DEVICE=C:\BOOT\XMSDISK.SYS 2560K

...

SHELL=C:\4DOS\4DOS.COM C:\4DOS @C:\4DOS\4DOS.INI /P

2. Add DUALVGA to your AUTOEXEC.BAT by inserting a line like

DUALVGA I 2 12 15 20

This command installs the screen blanker and switcher TSR and tells

it to blank the screen after two minutes of inactivity, then select

the DPMS power modes "standby", "suspend", and "off" after 12, 15,

and 20 minutes, respectively.  The monitor which is active at the

time DUALVGA is installed becomes the primary monitor, and the other

monitor becomes the secondary monitor.  Which monitor is active at

boot time depends on the order in which your system's BIOS scans

adapters; if the monitor which you wish to be the primary monitor

is not active at boot time, you will need to rearrange your video

cards.

Once DUALVGA has been installed, you may switch the active display

at any time by pressing RightShift-RightCtrl-F12.	You may also

cause an immediate switch into low-power mode by pressing

RightShift-RightCtrl-F11; on pressing this keystroke, both

screens are immediately blanked and then placed into VESA "off"

mode a second later; unblanking is disabled for three seconds to

give you time to release the keys.

3. Add MONITOR2 to batch files and DESQview .DVP files as appropriate.

Simply precede any line in a batch file which is to execute on the

secondary monitor with MONITOR2, i.e.

MONITOR2 edit foo.bar

For a DESQview .DVP, set the program to execute to MONITOR2.COM, and

set the program arguments to the actual program and arguments you

wish to execute on the secondary monitor.	For example,

Program...: c:\dv\monitor2.com

Parameters: c:\4dos\4dos.com

You may also want to add the startup script from DO-SCRIP.SCR to the

Learn scripts for the DESQview .DVP is you normally run your screen

at other than 80x25, since running MONITOR2 tends to reset the screen

to that mode.

Limitations

===========

DUALVGA assumes that a single copy of the video BIOS can drive either

video board, so both boards need to use the same chipset, and otherwise

be as nearly identical as possible.

Known Problems

==============

MONITOR2 v0.9 is not able to intercept DESQview's cursor emulation in

graphical modes (30-line or 60-line text display on the Rearrange/Video

Options menu).  The result is occasional disruption of graphics or even

text fonts on the secondary monitor, and sporadic cursor remnants on

the primary monitor if the program on the secondary display is running

in the background.  Neither problem appears when the primary monitor

is in a true text mode, a graphical program is running full-screen

on the primary monitor, or the secondary display's program is suspended

(i.e. in the background when background execution is turned off).

Source Code

===========

Full source code will be included when v1.0 is released.

Contact Information

===================

Ralf Brown

813 Copeland Way Suite 26

Pittsburgh, PA 15232-2217

(valid until May 1, 1997)

ralf@pobox.com

---End of File---