DOS USB DRIVER PACKAGE INTRODUCTION

�����������������������������������

Bret E Johnson

January 30, 2010

TABLE OF CONTENTS

�����������������

INTRODUCTION  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  5

COPYRIGHTED FREEWARE  . . . . . . . . . . . . . . . . . . . . . . . .  7

USB PRIMER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  8

USB TERMS  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  8

DEVICES & INTERFACES . . . . . . . . . . . . . . . . . . . . . . .  9

Common Device . . . . . . . . . . . . . . . . . . . . . . . . .  9

Physical Device . . . . . . . . . . . . . . . . . . . . . . . . 10

Logical Device  . . . . . . . . . . . . . . . . . . . . . . . . 10

Device/Interface  . . . . . . . . . . . . . . . . . . . . . . . 11

Examples  . . . . . . . . . . . . . . . . . . . . . . . . . . . 12

Device/Interface Configuration  . . . . . . . . . . . . . . . . 13

Summary . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

USB HOST CONTROLLERS & HUBS  . . . . . . . . . . . . . . . . . . . 15

HARDWARE & SOFTWARE  . . . . . . . . . . . . . . . . . . . . . . . 15

VENDOR-SPECIFIC DEFINITIONS  . . . . . . . . . . . . . . . . . . . 16

HOST SOFTWARE  . . . . . . . . . . . . . . . . . . . . . . . . . . 17

DEVICE INITIALIZATION & ENUMERATION  . . . . . . . . . . . . . . . 18

DESCRIPTORS  . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

DISABLING & SUSPENDING PHYSICAL DEVICES  . . . . . . . . . . . . . 21

STARTING & STOPPING HOSTS  . . . . . . . . . . . . . . . . . . . . 24

POWER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 25

UHCI, OHCI, & EHCI . . . . . . . . . . . . . . . . . . . . . . . . 26

PCI BUS MASTERING  . . . . . . . . . . . . . . . . . . . . . . . . 28

ISOCHRONOUS, INTERRUPT, CONTROL, & BULK TRANSACTIONS . . . . . . . 30

PACKET SCHEDULING  . . . . . . . . . . . . . . . . . . . . . . . . 31

BUS TIMING . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

LEGACY SUPPORT . . . . . . . . . . . . . . . . . . . . . . . . . . 34

What is Legacy Support? . . . . . . . . . . . . . . . . . . . . 34

How is Legacy Support Implemented?  . . . . . . . . . . . . . . 36

PROGRAM ARCHITECTURE  . . . . . . . . . . . . . . . . . . . . . . . . 39

TSR vs. CONFIG.SYS . . . . . . . . . . . . . . . . . . . . . . . . 39

BACKGROUND vs. FOREGROUND  . . . . . . . . . . . . . . . . . . . . 40

MODULAR vs. MONOLITHIC . . . . . . . . . . . . . . . . . . . . . . 40

PLUG-AND-PLAY vs. STATIC . . . . . . . . . . . . . . . . . . . . . 41

PROTECTED MODE vs. REAL MODE . . . . . . . . . . . . . . . . . . . 41

LEGACY EMULATION vs. NEW PROGRAM DEVELOPMENT . . . . . . . . . . . 42

DOS USB DRIVER INTRODUCTION            Page 1 of 186

THE DOS USB API  . . . . . . . . . . . . . . . . . . . . . . . . . 43

DRIVER, SUPPORT, & TEST PROGRAMS . . . . . . . . . . . . . . . . . 44

THE DOS PATH . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

GENERAL OPTION SWITCHES . . . . . . . . . . . . . . . . . . . . . . . 47

ALIASES  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 47

INSTALLATION INTO LOW MEMORY . . . . . . . . . . . . . . . . . . . 47

UNINSTALL FROM MEMORY  . . . . . . . . . . . . . . . . . . . . . . 48

"BAD" DEVICES  . . . . . . . . . . . . . . . . . . . . . . . . . . 48

VIEWING DESCRIPTORS  . . . . . . . . . . . . . . . . . . . . . . . 50

ENVIRONMENT VARIABLES & REDIRECTED INPUT . . . . . . . . . . . . . 51

INDIVIDUAL PROGRAMS . . . . . . . . . . . . . . . . . . . . . . . . . 52

USBUHCI & USBUHCIL . . . . . . . . . . . . . . . . . . . . . . . . 53

PCI Search Index  . . . . . . . . . . . . . . . . . . . . . . . 54

Memory Usage  . . . . . . . . . . . . . . . . . . . . . . . . . 55

IRQ & I/O Resources . . . . . . . . . . . . . . . . . . . . . . 56

Other Host Controller Drivers . . . . . . . . . . . . . . . . . 58

Disabling the Companion EHCI Host Controller  . . . . . . . . . 59

Debugging Program Installation  . . . . . . . . . . . . . . . . 61

Postponing Device Enumeration . . . . . . . . . . . . . . . . . 63

Bus Timing Manipulation . . . . . . . . . . . . . . . . . . . . 64

Device & Host Manipulation  . . . . . . . . . . . . . . . . . . 64

Enable  . . . . . . . . . . . . . . . . . . . . . . . . . . . 65

Disable   . . . . . . . . . . . . . . . . . . . . . . . . . . 66

Suspend   . . . . . . . . . . . . . . . . . . . . . . . . . . 66

Resume  . . . . . . . . . . . . . . . . . . . . . . . . . . . 67

Remote Wakeup   . . . . . . . . . . . . . . . . . . . . . . . 68

Reset   . . . . . . . . . . . . . . . . . . . . . . . . . . . 68

Configuration Value   . . . . . . . . . . . . . . . . . . . . 69

PowerOn, PowerOff, & PowerReset   . . . . . . . . . . . . . . 69

Test Mode   . . . . . . . . . . . . . . . . . . . . . . . . . 71

Resetting "Bad" Devices . . . . . . . . . . . . . . . . . . . . 71

Debug Mode  . . . . . . . . . . . . . . . . . . . . . . . . . . 72

Attached Devices  . . . . . . . . . . . . . . . . . . . . . . . 73

New Device Notifications  . . . . . . . . . . . . . . . . . . . 74

Host Controller Hardware Status . . . . . . . . . . . . . . . . 75

Schedule Status . . . . . . . . . . . . . . . . . . . . . . . . 76

Frame List (FL)   . . . . . . . . . . . . . . . . . . . . . . 76

Transfer Descriptor (TD)  . . . . . . . . . . . . . . . . . . 77

Descriptors . . . . . . . . . . . . . . . . . . . . . . . . . . 79

Miscellaneous Notes . . . . . . . . . . . . . . . . . . . . . . 80

USBHOSTS . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 82

Showing Host Controller Status  . . . . . . . . . . . . . . . . 82

Disabling EHCI Host Controllers . . . . . . . . . . . . . . . . 83

USBDEVIC . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 84

IRQ  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 86

USBHUB . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 88

USBKEYB  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 90

Typing Methods  . . . . . . . . . . . . . . . . . . . . . . . . 90

Keyboard Mapping  . . . . . . . . . . . . . . . . . . . . . . . 93

DOS USB DRIVER INTRODUCTION            Page 2 of 186

Translating USB Key Codes into Scan Codes   . . . . . . . . . 94

Translating USB System Codes into Scan Codes  . . . . . . . . 96

Setting Up a Keyboard Map   . . . . . . . . . . . . . . . . . 97

Miscellaneous Notes about Keyboard Mapping  . . . . . . . . . 99

Processing Special Keys . . . . . . . . . . . . . . . . . . .  100

Changing the Typeamatic Rate  . . . . . . . . . . . . . . . .  101

Enabling & Disabling Keyboards  . . . . . . . . . . . . . . .  102

Descriptors . . . . . . . . . . . . . . . . . . . . . . . . .  103

Miscellaneous Notes . . . . . . . . . . . . . . . . . . . . .  104

SCANTEST . . . . . . . . . . . . . . . . . . . . . . . . . . . .  105

USBMOUSE . . . . . . . . . . . . . . . . . . . . . . . . . . . .  107

DOS Mouse Drivers . . . . . . . . . . . . . . . . . . . . . .  107

PS2 Mouse BIOS  . . . . . . . . . . . . . . . . . . . . . . .  109

IntelliMouse Protocols  . . . . . . . . . . . . . . . . . . .  110

PS2 Mouse Hardware Emulation  . . . . . . . . . . . . . . . .  111

Hardware Emulation Methods  . . . . . . . . . . . . . . . .  113

Enabling & Disabling Mice . . . . . . . . . . . . . . . . . .  114

Exchanging Left & Right Buttons . . . . . . . . . . . . . . .  114

Descriptors . . . . . . . . . . . . . . . . . . . . . . . . .  116

Displaying Raw Mouse Data . . . . . . . . . . . . . . . . . .  116

Controlling Mouse Sensitivity . . . . . . . . . . . . . . . .  117

Resetting the PS2 Mouse . . . . . . . . . . . . . . . . . . .  119

Miscellaneous Notes . . . . . . . . . . . . . . . . . . . . .  119

PS2MTEST . . . . . . . . . . . . . . . . . . . . . . . . . . . .  121

USBJSTIK . . . . . . . . . . . . . . . . . . . . . . . . . . . .  123

Joystick Protocols  . . . . . . . . . . . . . . . . . . . . .  124

Analog Joystick Entity Names  . . . . . . . . . . . . . . . .  125

USB Joystick Control Names  . . . . . . . . . . . . . . . . .  127

Joystick Controls Mapping . . . . . . . . . . . . . . . . . .  129

Special Notes About Controls Mapping  . . . . . . . . . . .  131

Analog <-> Digital Conversions  . . . . . . . . . . . . . . .  133

Normalizing Axis Entities . . . . . . . . . . . . . . . . . .  134

I/O Virtualization  . . . . . . . . . . . . . . . . . . . . .  135

Descriptors . . . . . . . . . . . . . . . . . . . . . . . . .  136

Displaying Raw Joystick Data  . . . . . . . . . . . . . . . .  137

Miscellaneous Notes . . . . . . . . . . . . . . . . . . . . .  138

THRUST . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  139

USBDRIVE . . . . . . . . . . . . . . . . . . . . . . . . . . . .  141

Devices, LUNs, and Drive Letters  . . . . . . . . . . . . . .  142

Device/Interfaces (Devices)   . . . . . . . . . . . . . . .  143

LUNs (Disks)  . . . . . . . . . . . . . . . . . . . . . . .  144

Drive Letters (Drives)  . . . . . . . . . . . . . . . . . .  145

Viewing USBDRIVE Status . . . . . . . . . . . . . . . . . . .  146

Assigning Drive Letters . . . . . . . . . . . . . . . . . . .  147

Current Directory Structure (CDS)   . . . . . . . . . . . .  147

FAT32 Partitions  . . . . . . . . . . . . . . . . . . . . .  149

Legacy Support for Disks  . . . . . . . . . . . . . . . . .  150

Assigning Physical Disk Numbers . . . . . . . . . . . . . . .  151

SCSI Inquiry Responses  . . . . . . . . . . . . . . . . . . .  152

Partitioning & Formatting . . . . . . . . . . . . . . . . . .  152

Low-Level Formatting  . . . . . . . . . . . . . . . . . . . .  153

DOS USB DRIVER INTRODUCTION            Page 3 of 186

Resetting Disks . . . . . . . . . . . . . . . . . . . . . . .  155

Troubleshooting . . . . . . . . . . . . . . . . . . . . . . .  155

Bad Devices   . . . . . . . . . . . . . . . . . . . . . . .  156

Internal Buffers  . . . . . . . . . . . . . . . . . . . . .  156

SCSI Error Codes  . . . . . . . . . . . . . . . . . . . . .  157

Delaying Initialization   . . . . . . . . . . . . . . . . .  157

Sector Transfer Size  . . . . . . . . . . . . . . . . . . .  158

Using USBASPI   . . . . . . . . . . . . . . . . . . . . . .  160

Sandisk U3 Software   . . . . . . . . . . . . . . . . . . .  160

Security Software   . . . . . . . . . . . . . . . . . . . .  161

Wear Leveling   . . . . . . . . . . . . . . . . . . . . . .  161

Sector Size   . . . . . . . . . . . . . . . . . . . . . . .  162

Miscellaneous Notes . . . . . . . . . . . . . . . . . . . . .  162

DRIVES . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  164

USBPRINT . . . . . . . . . . . . . . . . . . . . . . . . . . . .  165

Printer BIOS  . . . . . . . . . . . . . . . . . . . . . . . .  167

Reassigning LPT Ports . . . . . . . . . . . . . . . . . . . .  170

Printer Device ID's . . . . . . . . . . . . . . . . . . . . .  170

Resetting a Printer . . . . . . . . . . . . . . . . . . . . .  171

Epson Printer Initialization  . . . . . . . . . . . . . . . .  171

Copying Files to a Printer  . . . . . . . . . . . . . . . . .  172

The CPU BOUND Bug . . . . . . . . . . . . . . . . . . . . . .  173

Miscellaneous Notes . . . . . . . . . . . . . . . . . . . . .  174

BOUNDTST . . . . . . . . . . . . . . . . . . . . . . . . . . . .  175

INKLEVEL . . . . . . . . . . . . . . . . . . . . . . . . . . . .  176

USBSUPT1 . . . . . . . . . . . . . . . . . . . . . . . . . . . .  178

HIDSUPT1 . . . . . . . . . . . . . . . . . . . . . . . . . . . .  180

UNI2ASCI . . . . . . . . . . . . . . . . . . . . . . . . . . . .  181

VENDORID . . . . . . . . . . . . . . . . . . . . . . . . . . . .  184

DOS USB DRIVER INTRODUCTION            Page 4 of 186

INTRODUCTION

������������

Not DPMI as such, but multi-taskers that cause problems.

Removed Disable/Enable from USBPRINT (never there).

This was intended to be a "short and sweet" document describing the

usage of the DOS USB drivers and associated utilities created by Bret

Johnson.  Though the intention was to be short, the reality is that it's

not very short at all -- there are just too many details that need to be

discussed.  These programs are still in a testing (alpha/beta) stage, so

there may be errors, omissions, idiosynchrocies, and inconsistencies

with them.

None of the programs have official, complete documentation yet.  Final

documents will be produced when they are ready to be officially

released.

One of the main purposes of this document is to give a brief description

of each program, yet to include details on their major options, and on

the interactions between the different programs.  This document should

provide enough detail for someone with a technical DOS background to be

able to install, test, and use the programs.  However, it will not

discuss every option for each program.  It is assumed that the user has

a basic understanding of how DOS command-line utilities work, and how to

request basic help from a program by using the "/?" switch.

Undoubtedly, this document will not answer every possible question or

problem that will arise, so queries can be addressed to:

Bret Johnson

bretjohn@juno.com

http://bretjohnson.us

505-994-3023 (New Mexico, USA)

The web site also has a Forum, which is probably the best place to ask

about the programs:

http://bretjohnson.us/forum

By posting questions there, other people can see what you're asking and

suggest answers.  Inevitably, if one person has a question, there are

many others with that same question.  This way, everyone can learn from

everyone else.

Another major purpose of this document is to introduce some of the

concepts involved in the USB architecture, as well as the design

architecture of the included USB programs.  USB architecture is

EXTREMELY complicated, and some of that complexity needs to be explained

before a user can understand why some of the programs are designed the

DOS USB DRIVER INTRODUCTION            Page 5 of 186

way they are, what some of the different terms mean, and how all of the

pieces fit together.  It is intended that this document contains enough

information so that the user will not need to pursue outside

documentation to any great degree.

DOS USB DRIVER INTRODUCTION            Page 6 of 186

COPYRIGHTED FREEWARE

��������������������

The accompanying programs are authored by Bret Johnson with suggestions,

programming, documentation, and testing help from Richard Bonner

(http://www.chebucto.ns.ca/~ak621).

This section contains the license for all of the enclosed programs.  I'm

not going to inject a bunch of legal language in here that a smart

lawyer can find a loophole in.  I'm writing down the intent of how I

want the programs to be distributed and used in my own words, and hope

that the intent is clear, no matter how the literal words can be twisted

to mean different things (remember when President Bill Clinton, a

lawyer, claimed he didn't understand the meaning of "is" during the

Lewinsky hearings?).

All of these programs, as well as their documentation and source code,

are freely available to anyone who wants them.  You can use the programs

without restriction, but you cannot directly or indirectly use the

executable programs, documentation, or source code to create or

distribute new programs that are not also freely available.  You also

cannot distribute the programs, documentation, or source code and charge

(even indirectly) for their distribution.  You can charge someone enough

to cover your actual, direct costs for distribution (disks, shipping

materials, postage, etc.), but cannot charge for "handling".  This also

means that you cannot distribute the programs, documentation, or source

code directly from a web site that charges a "registration fee" in an

attempt to make a profit or to recover direct or indirect costs for

maintaining the web site.

If you distribute related or derivative programs, you must use

essentially the same license.  You must provide and distribute the

programs and documentation for free, and you must include the

documentation with the program.  You must give credit where it is due,

and must make the source code freely available to anyone who wants it.

Perhaps a good test of whether your program is adhering to the license

or not would be if you are willing to let me post the program,

documentation, and source code on my web site, to be downloaded by

anyone who wants it.  In fact, if you create any new related or

ancillary DOS USB programs, I would like the opportunity to distribute

them from my web site.  I won't guarantee that I will necessarily do it,

but would like the opportunity.  Distribution from my web site

(http://bretjohnson.us) is not required by the license.

DOS USB DRIVER INTRODUCTION            Page 7 of 186

USB PRIMER

����������

The USB architecture is EXTREMELY complicated.  It's way too complicated

to explain in detail -- trust me, you'd end up falling asleep many times

before you got done.  This particular section of the document attempts

to explain some basic concepts about USB.  The concepts and details

described here are not intended to explain everything there is to know

about USB (in fact, that would be impossible -- nobody knows everything

there is to know about USB).  Rather, this section is simply intended to

explain enough about the USB architecture that you can understand what

the attached programs are doing (and not doing) and get a sense of how

all the different pieces fit together.  If some major concept is still

baffling to you or isn't explained clearly enough, write me (preferably

using the Forum available at http://bretjohnson.us/forum) and somebody

will try to explain it.

USB TERMS

���������

As you are reading this document and using the various DOS USB Driver

programs, you may see many different terms used from time to time and

may not understand exactly what they mean.  Here is a non-comprehensive

list of some terms you will run across at various times as you are

"messing around" with USB.  If you already have a pretty good

understanding of what these mean (in a USB context), you probably have a

pretty good working knowledge of USB:

Host

Descriptor

Report

Hub

Port

Device

Configuration

Interface

Alternate Interface

End Point

Class

SubClass

Protocol

Packet

Schedule

Frame

Queue

Control

Bulk

Interrupt

Isochronous

Transaction

DOS USB DRIVER INTRODUCTION            Page 8 of 186

Legacy

In this document, we do not provide a condensed Glossary of these terms

and their definitions.  Rather, we will use many of these terms (and

others) throughout this document, and define them in a specific context.

Some of these terms have more than one definition, depending on the

context.  In addition, some of these terms have a special meaning inside

the USB architecture, which may not necessarily correspond to your

common understanding of the term.

This document is not intended to give a detailed description of USB, but

rather just to give enough of an overview that you can successfully use

the DOS USB Drivers.  If there's something that you come across that you

don't understand, do some studying and ask some questions, and you may

be amazed at what you find out.

DEVICES & INTERFACES

��������������������

I'm going to start off by attempting to explain one of the most

fundamental, and yet most confusing, concepts in USB: the concept of a

"Device".  In the USB architecture, there are multiple definitions and

concepts of what a "Device" is, and none of them necessarily correspond

to what you would commonly think of as a "Device".  It is pretty

critical that you can understand and differentiate between the multiple

definitions of a "Device", or when something goes wrong you won't be

able to troubleshoot and fix it.  You'll end up chasing your own tail

because you'll get bogged down is USB complexities and won't understand

what anything means.  Trust me, I've already struggled (and still

struggle) through this, and would like you to avoid struggling

unnecessarily.  Unfortunately, this is going to take quite a bit of

explaining, so please bear with me for awhile.

Common Device

�������������

In this document, we will differentiate between four different

definitions of "Device": Physical Device, Logical Device,

Device/Interface, and Common Device.  Common Device, in this

document, will correspond to what you as a computer user (not a USB

programmer) would commonly think of as a "Device".  For example, a

mouse, keyboard, camera, and multi-function printer (printer-scanner-

copier-fax) are each considered a single Common Device.

DOS USB DRIVER INTRODUCTION            Page 9 of 186

Physical Device

���������������

USB is designed to allow many different kinds of "things" to plug

into it (everything from mice to disk drives to video cameras to

ethernet ports).  These different "things" do not have much in

common, other than the fact that they need to send data back and

forth to the computer.  In this document, we will call the "thing"

that gets plugged into a single USB port (with a single USB cable or

connector) a Physical Device.

Logical Device

��������������

One of the marketing phrases that USB uses is that up to 127

"Devices" can be attached to a single USB bus.  The "Device" that is

being referred to here will be in this document called a Logical

Device.  A Logical Device is the "thing" that gets assigned a USB

Address by the software on the computer after a Physical Device gets

plugged into the bus.  The assigned Address is a number between 1 and

127, which is where the "magic" 127 number comes from.

When you first plug a Physical Device into a USB port, the bus simply

recognizes that a Physical Device got plugged in, but has no idea

what kind of "Device" it is (not at first, anyway).  The software on

the computer Enables the Physical Device, which in turn causes the

Logical Device ("inside" the Physical Device) to assume Address 0.

The Logical Device then goes through a lengthy and complicated

initialization & enumeration process (discussed later in this

document).  During the enumeration process, the Logical Device is

assigned a "real" Address (a number between 1 and 127).

Also as part of the enumeration process, the software on the computer

"asks" the Logical Device exactly what kind of "Device" it is.  The

Logical Device sends a bunch of data back to the computer, organized

into sub-groups called Descriptors.  One of those Descriptors is

called a Device Descriptor, which contains some basic information

about the Logical Device (USB version compatibility, manufacturer &

model number, etc.).  A Physical Device must have at least one Device

Descriptor "inside" it.  If a Physical Device has more than one

Device Descriptor "inside" it, USB refers to it as a Compound Device.

Each Device Descriptor corresponds to a Logical Device, and is

assigned a separate Address.

DOS USB DRIVER INTRODUCTION            Page 10 of 186

Device/Interface

����������������

Even after the Device Descriptor has been downloaded and an Address

has been assigned to a Logical Device, the software on the computer

still does not know what "kind" of Device it is.  USB uses a

complicated, hierarchial classification system to describe the

various "kinds" of Devices.

The top tier of this hierarchy is called the "Device", and is defined

by some of the data that is in the Device Descriptor (specifically,

the Device Class, Device SubClass, and Device Protocol).  USB calling

this hierarchy tier a "Device" is very unfortunate, however, because

it has absolutely nothing to do with what anybody would normally call

a "Device".  This is where most of the confusion regarding the term

"Device" comes from.

In addition to the Device Descriptor, there are also several other

Descriptors downloaded during the enumeration process.  One of these

is called an Interface Descriptor (and a Logical Device can have more

than one Interface Descriptor).  The second tier of the hierarchy is

called the "Interface", and is defined by some of the data that is in

the Interface Descriptor (specifically, the Interface Class,

Interface SubClass, and Interface Protocol).  A Logical Device

(Device Descriptor) must also have at least one Interface (Interface

Descriptor).  In some cases, the COMBINED six bytes of data from the

Device Descriptor and Interface Descriptor (Device Class, Device

SubClass, Device Protocol, Interface Class, Interface SubClass, and

Interface Protocol) is enough to determine what "kind" of Device it

is.  In this document, we will call this the Device/Interface, and is

the closest thing there is to a Common Device (the "thing" that you

would normally call a "Device").  The three bytes of data from the

Device Descriptor alone is NEVER enough to describe what a "Device"

is or does (in spite of the fact that it's called a Device

Descriptor).

Unfortunately, the story does not end there.  Sometimes, the

combination of data in the Device Descriptor and Interface Descriptor

still is not enough to define what "kind" of Device it is.  There are

times when additional hierarchy tiers are needed before you know what

a "Device" actually is and does.  As you might expect, these are

defined with more Descriptors which must be downloaded from the

Logical Device.  In this document, we will still refer to the "kind"

of Device as the Device/Interface, even when it requires more than

the Device tier and Interface tier to figure out what it is.

DOS USB DRIVER INTRODUCTION            Page 11 of 186

A common example of this is what USB calls Human Interface Devices or

HID's (mice, keyboards, and joysticks are all "types" of HID's).

Human Interface Devices have a Device Descriptor, but what's

contained in the Device Descriptor essentially says, "This isn't a

Device -- You need to look at the Interface Descriptor to figure out

what this 'thing' is."  So, even though the word "Device" appears in

the title "Human Interface Device", an HID is not considered to be a

"Device" in any USB-defined sense of the word.

Because there is nothing of substance in the Device Descriptor for an

HID, you must look at the Interface Descriptor.  Well, with an HID,

it turns out that the Interface Descriptor usually doesn't tell you

anything you need to know, either.  For most HID's, an Interface

Descriptor essentially says, "This Interface is a Human Interface

Device."  Because that's pretty useless information, you must

download yet another Descriptor, called an HID Report Descriptor

(which is unique to the HID "class" of Interfaces).  In the HID

Report Descriptor is where you finally learn that the "thing" is a

joystick, credit card reader, telephone keypad, or one of the myriad

of other things that USB generically classifies as HID's.  A joystick

would be an example of a Device/Interface in this document.

Examples

��������

Let's go into some specific examples of the different definitions of

"Device".  I've seen a "thing" that is a combination keyboard and

mouse in one unit.  This one Physical Device (a single USB cord gets

plugged into a single USB port), one Logical Device (it gets assigned

a single USB Address), and two Device/Interfaces (one each for the

keyboard and the mouse).  Most people would consider a mouse and a

keyboard to be two separate Common Devices, even though only one USB

cord gets plugged into a USB port.

I've also seen a "thing" that is a keyboard with extra USB ports on

it, which can be used to plug other Physical Devices into.  The ports

that Physical Devices plug into are associated with special

Device/Interfaces called USB Hubs, which are described later in this

document.  A keyboard with extra USB ports is a single Physical

Device, two Logical Devices (the keyboard and the Hub are assigned

different Addresses), and two different Device/Interfaces.  This

would be considered one Common Device by most people.

Although I've never actually seen a USB modem, based on what the USB

specifications say, it would work like this.  A USB modem would be a

single Physical Device, a single Logical Device, but it would have

two Device/Interfaces -- on for the "data" and one for the "control"

(e.g., the Hayes "AT" commands that you usually use to control a

modem).  Of course, a modem is a single Common Device.

DOS USB DRIVER INTRODUCTION            Page 12 of 186

There are also Physical Devices like "reading lights" that plug into

a port on the USB bus, but are not a "Device" in any kind of USB

architecture (software) sense at all.  These Physical Devices simply

"steal" power from the bus, but do not require any software to

operate.  Such Physical Devices are not Logical Devices, nor are they

Device/Interfaces.  Depending on your perspective, they may be

considered a Common Device, or you may instead consider them to be

more of an "accessory" or a "power leach" than a "Device".  As far as

the USB architecture is concerned, such "Physical-only Devices" don't

even exist, even though they use USB-provided resources (power).

Device/Interface Configuration

������������������������������

Sometimes, a Physical Device can be two different "things", but can

only be one of them at a time.  For example, you may have a camera

that can either be a video camera or a still camera, but not both at

the same time.  In the USB architecture, this could be set up as two

different Configurations of the same "Device" (as defined in the

"Device" part of Device/Interface).  As the "Device" part of the

Device/Interface is being "set up", only one of the Configurations

can be selected at a time.

In addition, an Interface (as defined in the "Interface" part of

Device/Interface) can have multiple "configurations", only one of

which can be selected at a time.  In the USB architecture, these are

called "Alternate Interfaces".  An example of this could be a video

camera with different "resolutions" (number of pixels, number of

colors, frames rates, etc).  As the "Interface" part of the

Device/Interface is being "set up", only one of the Alternate

Interfaces can be selected at a time.

Note that not all Device/Interfaces have multiple "configurations",

but some of them do.  Generally, "simpler" Device/Interfaces (mice,

keyboards) do not, while more "complicated" Device/Interfaces

(printers, cameras) do.

DOS USB DRIVER INTRODUCTION            Page 13 of 186

Summary

�������

This was a long section, and I certainly hope you are not even more

confused or bored than you were when you started (that was definitely

my intent).  The main point I want you to take from this is that USB

is EXTREMELY complicated and confusing, and that you must be VERY

careful when the terms "Device" and "USB" are used at the same time.

They can sometimes mean very different things depending on the

audience and the context, and you must make sure that both sides

understand what kind of "Device" is being referred to.  In this

document, we will try to use the most appropriate term (Physical

Device, Logical Device, Device/Interface, and Common Device) in the

context, as defined here.  The terms do overlap to some degree, so

there will be cases where the terms might be somewhat ambiguous

(interchangeable).  Let me know if there's an instance where I don't

appear to be using the correct term, and I will try to fix it or

explain it.

I want to also point out that most other USB Drivers and

Documentation (DOS, Windows, etc.) try to "hide" all of these kinds

of details from you (especially Windows).  That certainly makes

things "easier" to use, at least when everything works like it's

supposed to.  Unfortunately, in USB, almost nothing works like it's

supposed to all the time.  When something doesn't work in Windows,

the way you "fix" problems is to "reinstall or update the software,

reboot, and see what happens".  That isn't really "fixing" anything

at all.

These DOS USB programs provide you a lot of detail, many times much

more detail ("clutter") than really want to see.  This detail is

provided to help you figure out how things work, especially when

things aren't working like you think they're supposed to.  The detail

is provided to help you figure out what's wrong, fix it, and move on

to something productive.

DOS USB DRIVER INTRODUCTION            Page 14 of 186

USB HOST CONTROLLERS & HUBS

���������������������������

USB is designed as a host-centric (poll & response), tree-shaped Bus.

There is a Host Controller at the top of the tree (and a computer can

have more than one Host Controller).  Embedded with the Host Controller

electronics is a "thing" called the Root Hub, which is what actually

contains the ports that the Physical Devices plug into.  A Root Hub is a

"logical entity", in the sense that it is physically inseparable from

the Host Controller, but is treated as a different "entity" (performs

entirely different functions) in the software.

A Root Hub typically only contains a few physical ports.  If the Root

Hub does not contain enough physical ports to support all of the

Physical Devices that a user wants to plug in, the number of available

ports can be expanded by plugging in a special Device/Interface called a

USB Hub.  According to the USB specifications, there can be up to five

"levels" of USB Hubs between a Physical Device and the Root Hub.

In one sense, a USB Hub is simply another Device/Interface that can be

attached to the Bus.  However, it is a very special Device/Interface,

because it increases the port capacity of the entire bus.  There are

some special communications that need to take place between a Host

Controller and a USB Hub to manage these port expansions.

Theoretically, there can be up to 127 Logical Devices attached to a

particular Bus (the 127 number includes the USB Hubs), though in

practice the number of useful Logical Devices is usually much smaller

than that (largely due to the limited bandwidth of the Bus).

HARDWARE & SOFTWARE

�������������������

For a USB Bus to actually be of benefit to anyone, there are two

different aspects that must be addressed: hardware and software.  The

hardware includes the PCI bus inside the computer that the Host

Controller plugs into, the Host Controller itself, the embedded Root Hub

and its physical ports, any Physical Devices that get plugged into the

Bus (including USB Hubs), all associated cabling required to connect

things together, and in some cases power cables, power supplies, or

batteries to power the Physical Devices.

It is not enough, however, to simply have the hardware.  USB also

requires a large amount of software before it can do anything useful.

For our purposes, we will define software to be the special software

that is installed in memory on the computer.  All USB hardware actually

has embedded software (technically, firmware), but in this document we

will not usually differentiate between the hardware and its embedded

firmware -- we will just call it the hardware.

DOS USB DRIVER INTRODUCTION            Page 15 of 186

USB is specifically designed to be very software-centric.  A Physical

Device simply plugged into the Bus with no software will not even "turn

on".  There is a complicated, software-driven initialization &

enumeration process that takes place after a Physical Device is plugged

into the bus.  This process actually takes place between the Host

Controller Driver (software) and the firmware in the Physical Device,

using the USB bus to transport the signals back and forth.  The entire

enumeration process must be completed successfully before the Physical

Device is even considered a "USB Device".

After a successful enumeration process, the Physical Device will be

"enabled", but still will not do anything useful.  There must then be

some Device/Interface-specific software installed to recognize what the

Device/Interface(s) actually are (Hub, Mouse, Keyboard, Printer, Disk

Drive, Joystick, etc.) and perform functions with them that end up

benefitting the computer user in some fashion.  In the more "advanced"

Operating Systems (Linux/Mac, Windows, etc.), the Operating System may

attempt to load appropriate software automatically after the Logical

Devices have been enumerated.  In DOS, that is not the case.  The user

is responsible for loading the appropriate DOS software drivers -- the

Operating System cannot do it automatically.

VENDOR-SPECIFIC DEFINITIONS

���������������������������

USB makes an attempt to "classify" all Devices that get attached to a

USB bus.  There are multiple levels of these classifications, with the

top level of classification being the Device Class (one of the items

contained in the Device Descriptor).  There are far too many types of

"sub-classifications", and even too many ways of defining and

enumerating the classifications, to go into here.  And, even if we

detailed all of the classifications that are strictly defined with

official USB standards documents (of which there are many), we still

wouldn't have all of our bases covered.

While USB is intended to be "standards-based", it is also intended to be

"flexible".  Therefore, even if an official standard exists for how a

particular "class" or "type" of Device/Interface should work, a

Device/Interface doesn't necessarily need to follow the standard.  USB

also allows a classification of "Vendor-Defined" to be used almost

anywhere that a manufacturer sees fit.  As you might imagine, any

Device/Interface that falls into the "Vendor-Defined" category requires

a special "Vendor-Written" software Driver, since the Vendor is the only

one who knows how the Device/Interface really works.  As it turns out,

there are a LOT of USB Device/Interfaces that fall into this "Vendor-

Defined" category, and require custom software Drivers to use them.

Usually, the manufacturer of the Device/Interface is the one that needs

to write the Driver, and you can have a lot of fun trying to talk them

into even bothering to write a Driver for DOS.

DOS USB DRIVER INTRODUCTION            Page 16 of 186

All of the DOS Drivers written so far have been written for "generic"

USB Device/Interfaces (Mice, Keyboards, Printers, etc.) that follow the

USB standards.  Most "generic" Device/Interfaces should work with these

generic Drivers, as long as the Device/Interface doesn't do anything too

unusual.  Again, USB is flexible, so sometimes hardware manufacturers do

strange things with their Device/Interfaces that a generic Driver does

not understand.  Sometimes, the Driver can be "tweaked" to accommodate

the almost-generic Device/Interface, and sometimes you're better off

getting a different Device/Interface made by a different manufacturer

that is truly "generic".

In addition, the entire USB architecture is widely recognized as being

"flaky".  In fact, when USB version 2 was being worked on (several years

ago), Via Technologies (a large manufacturer of USB Host Controllers)

tried to talk the industry out of it because of all the problems there

were with USB version 1.  There are a lot of times when Physical Devices

simply aren't recognized correctly when plugged in (even under Windows),

with no apparent or legitimate reason why they shouldn't work.  I do not

know if it's because the USB specifications are faulty, or the

specifications are just fine and the manufacturers simply don't follow

them.  I just know that USB doesn't work correctly all of the time, and

anybody who has been around USB very much will tell you the same thing.

HOST SOFTWARE

�������������

The Host software is responsible for managing everything that happens on

the bus.  This includes things like Physical Device connects &

disconnects, assigning & maintaining Addresses, notifying other software

(such as individual Device/Interface Drivers) of changes to the bus,

data packet scheduling, etc.  The Host software literally contains a

"map" of the entire bus: which Physical Devices are attached to which

ports of which Hubs, what kinds of Device/Interfaces they are, and

whether or not there is a software Driver currently "controlling" the

individual Device/Interface.  Host software is quite complicated, and

requires a significant amount of memory.

DOS USB DRIVER INTRODUCTION            Page 17 of 186

As part of managing the bus, the Host software needs to "do" things to

an individual Physical Device, Logical Device, or Device/Interface at

various times (assign Addresses, Enable/Disable, Reset, Suspend/Resume,

etc.).  For some of these things, the Host software "talks" (sends a

data packet across the bus hardware) directly to the Logical Device (the

firmware) it is trying to control.  An example of this is when the Host

software assigns an Address to a Logical Device.  In some cases,

however, the Host software does not interact directly with the Physical

Device it is trying to control, but rather "talks" to the Hub software

of the Hub that the Physical Device is plugged in to.  For example, to

Enable or Disable a particular Physical Device, what the Host software

actually does is Enable or Disable the port on the Hub that the Physical

Device is attached to.  It doesn't directly tell the Physical Device

"Enable or Disable yourself".  Similarly, to Reset a Physical Device,

the Host software tells the appropriate Hub to send a special Reset

Signal out of the port that the Physical Device is attached to.

When the Host software wants to send a data packet to a particular

Logical Device, it essentially sends a "broadcast" packet that goes out

over the bus hardware.  All Logical Devices attached to the bus will see

the data packet.  However, part of the data in the header of the packet

is the address of the Logical Device that the packet is intended for,

and only the Logical Device with the correct address is allowed to

respond to the packet.  Unlike an ethernet bus, there is no "broadcast

address" that every Logical Device on the bus will respond to -- each

data packet is intended for only one specific, unique Logical Device.

DEVICE INITIALIZATION & ENUMERATION

�����������������������������������

When a Physical Device is first attached to the bus (plugged into a port

on one of the Hubs), the Host software is notified that a Physical

Device has been attached.  However, the Physical Device is not "enabled"

in any fashion whatsoever, and will not actually do anything yet.  The

Physical Device must wait for the Host software to initialize and

enumerate it.

The Host software Resets and Enables the Physical Device, at which time

the Logical Device automatically assumes Address 0.  The Host software

will download several Descriptors from the Logical Device, to determine

exactly what kind of Device/Interface it is and configure it

appropriately.  During the enumeration process, the Host software will

assign the Logical Device a "real" Address (something other than 0).

After the enumeration process is complete, the Host software will allow

an appropriate software Driver to begin to control the Device/Interface

and make it useful.

DOS USB DRIVER INTRODUCTION            Page 18 of 186

Each Logical Device on the bus must have a unique address, so that the

Host software knows exactly which Logical Device it is talking to.

Because of this, it is only possible to enumerate one Logical Device at

a time (only one Logical Device at a time can respond to Address 0).  As

noted before, USB Devices are not always very reliable, and may fail

during the complicated enumeration process (for any number of reasons).

The Host software will try multiple times to enumerate a Logical Device

that is not responding correctly, in hopes that it may work better the

second or third time than it did the first.  An unreliable Logical

Device can take several seconds (or even tens of seconds) to go through

the entire enumeration process multiple times.  Unfortunately, some

Logical Devices still fail to respond even after several attempts, and

the Host software will finally give up and declare a Logical Device to

be "bad".

The Host software must remember that a particular Logical Device is

"bad", so that it will not try to enumerate it again.  In these DOS

Drivers, the Host software does this by assigning a "fake" Address to

the Logical Device.  That is, the Host software keeps track of the

Physical Device much as if it were a valid Logical Device, including

storing an Address associated with the Logical Device.  However, the

Host software internally flags the Logical Device as "bad", and the

Logical Device itself is actually Disabled and does not know anything

about the fake Address.  Among other things, the fake Address gives you

(the user) a way to tell the Host software to try and enumerate the

Logical Device again (you do this by telling the Host software to Reset

the Logical Device, using the fake Address).  Some Logical Devices will

eventually enumerate properly if you Reset them enough times, and some

will never work no matter how many times you do it.  Such is the nature

of USB.

While a particular Logical Device is in the enumeration process, all

other Logical Devices which are waiting to be enumerated must simply

continue to wait until the first one is finished.  If you have several

Logical Devices attached to a bus, it can take a long time (many

seconds) to enumerate all of them, even if they all work properly and

enumerate correctly the first time.  If you have any unreliable Physical

Devices, though, this takes even longer and can seem to drag on into

eternity.  If you have a Physical Device that turns out to be

unreliable, and you have other Physical Devices that work just fine, you

should try to arrange them on the Hub ports so that the unreliable

Physical Device gets enumerated last rather than first.  That way, you

can begin to use the reliable Physical Devices much sooner.

DOS USB DRIVER INTRODUCTION            Page 19 of 186

In these DOS USB Drivers, Address 1 is assigned to the Root Hub.

Technically, the Root Hub does not need to be assigned an Address at all

(at least according to the USB specifications), but assigning it an

Address makes management of the bus much more convenient.  To you (the

user) the Root Hub appears and reacts in the software much like any

other Hub Device/Interface, and you don't need to refer to it in some

special way.  In the DOS USB Drivers, therefore, all "real" Logical

Devices will be assigned Addresses between 2 and 127.  Address 1 always

refers to the Root Hub.

Some Physical Devices will work correctly (enumerate properly) in MS

Windows (or some other Operating System), when the exact same hardware

setup doesn't work with these DOS USB Drivers.  There are also times

when something works in DOS more reliably than it does in Windows.  To

be frank, I have no idea why this happens.  However, I can tell you that

the Windows software is technically not USB-compliant (it doesn't do

everything the way the USB specifications say it's supposed to).  Just

because a Physical Device works in Windows doesn't necessarily mean it's

truly USB-compliant, in spite of what Microsoft (or the USB

organization) might say or believe.

DESCRIPTORS

�����������

Embedded in the firmware of every USB Device is a set of data divided

into Descriptors.  As the name implies, the intent of a Descriptor is to

"describe" to the software on the computer something it needs to know

about the Device/Interface.

Descriptors can be divided into two different types: General and

Specific.  General Descriptors are the ones that the Host Driver

software downloads and uses to initialize & enumerate the Logical

Device, and describe things like power requirements, manufacturer &

model information, and general Device Classification.  General

Descriptors have specific names (Device Descriptor, Configuration

Descriptor, String Descriptor, Interface Descriptor, Endpoint

Descriptor, etc.).  ALL Logical Devices have General Descriptors.  These

DOS USB Drivers have options that allow you to view the Descriptors for

specific Device/Interfaces if you have an interest in that.

DOS USB DRIVER INTRODUCTION            Page 20 of 186

In addition to the General Descriptors that ALL Physical Devices must

have, most Device/Interfaces also contain Specific Descriptors.  The

most common of these are called Class-Specific Descriptors.  For

example, Device/Interfaces that declare themselves to be in the Printer

Class will have Printer-Class-Specific Descriptors, in a format as

defined by the official USB Printer Class Specifications.  Printer-

Class-Specific Descriptors will contain details about things that are

unique to Printers, and that would make absolutely no sense if you were

trying to describe some other type of Device/Interface (say, a

microphone or a mouse).  The Printer Device/Interface Driver will

download the Printer-Class-Specific Descriptors in order to effectively

manage the Printer and make it useful (to allow it to print things).

In addition to the Class-Specific Descriptors, a Device/Interface may

also contain other Specific Descriptors.  These are usually Vendor-

Specific or Model-Specific Descriptors, and they do not have any kind of

"standard" format.  Rather, they are in a proprietary format that is

unique to the Vendor or Model of a particular Device/Interface.  In such

cases, it is usually not possible to create a Driver for the

Device/Interface unless the vendor publishes the details about the

Descriptor, which they rarely do.

Because a Device/Interface Driver needs to know more specific

information about a Device/Interface than the Host software does, it

must interact with the Device/Interface at an extended level (download

additional Descriptors, set special configuration items, etc.).

Therefore, even if a Logical Device is successfully initialized and

enumerated by the Host software, it does not necessarily mean that it

can be successfully controlled by a Device/Interface Driver.  You must

always remember in the back of your mind that USB is "flaky" and

unreliable, and things that should work sometimes don't, and there is

not always a logical reason for it.

DISABLING & SUSPENDING PHYSICAL DEVICES

���������������������������������������

There are several special "states" that a Physical Device can be in.

These states are not entered by "talking" directly to the Physical

Device, but rather are entered by "talking" to the Hub that the Physical

Device is attached to (which may be the Root Hub/Host Controller).  The

Hub responds to the request by generating the appropriate special signal

on the designated port.

Enabled:

The Physical Device is "turned on", but is not necessarily

Configured or Enumerated.  If Enabled but not Configured, a

Physical Device cannot draw more than 500 mW (100 mA) of power

from the bus.

DOS USB DRIVER INTRODUCTION            Page 21 of 186

Disabled:

The Physical Device is "turned off", and is not allowed to respond

to any commands or data that may be on the bus.  It cannot draw

more than 500 mW (100 mA) of power from the bus.  From a Disabled

state, a Physical Device must be Reset in order to start running

"normally" again.  Enabling a Disabled Physical Device is not

sufficient to begin normal operation.

If the Disabled Physical Device is a Hub, all Physical Devices

downstream of the Hub are also Disabled.

Suspended:

The Physical Device has been Enabled and Configured, but is not

allowed to draw more than 2.5 mW (0.5 mA) from the Bus.  The

Physical Device must remain in this low-power state until it is

told to Resume (usually with a Resume signal).  Suspending a

Physical Device is generally the result of some sort of "power-

saving" mode that the computer has entered.

If the Suspended Physical Device is a Hub, all Physical Devices

downstream of the Hub are also Suspended.

Resuming:

Once a Physical Device has been Suspended, the normal way of "un-

Suspending" it is to issue a Resume signal.  However, a Physical

Device will "un-Suspend" as a result of any special signal on the

bus, including a Reset.

Some Device/Interfaces, such as Mice, Keyboards, and Hubs also

have a capability called "Remote Wakeup".  If Remote Wakeup is

supported and enabled in a Device/Interface, and the Physical

Device is currently Suspended, the Device/Interface will generate

a "Wakeup" signal back towards the Root Hub under certain

circumstances (such as if the mouse is moved, a key on the

keyboard is pressed, or a Physical Device is plugged or unplugged

from the Hub).  The Physical Device will automatically Resume

itself, and the Wakeup signal will be recognized by the

appropriate software Driver and Resume all other Suspended

Physical Devices.  These DOS USB Drivers, by default, always

enable Remote Wakeup in Device/Interfaces that support it.

If the Resuming Physical Device is a Hub, all Physical Devices

downstream of the Hub are also Resumed.

DOS USB DRIVER INTRODUCTION            Page 22 of 186

Resetting:

A Reset is the software equivalent of physically unplugging and

then re-plugging a Physical Device into a Hub port.  It will cause

the Physical Device to be un-configured and Disabled.  If the

Driver architecture supports plug-and-play, the Physical Device

will automatically go through the entire initialization and

enumeration process again.

If the Physical Device being Reset is a Hub, all Physical Devices

downstream of the Hub will be Disabled.  If the architecture of

the Drivers supports plug-and-play, the downstream Physical

Devices will automatically be re-initialized and re-enumerated

after the Hub.  If the architecture does not support plug-and-

play, the downstream Physical Devices will remain Disabled.

Power On & Power Off:

Depending on the type of Hub hardware installed, it is sometimes

possible to control (turn on or off) the bus power to a particular

Physical Device.  However, if the Physical Device is self-powered,

it should continue to work whether bus power is provided or not.

If bus power is turned off to a bus-powered Physical Device,

however, it will cease to function (much as if you had physically

unplugged it).

UHCI Host Controllers (Root Hubs) do not provide any means at all

to control the power -- the power is always on, and there is no

way to turn it off.  USB Hubs ("remote" Hubs) can be manufactured

either with per-port-power-switching or global-power-switching.

With per-port-power-switching, the power to an individual port

(Physical Device) can be controlled independently of the other

ports.  With global-power-switching, the downstream ports are

"tied together" from a power perspective -- they either all have

power or they all do not.  It is therefore not possible to control

the power to an individual port (Physical Device) without

potentially affecting several other Physical Devices at the same

time.

It is usually best to avoid the powering on and off of Physical

Devices, because the end results may not always be what you

expect.

DOS USB DRIVER INTRODUCTION            Page 23 of 186

STARTING & STOPPING HOSTS

�������������������������

Much like the different "states" that an individual Physical Device can

be in (discussed in the previous section, "DISABLING & SUSPENDING

PHYSICAL DEVICES"), a Host Controller (and its integral Root Hub) can

also be in different "states".

Running:

This is the "normal" state for a Host Controller.  It is

generating frames, accessing the Schedule in memory, sending data

packets across the bus, etc.

Stopped:

When Stopped, the Host Controller is essentially "paused".  It is

not sending any data at all down the bus, and frames are no longer

being generated.  If a "Run" command is received, the Host

Controller will begin processing things (such as the Schedule) in

the same place it was just before it was Stopped (paused).

Suspended:

This is the same thing as a Suspend to an individual Physical

Device as discussed in the previous section, but is generated by

the Root Hub.  When it is sent to a specific Physical Device, it

is called a "Selective Suspend".  When sent to all Physical

Devices via the Root Hub, it is called a "Global Suspend".

Resuming:

This is the same thing as a Resume to an individual Physical

Device as discussed in the previous section, but is generated by

the Root Hub.  When it is sent to a specific Physical Device, it

is called a "Selective Resume".  When sent to all Physical Devices

via the Root Hub, it is called a "Global Resume".

Resetting:

Upon receiving a Reset command, the Host Controller will disable

and disconnect all downstream Physical Devices (via the Root Hub),

and then "reboot" itself.  This will reset internal registers and

parameters to their default values, and start the Host Controller

running much like it was when the computer was first turned on.

Any Physical Devices attached to the bus will be required to go

through the entire initialization and enumeration process again.

Power On & Power Off:

This is the same thing as the Power On and Power Off discussed in

the previous section, but is generated by the Root Hub.  This is

not supported by the Root Hubs associated with UHCI Host

Controllers.

DOS USB DRIVER INTRODUCTION            Page 24 of 186

Debug Mode:

This is a special mode of the Host Controller that allows hardware

and software manufacturers to debug (troubleshoot) things as they

are being developed.  Exactly what "Debug Mode" does depends on

the hardware specification being used; i.e., UHCI, OHCI, and EHCI

all have different interpretations and implementations of Debug

Mode.

In UHCI Host Controllers (the only ones currently supported by the

DOS USB Drivers), Debug Mode Stops (pauses) the Host Controller as

defined a few paragraphs up above under "Stopped".  The Host

Controller can then be told to Single-Step, where it will generate

a single frame across the bus, and then Stop (pause) again.  The

single frame will do everything a frame running in "normal" mode

would do (read the schedule, generate and send packets down the

bus, generate IRQ's, etc.).  In UHCI Host Controllers, Debug Mode

works very much like the Debug/Single-Step operation in the CPU

familiar to all programmers.

POWER

�����

Built into the USB architecture (hardware & software) is the concept of

power management.  Unfortunately, there are several flaws in the actual

implementation that prevent it from being very useful.

The cables used to connect a downstream Physical Device to its upstream

Hub (either a Root Hub or a USB Hub) contain four wires.  Two of the

wires are the actual USB communications bus, which allow two-way

communication to take place between the Physical Device and the Host

Controller.  The other two wires contain 5 VDC power generated by the

upstream Hub, which may be used by the downstream Physical Device to

power itself.  If the Physical Device uses the power generated by the

upstream Hub to power itself, it is called a bus-powered Physical

Device.  If it uses an external power source of some sort (usually, a

power cord or transformer that plugs into an AC outlet on the wall), it

is called a self-powered Physical Device.

Hubs themselves are simply another type of Device/Interface, and can be

bus- or self-powered.  A self-powered Hub (including Root Hubs) can

deliver up to 500 mA (2.5 W) of current on each of its downstream ports.

A bus-powered Hub draws the power to operate itself from its upstream

Hub, and then also uses whatever available power is left over from the

upstream Hub to power any bus-powered downstream Physical Devices that

may be attached to it.  During the enumeration process, one of the

things that a Physical Device "tells" the Host software is whether it is

self-powered or bus-powered, and if it is bus-powered, how much power it

uses (in amperes).

DOS USB DRIVER INTRODUCTION            Page 25 of 186

Theoretically, the Host software can use this information to determine

when the addition of a downstream Physical Device will overload the

power supply of its upstream Hub, and not even bother to enumerate the

Logical Device.  There a couple of problems with this approach, the main

one being that some Physical Devices lie about their power requirements.

I have personally tested a few dozen different Physical Devices, and at

least two of them claim (in their Descriptors) they are self-powered

when I know for a fact that they are bus-powered.  In addition, there is

also a concept in USB known as a "Unit of Power", which is equal to 500

mW (100 mA).  Many Physical Devices report their power requirements in

Units of Power (multiples of 100 mA), instead of reporting their actual

power requirements (something less than 100 mA).  There are also

Physical Devices (such as "reading lights") that simply "steal" power

from the bus, and do not report their power draw from the bus in any

manner whatsoever.  The bottom line on all of this is that you really

can't believe what any particular Physical Device tells you about its

power requirements.  Some Physical Devices tell the truth, some don't,

and you don't know which are which.

Because the information received from the Physical Devices about their

power requirements cannot be trusted, it is impossible to correctly

manage the power distribution on the bus through software.  Therefore,

in these DOS USB Drivers, we do not even attempt to manage the power.

Instead (and unfortunately), the software will provide the information

about declared power requirements to the user when requested, but the

actual management and resolution of power-related problems are left to

the discretion of the user.  It is simply impossible to do it in a

reliable, efficient, trustworthy manner using software.

UHCI, OHCI, & EHCI

������������������

The following chart shows the progression of the USB standards that have

occurred over the years.  The main intent of each new major revision

number has been to increase speed.  As of the writing of this document

(July, 2009), Wireless USB (technically, Certified Wireless USB) is

still in its infancy, and there are only a few real Wireless Logical

Devices available.  There are no general-purpose Wireless drivers for MS

Windows (or any other Operating System) yet, so all of the Wireless

Logical Devices currently available use proprietary (vendor-provided)

Drivers.  The official USB 3.0 standard was supposed to be released last

year (2008), with no real USB 3.0 Physical Devices expected to be

available until around 2010.

DOS USB DRIVER INTRODUCTION            Page 26 of 186

Host

USB      Controller        Speed        Speed

Version  Interface         Designation  Mbps

�������  ����������������  �����������  ���������

1.0/1.1  Open & Universal  Low          1.5

Full         12

2.0      Enhanced          High         480

2.5      Wireless          Wireless     480 (Max)

3.0      Extensible        Super        4800

The main purpose of this section is to talk about the Host Controller

Interfaces (HCI's - the "protocols" used to manage the Controllers), so

we won't go into a lot of other details about the different USB

versions.  There is plenty of information available on the internet

about that you can look up if you're so inclined.  The one thing I will

point out, however, is that the different standards are all backward

compatible (except for Wireless USB, which is an oddball).  What

"backward compatible" means in the USB sense is that a USB 1.x-

compatible Physical Device can be plugged into a 2.0- or 3.0-compatible

Hub, and will work (albeit at a lower speed than the faster Hub is

capable of).  It also means that a 2.0- or 3.0-compatible Physical

Device can be plugged into a 1.x-compatible Hub and it will also work,

although the Device/Interface will not transfer data as fast as it would

if it were plugged into a newer version of Hub.

The different Host Controller Interfaces (HCI's) have acronyms based on

the first or second letter of their description.  For instance,

Universal Host Controller Interface = UHCI.  So, there are currently 5

different ones (UHCI, OHCI, EHCI, WHCI, and XHCI).

Embedded in the USB Host Controller architecture is the concept of

"companion controllers".  As the above chart indicates, a Host

Controller that is capable of USB 2.0 speeds will have an EHCI-

compatible controller.  However, the only Physical Devices that the EHCI

controller (and software) manages are the ones that are actually

operating at high-speed (480 Mbps).  A Physical Device that is not

capable of high-speed operation, or is attached downstream of a Hub that

is not capable of high-speed operation, or does not have a high-speed

compatible software Driver installed, will not operate at high-speed,

and the EHCI controller will not manage it.  Instead, the EHCI

controller "transfers" control of the Physical Device to the appropriate

full-speed/low-speed companion controller (which uses the OHCI or UHCI

protocol).  The high-speed EHCI controller and its associated low-speed

companion controller(s) (UHCI or OHCI) are built into the same chip set.

So, if you have an EHCI (USB 2.0/high-speed) controller, you will also

have at least one OHCI or UHCI (USB 1.x/low-speed/full-speed) controller

-- you don't need to buy them separately.

DOS USB DRIVER INTRODUCTION            Page 27 of 186

Currently, these DOS USB Drivers only support one type of Host

Interface: UHCI.  There are plans to update the Drivers to include the

later standards (at least OHCI & EHCI), though that will not happen any

time in the near future (don't try to hold your breath until then).  As

far as I know, the only manufacturers who make UHCI-compatible

controllers are Intel and Via Technologies (Via).  All the other

manufacturers make OHCI-compatible controllers.

If you're like most people, and have absolutely no idea what kind of USB

controllers you have, the enclosed USBHOSTS program will help you find

out.  If yours aren't UHCI (made by Intel or Via), you won't be able to

use these DOS USB Drivers as they currently stand.  However, if you have

an extra PCI slot available on your computer, you can purchase a USB

Host Controller card that you can plug into the PCI slot.  Of course,

you must you must make certain that it is UHCI-compatible.  You can

either look at the specifications for the card (make sure it says "UHCI"

or "Universal", and not "OHCI" or "Open").  You can also look at the

name that's stenciled on the main USB chip that's on the card, and make

sure it says "Via" or "Intel".

PCI BUS MASTERING

�����������������

All USB Host Controllers manufactured today are attached to the computer

through the PCI (Peripheral Component Interconnect) Bus.  When data is

being transferred from a Device/Interface into a program (such as a

software Driver) to be used, the data first traverses the Bus from the

Physical Device to the Host Controller.  The Host Controller then puts

the data in the computer's memory (Random Access Memory or RAM).  The

program (Device/Interface Driver) that needs the data then looks in RAM

to see what data was transferred by the Device/Interface.  In essence,

the RAM acts as a "middle man" between the world of the program

(executing code on the CPU) and world of the USB Device/Interface (which

understands the USB bus, but doesn't know anything about computer RAM).

A problem arises because the CPU (where the program is running) speaks a

different "language" than the PCI bus (where the Physical Device is

attached) when it comes to RAM.  In the CPU world, there are different

"kinds" of memory (Conventional/Lower, Upper, High, Expanded, Extended,

Virtual, Paged, etc.), different addressing schemes (Segment:Offset,

Linear, etc.), and different modes of CPU operation (Real Mode,

Protected Mode, "Unreal" Mode, etc.).  In the CPU world, there can be

many different ways to talk about (address) the same "chunk" of memory,

depending on the situation.

DOS USB DRIVER INTRODUCTION            Page 28 of 186

The PCI bus uses a process called Bus Mastering to access RAM, which

does not use the CPU.  The ONLY type of memory address that a PCI

Controller understands is a Physical Address, which is something you

normally never see in the CPU world.  As you might imagine, there must

be a translation that occurs between the CPU language and the PCI

language to make sure that the same "chunk" of memory is being used by

both sides.  In these DOS USB Drivers, the Host Controller Driver

(software) acts as the memory address translator.

As it turns out, translating memory addresses in a DOS program is very

easy in some situations, but virtually impossible in other situations.

In these DOS USB Drivers, there are two different environments where the

address translation works correctly.  The first environment is when the

CPU is running in Real Mode.  This happens if you are using "basic" DOS,

with no memory manager (like EMM386.EXE) installed.  Of course, using

EMM386 offers many advantages (such as providing access to Upper Memory

& Extended Memory), so computers rarely run in Real Mode.

The other environment where the DOS USB Drivers can correctly translate

memory addresses is when the computer is in Protected Mode with a VCPI-

compliant (Virtual Control Program Interface) memory manager (like

EMM386.EXE).  If you use Microsoft's EMM386.EXE program (loaded via your

CONFIG.SYS file), you must make sure that you do NOT use the NOVCPI

option, because that will disable the VCPI memory manager that these DOS

USB Drivers need to translate memory addresses.  Also, unless you

actually need Expanded Memory for some particular reason, I suggest that

you DO use either the NOEMS or FRAME=NONE option to disable the Expanded

Memory Page Frame.  Most modern DOS programs, if they require a large

amount of memory, can use either Expanded Memory (EMS) or Extended

Memory (XMS) to access the extra memory.  Extended Memory does not need

a Page Frame the way Expanded Memory does, and consequently does not

"steal" memory that other programs could use.

It is NOT possible for the DOS USB Drivers to translate memory addresses

in a DPMI (DOS Protected Mode Interface) environment, such as when you

are running underneath Microsoft Windows.  Depending on exactly when a

DOS USB Driver gets loaded into memory compared to when the DPMI

environment starts, however, at least some of the DOS Device/Interface

Drivers may still be able to work correctly.  If you "mess around" with

DPMI programs, you may just need to experiment with things to see what

happens.  Some of the USB-attached Physical Devices may work in your

DPMI programs and some may not.  It is usually best to avoid DPMI

altogether if you can, and just do things the "old-fashioned" way.

DOS USB DRIVER INTRODUCTION            Page 29 of 186

ISOCHRONOUS, INTERRUPT, CONTROL, & BULK TRANSACTIONS

����������������������������������������������������

It is possible to attach many different kinds of Physical Devices to a

USB Bus, everything from mice to disk drives to networks to video

cameras.  The different kinds of Physical Devices are VERY different

from each other in several respects.  For instance, mice only transfer

small amounts of data at a time, need to do it tens or hundreds of times

every second, usually don't ever get "turned off", and it's not a big

deal if some of the data gets "lost".  A disk drive, on the other hand,

usually transfers relatively large amounts of data, only needs to do it

every once in awhile (when it is "asked" to do it for some reason),

doesn't necessarily need to respond right away when it is asked for the

data (it can take a little bit of time to "think" about it if it needs

to), but must be assured that all of the data is transferred correctly

with no errors.  The USB architecture is designed to handle all of these

different requirements by using a hierarchial, prioritized packet

scheduling scheme.

Data gets transferred across the USB Bus in packets.  Unlike the "random

access" packet scheme of the ethernet bus that many people are used to

(CSMA/CD), USB uses a very structured scheme.  The Host Controller

Driver (software) controls everything that happens on the bus, and it

decides exactly when and where each packet will go.  The process of

requesting, scheduling, and actually transferring a packet across the

bus in USB jargon is called a Transaction.

USB defines four different types of Transactions, which is what allows

USB to properly process and prioritize the different "types" of data

packets: Isochronous, Interrupt, Control, and Bulk.  Incidentally, those

are listed in order of scheduling priority -- Isochronous Transactions

are scheduled first, and Bulk Transactions are scheduled last.

Isochronous Transactions are used to send data where there is a

significant amount of data to be transferred, but WHEN the data is sent

is more important than IF the data ever makes it to the other end.

Device/Interfaces that use Isochronous Transactions are generally

considered "real-time" or "streaming" Device/Interfaces, such as

microphones, speakers, and video cameras.  Isochronous packets are sent

(scheduled) at a very specific time, and the USB bus will make its best

attempt to make sure the data successfully reaches the other end.

However, if the data doesn't make it to the other end on the first

attempt for some reason, the data is simply "lost".  USB doesn't even

monitor whether the data actually makes it to the other end or not, and

never bothers trying to resend it if it doesn't.

DOS USB DRIVER INTRODUCTION            Page 30 of 186

Interrupt Transactions are used to send data when there is a small

amount of data to be transferred, the data is fairly continuous or comes

in infrequent bursts, the EXACT timing of when the data is transferred

is not critical (the timing can "slide" back and forth a little bit),

and (similar to Isochronous Transactions) the data is not required to

make it successfully to the other end.  Device/Interfaces that use

Interrupt Transactions include mice, keyboards, and joysticks.  Unlike

Isochronous packets, however, Interrupt packets ARE monitored and USB

knows whether or not the data makes it to the other end.  However, USB

does not automatically attempt to resend an Interrupt packet if it

doesn't make it the first time.

Bulk Transactions can be used to send either small or large amounts of

data, exactly when the data gets transferred doesn't matter (it can get

"delayed" a little bit), and the data must be "guaranteed" to reach the

other end.  Device/Interfaces that use Bulk Transactions include disk

drives, still cameras, and printers.  USB closely monitors Bulk packets,

performs data integrity checks, and automatically retries multiple times

(if needed) in an attempt to make certain the data is transferred

successfully.

Control Transactions are used to monitor and control the USB Bus and

Devices.  All types of Device/Interfaces use Control Transactions.  For

example, in the initialization and enumeration process discussed

previously, Control Transactions are used to download the Descriptors

from the Physical Device, to set the Logical Device Address, and to

select various configuration options.  Like Bulk packets, Control

packets are closely monitored and retried multiple times (if necessary)

in an attempt to make certain that the packets are transferred

successfully.

PACKET SCHEDULING

�����������������

A USB 1.x Bus operates at a speed of 12 Mbps.  This is divided into 1000

smaller "divisions" called Frames (each taking 1/1000 of a second).

Therefore, each Frame can transmit 12,000 bits of data.  There is some

overhead associated with maintaining the Bus and its packet-oriented

data transfers, so it is not actually possible to transmit 12 million

bits of "real" data every second.  The actual data throughput depends on

many factors, but will always be something less than 12 Mbps.

DOS USB DRIVER INTRODUCTION            Page 31 of 186

The Host Controller hardware and Host Controller Driver (software)

"communicate" with each other to transfer packets back and forth across

the Bus using a Schedule.  The Software places items in the Schedule (a

chunk of RAM) that tell the Hardware where to get data from (other

chunks of RAM), where to send it (Logical Device address, end point,

etc.), and how to send it (data integrity check and retry options,

etc.).  At the start of each Frame (once every millisecond), the

Hardware looks at the Schedule (in RAM), transfers data according to the

Schedule (as defined by the Software), and notifies the Software when

Transactions have been completed by modifying certain sections of RAM

and generating hardware interrupts (IRQ's).  There is this constant

back-and-forth between the Host Controller Software and Hardware that

results in data packets getting sent across the USB Bus, but the control

of everything that happens lies with the Host Software (the Hardware

just does what the Software tells it to do in the Schedule).

There are several requirements that the Host Controller Driver

(software) must follow in determining how to set up the Schedule.  For

instance, it must attempt to make sure that Isochronous packets get

scheduled first, and Bulk packets get scheduled last.  In addition,

according to the USB specifications, 10% of the Bus bandwidth must be

"reserved" for non-time-critical Transactions (Control and Bulk).  In

other words, the Isochronous and Interrupt Transactions cannot consume

the entire Bus, even though they are considered higher priority than the

Control and Bulk Transactions.  There are many other requirements as

well, and as a result, the Host Software is very complicated and uses a

substantial amount of memory.  Unfortunately, that is a "necessary evil"

if you want to use something as advanced and flexible as USB.

BUS TIMING

����������

A USB version 1.x bus normally operates at 12 Mbps.  The Host Controller

has an on-board oscillator that controls the bus speed.  There are times

when it may be necessary to adjust the speed (either slightly faster or

slightly slower).  Usually, the only time this is required is when you

are using an Isochronous Device/Interface (such as a telephone or video

camera) with its own timing source, and you must synchronize the two

timing sources together to avoid losing data.  The USB architecture

allows the timing to be adjusted by small amounts around the "optimum"

12 Mbps to allow for synchronization to outside sources.

DOS USB DRIVER INTRODUCTION            Page 32 of 186

The DOS Application Programming Interface (discussed below, page 43)

provides a way for a program (presumably a software Driver for an

Isochronous Device/Interface) to manipulate and control the bus Timing.

One thing to keep in mind, however, is that there can be only one

"Owner" of the bus Timing.  This means, for example, that you cannot

install two different Device/Interfaces, both of which must be

synchronized to two different external timing sources, on the same bus

(they cannot be on the same Host Controller).  If you ever have such a

situation, you must install the two Device/Interfaces onto two separate

busses (two different Host Controllers).

The way that the USB architecture controls the Timing of the bus is by

manipulating the number of bits that are sent in a Frame.  There are

1000 Frames per second, and (normally) 12,000 bits per Frame.  Each

version of Host Controller (UHCI vs. OHCI) allows a different "range" of

manipulation.  For example, in a UHCI Host Controller (the only one

currently supported by the DOS USB Drivers), you can set the number of

bits per Frame to any number between 11,936 and 12,063 (the default

value, obviously, is 12,000).  Increasing the number of bits per Frame

makes the bus run slower, and decreasing the number makes the bus run

faster.

In addition, the USB specification states that the Timing cannot be

changed by a large amount instantaneously while the bus is running --

the changes must occur smoothly and slowly.  Specifically, the USB

specification states that the Timing cannot change by more than one bit

per Frame for every six Frames.  The change from one extreme to the

other (in UHCI, 11,936 to 12,063, or vice-versa), for example, takes

approximately 3/4 of a second.  These DOS USB Drivers handle the bus

Timing change process automatically, and perform Timing changes smoothly

and slowly according to the USB specifications.  Basically, you just

need to give the Driver a "target" Timing value, and it handles all of

the details automatically.

DOS USB DRIVER INTRODUCTION            Page 33 of 186

LEGACY SUPPORT

��������������

What is Legacy Support?

�����������������������

USB has a term called "Legacy Support".  Exactly what that means has

changed over the years since USB's introduction in the mid-1990's,

and exactly how it is implemented also varies from computer to

computer.

One of the main goals of USB was an attempt to replace all of the

different "Legacy" external ports on the computer with a new, single,

"universal" port/bus architecture.  Legacy ports include PS2 ports

for keyboards and mice, parallel ports for printers and scanners,

serial ports for modems and terminal emulators, SCSI ports for

external hard drives and CD-ROM's, and various other specialized

ports dedicated to many different kinds of external Devices.  Also,

as a rule, the software required to support Legacy hardware was

fairly efficient (didn't require a lot of memory), and was usually

installed in the BIOS.  The BIOS is loaded before the Operating

System, so Legacy hardware and software was generally Operating

System independent.

From a user perspective, USB is "easier" because there is only one

kind of physical port to connect to, instead of the multitudes of

specialized and proprietary ports there were used before.  However,

USB is very software-centric (memory-intensive), and is intentionally

designed so that the software is loaded by the Operating System

instead of the BIOS.  Of course, the only Operating System that

anybody (especially Microsoft) thinks actually matters is MS Windows,

so usually the only kind of vendor-provided USB Drivers you can find

are made for the newer Windows Operating Systems.

There is a major problem with this philosophy, however.  There are

certain kinds of Device/Interfaces that need to work before the

Operating System is even loaded.  For instance, in order to get into

the computer's BIOS/CMOS settings when the computer first turns on,

you need to be able to use a keyboard (and, on some computers, a

mouse).  In fact, sometimes you need to be able to configure

something in the BIOS before it is even POSSIBLE to load the

Operating System.  There are also times when it is necessary to

access a Disk Drive before an Operating System is loaded, such as

when you are trying to install a new Operating System.

DOS USB DRIVER INTRODUCTION            Page 34 of 186

What "USB Legacy Support" means in a general sense is the ability to

access and use a USB Keyboard, Mouse, or Disk Drive when the computer

first turns on, before the part where the computer even decides which

Operating System to load.  This also "spills over" into the ability

to use certain Device/Interfaces (Keyboards, Mice, and Disk Drives)

in Operating Systems that pre-date USB (e.g., DOS and older versions

of Windows), though that is not its true intended purpose.

Note, however, that "Legacy Support" means different things on

different computers.  On some computers, it means all three

Device/Interfaces can be used (Keyboards, Mice, and Disk Drives).  On

others, it may mean just one or two of them, but not necessarily all

three.  On some computers, it means "plug-and-play", and on others it

means all of the Legacy Physical Devices you intend to use must be

plugged in when the computer is first turned on.  On some computers,

the Legacy Physical Devices must be attached directly to a Root Hub

(or a specific port on a specific Root Hub if you have more than

one), and on other computers you can have some "regular" USB Hubs

between the Legacy Physical Device and the Root Hub.  So, even if you

enable Legacy Support on your computer, it doesn't necessarily mean

the computer will do exactly what you want or expect it to do.

Also, FYI, you usually enable Legacy Support by getting into the

BIOS/CMOS settings as the computer is first turning on (usually, by

pressing some special key or combination of keys on the keyboard).

This poses an interesting situation.  If the only kind of keyboard

you have is a USB keyboard, and Legacy Support is turned off in the

BIOS, it is impossible for you to get into the BIOS to enable the

Keyboard.  Likewise, if you use the USB Keyboard to disable USB

Legacy Support in the BIOS, the next time you try to get into the

BIOS to change something, you won't be able to because your USB

Keyboard won't work.

In my opinion, all computers SHOULD have a PS2 keyboard port, and you

should ALWAYS have a regular PS2 keyboard lying around somewhere that

you can use. If you don't, you may find yourself in a situation where

it is impossible to boot the computer, because you need a "real"

keyboard to be able the turn on the USB keyboard.  Consider yourself

warned -- USB is not the panacea it's advertised to be.

DOS USB DRIVER INTRODUCTION            Page 35 of 186

How is Legacy Support Implemented?

����������������������������������

Now that we've talked about why Legacy Support is necessary and some

of the problems you can have while using it, let's talk about how it

actually works.  Most people have heard of some of the different

"modes" the CPU can be in: Real Mode, Protected Mode, and Unreal

Mode.  As it turns out, there is another obscure mode that modern

CPU's can be in, called "System Management Mode".  I don't know very

much about why System Management Mode actually exists or what exactly

is supposed to be used for, though admittedly I haven't really looked

very hard.

When USB Legacy Support is turned on for Keyboards or Mice, and a USB

key is pressed or a USB mouse is moved, the USB Host Controller sends

a special signal via the PCI bus to the CPU that tells the CPU to go

into System Management Mode.  While in this mode, the computer

essentially "plays a bunch of games" with the motherboard hardware

and is able to emulate a keystroke or mouse movement through the

keyboard controller hardware.  Any software that is running on the

computer thinks that the keystroke or mouse movement was generated by

a "real" keyboard or mouse (attached to a "real" PS2 port).

The advantage of Legacy Support is that it works at such a low level

(direct hardware interaction) that the keyboard/mouse emulation

should be able to fool ANY Operating System or Application into

believing that the computer actually has a PS2 keyboard or mouse

attached to it.  Unfortunately, there is a huge disadvantage to

Legacy Support as well: Legacy Support is limited to Keyboards, Mice,

and Disk Drives (or some subset thereof).  Legacy Support does not,

and will probably never, include support for the myriad of other USB

Device/Interfaces you may want to use (Printers, Joysticks, Cameras,

etc.).

There is another disadvantage to using Legacy Support as well.  In

Legacy Support mode, mice and keyboards do not usually have complete

functionality.  A "basic" mouse for a PC has just two buttons;

however, almost all USB mice these days have a scrolling wheel (which

inherently includes a third button), and many "fancy" mice have

several additional buttons and wheels besides those.  A "basic"

keyboard has 101 or 102 keys; however, most USB keyboards have

additional "system control" keys (Power, Sleep, Wakeup) or

"multimedia" keys (speaker volume/mute, CD player

Stop/Start/Pause/Skip, etc.).

DOS USB DRIVER INTRODUCTION            Page 36 of 186

USB Mice and keyboards have two different modes they can operate in:

Boot Mode and Report Mode.  A Mouse in Boot Mode is a simple two-

button mouse, and any of the extra buttons or wheels it may have are

completely ignored -- it's like they don't even exist.  Similarly, a

Keyboard in Boot Mode has only 101 or 102 keys, and any of the

"extra" keys it has are simply ignored.  When a USB Mouse or Keyboard

is first enabled, it comes up in Boot Mode, and the BIOS (Legacy

Support) software usually leaves it in Boot Mode.  The reason Boot

Mode exists is to make the BIOS software simpler and smaller.  The

Legacy Support software can simply assume a "basic" mouse or keyboard

configuration, and doesn't need to download and decipher the special

Descriptor (called a Report Descriptor) that tells it how many

buttons and wheels and keys the Mouse or the Keyboard actually has.

An Operating System driver will (or at least should) change the Mouse

or Keyboard into Report Mode, and allow you to use the extra buttons

and wheels and keys that exist on your Mouse or Keyboard.  If you

want to (or need to) use the extra "features" that your Mouse or

keyboard has, you usually can't use Legacy Support -- you must use an

Operating System Driver.

There can only be one Host Controller Driver (software) controlling a

USB Host Controller.  If the Legacy Support Driver (in the BIOS) is

controlling the Host, an Operating System Driver cannot try to

control the Host at the same time.  Once the Operating System Driver

takes control, it is usually not possible to revert back to the BIOS

(Legacy Support) Driver without rebooting the computer.  It would

have been possible to design a mechanism that allowed the Host

Controller to "share" two Drivers (where the Legacy Support software

controls the Keyboard and Mouse, and the Operating System software

controls all the other Device/Interfaces), but that's not the way

they did it.

Most modern computers have more than one USB Host Controller

installed in them.  When you enable or disable Legacy Support in the

BIOS (at boot time), it usually affects the Legacy Support for ALL of

the Host Controllers that are installed in the computer.  However,

Legacy Support is actually configured independently on each

Controller, so you can leave Legacy Support enabled on some

Controllers and disable it on others.  So, if you have more than one

Controller, and you connect and set things up correctly, you can have

your keyboard and/or mouse on a BIOS (Legacy Support) Controller, and

all of your other Device/Interfaces on an Operating System

Controller.

DOS USB DRIVER INTRODUCTION            Page 37 of 186

These DOS USB Drivers for the Keyboard (USBKEYB) and Mouse (USBMOUSE)

provide reasonably good functionality, but it is not nearly as

comprehensive as the Legacy Support (the Operating System drivers do

not have access to System Management Mode and the low-level hardware

resources that the Legacy Support driver does).  These DOS Drivers

should work in most situations and with most DOS programs, but there

are times when they don't.  If you find yourself in a situation (such

as in certain DPMI environments) where the DOS Keyboard or Mouse

Drivers don't work, and that situation can't be avoided (by using a

different program, or using a "real" PS2 keyboard or mouse), then you

may need to use Legacy Support instead of using the Operating System

Drivers.  Just keep in mind that if you do that, the "extra" buttons

and wheels and keys that your Mouse and Keyboard may have will

probably be ignored.  It would be very nice if you could have it both

ways: to be able to use all of the buttons and wheels and keys that

are available to you, and to be able to use them in any program at

any time.  Unfortunately, that's simply not possible because of the

inherently short-sighted design of USB.

DOS USB DRIVER INTRODUCTION            Page 38 of 186

PROGRAM ARCHITECTURE

��������������������

This section will discuss the overall architecture and philosophy that

was used while writing the DOS USB Driver programs.  There are more than

20 programs altogether, and I'm sure you are wondering why there are so

many.  Hopefully, this section will explain that.

There are some other DOS USB Driver programs out there, some of which

you are probably already using.  This section will compare the way these

DOS USB Drivers are written with the ways the other Drivers are written,

in an attempt to show you why my programs are, in several ways, far

superior to the others.  I will admit that the programs still have

several shortcomings and issues, but the overall design philosophy

allows those to be addressed and overcome.  The architecture is open and

extensible (not proprietary), which means that the improvements and

additions can actually be written by anyone -- you don't necessarily

need to wait for me to get around to it.

TSR vs. CONFIG.SYS

������������������

Most (if not all) of the other DOS USB Driver programs that are

available require at least some of the Driver software to be loaded at

boot time, using CONFIG.SYS.  If you want to change something, or if you

just want to sometimes use the USB hardware and other times don't, you

need to edit your CONFIG.SYS file and reboot.  Luckily, booting to DOS

doesn't take very long (compared to any other "modern" Operating System,

at least), but it's still a hassle and should be avoided unless it's

absolutely necessary.

These DOS USB Drivers are all TSR's, which can be loaded and unloaded

from memory at any time without rebooting.  You can load some or all of

the Drivers at boot time if you want to (using AUTOEXEC.BAT), but it's

not required.  You can also load and unload the appropriate Drivers you

need manually (from the command-line) or "automatically" (using batch

files) just when you actually need them.  This way, you're only using

precious memory for the Drivers when you're actually using them, and not

wasting memory for something you only need every once in awhile.

DOS USB DRIVER INTRODUCTION            Page 39 of 186

BACKGROUND vs. FOREGROUND

�������������������������

Most of the other DOS USB Driver programs that are available send data

packets across the USB bus as a foreground operation.  That is, when you

tell the Driver to send some data to a particular Logical Device, you

must wait until the data transaction has been completed before you can

do anything else.  The Driver can only do one thing at a time, and that

thing must be "triggered" by a foreground operation.  That is why most

of the other DOS USB Driver programs only support Device/Interfaces such

as flash drives and CD-ROMs, which are inherently only accessed with

foreground applications.  Driver programs that are designed to operate

in the foreground will never to be able to support inherently background

Device/Interfaces, such as mice and keyboards.

These DOS USB Drivers are specifically designed to be run in the

background.  That is, when you tell the Driver to send some data to a

particular Logical Device, the Driver essentially says, "OK -- now go do

something else and I'll let you know when it's done."  The Driver takes

care of sending the data ENTIRELY in the background, while the computer

is busy doing other things in the foreground.  As you might imagine,

running a process in the background is much more complicated (requires

more memory) than doing it in the foreground.  However, the advantages

it provides (such as the ability to support mice and keyboards)

generally makes it worth the extra complexity.  In addition, a

background process can very easily "simulate" a foreground process in

situations where that is required, while it is impossible for a

foreground process to simulate a background process.

MODULAR vs. MONOLITHIC

����������������������

Most of the other DOS USB Driver programs are monolithic in nature.

That is, once they are installed, "what they do is what they do".  For

instance, there are several DOS USB Drivers whose sole purpose is to

allow you to use USB disk drives (flash drives, external hard drives,

etc.).  They are not modular nor are they extensible, so if there is no

internal support in the monolith for USB mice or keyboards or printers

or whatever (and there almost never is), you'll never be able to use

those other kinds of Device/Interfaces.

These DOS USB Drivers are modular, not monolithic.  For instance, if you

have more than one USB Host Controller (most modern computers do), you

need to load a separate Driver for each Controller.  If you only want to

"turn on" some of them rather than all of them (which is usually the

case in DOS), you just load Drivers for the ones you want to use.

DOS USB DRIVER INTRODUCTION            Page 40 of 186

However, in this architecture, a Host Controller Driver "module" does

not control ANY type of USB Device (not even disk drives, like the other

Drivers do).  Rather, there are separate Drivers for each type of

Device/Interface (hubs, disk drives, mice, etc.), and you just install

the ones you want to use.  If you don't have any USB Hubs (or don't want

to use any of the Physical Devices you have attached to the Hubs), you

don't need to load the Hub Driver.  If you don't have any flash drives

(or aren't using them right now), you don't need to load the disk drive

"module".  You only load the Drivers for the Device/Interfaces that you

want to use right now, and can simply uninstall them when you don't need

the Logical Device any more.  There are currently Drivers for several

kinds of common Device/Interfaces written (hubs, disk drives, mice,

keyboards, joysticks, and printers).  Because of the modular and open

nature of the architecture, however, Drivers can be written for almost

any type of Device/Interface (you can even write one yourself).

PLUG-AND-PLAY vs. STATIC

������������������������

Most of the other DOS USB Driver programs are "static".  That is, you

must have the particular Physical Device that you want to control

plugged into the USB bus before the Driver (software) is installed.  If

the software can't find the correct hardware while the software is being

loaded, the software just stops.

These DOS USB Drivers are "Plug-and-Play".  It doesn't matter if the

software is installed before the hardware, or vice-versa.  For instance,

you can leave a USB joystick plugged in all the time, and just load the

joystick driver immediately before you load the game that uses the

joystick, and unload the drivers as soon as the game is done.  Or, you

can load the disk drive software through your AUTOEXEC.BAT file, leave

it running all the time, and just plug and unplug your flash drives when

you want to use them (just like you do with floppy drives).  The program

architecture allows the correct hardware and software to "find" each

other, no matter which one gets installed first.

PROTECTED MODE vs. REAL MODE

����������������������������

Most of the other DOS USB Driver programs require the computer's CPU to

be running in Real Mode.  In particular, that means you are unable to

use the EMM386.EXE memory manager (or some equivalent), because it puts

the CPU into Protected Mode.  Without something like EMM386 installed,

you do not have access to Upper, Expanded, or Extended Memory, which

greatly decreases the amount of available DOS memory.

DOS USB DRIVER INTRODUCTION            Page 41 of 186

These DOS USB Drivers do not require the CPU to be in Real Mode.

However, if the CPU is in Protected Mode, it must be in a VCPI (Virtual

Control Program Interface) environment.  This is exactly what happens if

you install EMM386.EXE (as long as you don't use the NOVCPI option).

While the programs won't work in a non-VCPI protected mode environment

(like DPMI or DPMS), they do at least work in some protected mode cases.

This greatly increases the amount of memory you have to run DOS programs

while these USB Drivers are installed.

LEGACY EMULATION vs. NEW PROGRAM DEVELOPMENT

��������������������������������������������

Some of the other DOS USB Driver programs are written with the idea in

mind that in order to use them, you should create or update a new DOS

program specifically designed to interface with their Driver.  For

example, there is a DOS USB Driver program out there that has an example

of how to start writing a program to use a USB-to-Serial cable.  The

problem with this approach, of course, is that it requires you to write

a new program.  There simply aren't very many new DOS programs being

written these days.  I think what DOS users usually want is a way to use

their existing DOS programs with newer hardware.  There may be

absolutely nothing wrong with their existing program, except that it

doesn't "understand" USB.  In fact, there are some DOS programs that are

FAR superior to any Windows program that's ever been written, and there

are also some DOS programs that do useful or unique things that nobody's

ever even bothered writing a Windows program to accomplish.

These DOS USB Drivers are written with the full intention of "emulating"

the Legacy hardware that USB is trying to replace.  For example, USBKEYB

(the USB DOS keyboard Driver) emulates, as well as it can, a "real" PS2

keyboard.  A DOS program that expects to see a PS2 keyboard will think

that's what it's seeing, and nothing about the program needs to change

or be re-written in any way.  The problem, of course, is that emulating

hardware, especially in a DOS environment, is extremely haphazard and

difficult: sometimes it works and sometimes it doesn't.  I've garnered

some "tricks" over the years that I use to try and improve the emulation

for different kinds of Device/Interfaces, but the simple fact of the

matter is that it's not perfect.

DOS USB DRIVER INTRODUCTION            Page 42 of 186

THE DOS USB API

���������������

Many of these "features" that these DOS USB Drivers have and that others

don't are available because of the sophisticated API (Application

Programming Interface) that is installed with the Host Controller

Drivers (USBUHCI or USBUHCIL).  There is a separate and lengthy document

(USBAPI.DOC) that describes the API in detail.  USBAPI.DOC is intended

to be read by programmers, so if you're not a programmer the details it

contains will likely bore you to tears.  However, you may be interested

in reading the first several pages of it (before it gets into all of the

gory details about CPU registers and call-back notifications and such).

The first part of the document may give you some insight into the inner

workings and interactions of the programs that can help you use and

troubleshoot them much more effectively.

Because the API is described in detail in USBAPI.DOC, I will not go into

a lot of details here.  I will tell you , however, that the API is very

extensive.  It is extensive enough, in fact, that it allows almost

complete control and management of the USB busses.  Using only API

calls, you can develop a "map" and discover basic information about of

all of the Host Controllers, Root Hubs, and Device/Interfaces that are

attached to the busses.  Using only the API, you can also manage and

control all of the Physical Devices (Reset, Enable/Disable, Suspend,

etc.).  If someone had a burning desire, in fact, they could develop a

"DOS GUI USB Manager" program that would allow display and manipulation

of the entire USB structure on the computer.  I do not intend to write

such a program, but somebody else certainly could.

There is one specific thing I feel I should mention about the API here

(just in case you don't read the API documentation), because it can be

very confusing.  The Program Architecture allows for a total of 16 USB

Host Controllers to be managed (each requiring a separate Host

Controller Driver program).  Each managed Controller is assigned a "Host

Index" number between 0 and 15, but the numbers are NOT assigned

sequentially (starting with 0 and working up towards 15).  There is also

another type of index, called a "PCI Search Index", that is used to

identify individual Host Controllers at a hardware level.  A PCI Search

Index is only unique to each type of controller -- e.g., you can have a

UHCI PCI Search Index 0, an OHCI PCI Search Index 0, and an EHCI PCI

Search Index 0, all on the same computer.  PCI Search Indexes ARE

assigned sequentially, starting with 0, and are assigned by the PCI BIOS

as the computer is booting up.

DOS USB DRIVER INTRODUCTION            Page 43 of 186

In various screen displays associated with the DOS USB Drivers, you will

see both terms appear (Host Index and PCI Search Index), and they will

always be the same number (Host Index 2 is always PCI Search Index 2).

That is because there is only one type of Host Controller Driver

currently available, for UHCI Host Controllers.  The UHCI Host

Controller Driver program (called USBUHCI or USBUHCIL) purposely assigns

the Host Index to match the PCI Search Index, in the hopes of avoiding

confusion.  However, as other types of Host Controller Drivers are

developed in the future (for OHCI & EHCI), that will change.  Therefore,

although the Host Index and PCI Search Index are currently always the

same number, it is incorrect to assume they always will be.

DRIVER, SUPPORT, & TEST PROGRAMS

��������������������������������

The modular design of the DOS USB Driver architecture divides the

programs into three different "classes" or "types": Device/Interface

Drivers, Support Programs, and Test Programs.  We've already talked

about the Device/Interface Drivers (the TSR's that actually "talk to"

and control the various USB Controllers and Device/Interfaces).  There

are also several Support and Test Programs included as well.

There are two major purposes for a Support Program.  The first purpose

is to "translate" some sort of code (number) into a plain language

description that you (as a user) can understand.  In its attempt to be

efficient (to not transfer more data than is necessary to perform the

job), the USB architecture contains oodles of code numbers.  Just as an

example, there are several different types of General Descriptors that

are required to "describe" details about a Device/Interface to the

software on the computer.  Rather than transferring long strings of data

to describe the Descriptor Type, simple code numbers are used.  For

example, here are the code numbers associated with some of the General

Descriptor Types:

1 = Device Descriptor

2 = Configuration Descriptor

3 = String Descriptor

...

The second major purpose for a Support Program is to provide a "common"

place to store things that a whole bunch of other programs may need.

This ultimately makes each program smaller, and also makes things easier

to update.  Let's use the General Descriptors again as an example.

DOS USB DRIVER INTRODUCTION            Page 44 of 186

All Device/Interfaces have General Descriptors, and all Device/Interface

Drivers need to download the General Descriptors in order to configure

and control the Device/Interfaces.  All of the DOS USB Drivers have

options to let you view the Descriptors of the Device/Interfaces it is

controlling.  You can view the Descriptors just for casual interest, or

because something is not working correctly and you need to do some

troubleshooting.  It actually takes quite a bit of programming to decode

and print the various Descriptors in an understandable fashion.  So,

rather than "copying" everything it takes to do that into every single

program that needs it, it is much easier and more efficient to create a

separate program (or set of programs) dedicated to that purpose.  When a

Device/Interface Driver needs to decode and print a Descriptor for you

to view, it just "calls" the Support Program instead of trying to do it

internally.

In these DOS USB Drivers, the program that actually decodes the General

Descriptors is called USBSUPT1 (USB SUPporT program #1).  In turn, the

Support Program USBSUPT1 at various times may call other "sub-support"

programs, such as VENDORID (which translates USB or PCI Vendor ID's into

Vendor Names) and UNI2ASCI (which translates Unicode strings into ASCII

strings).  Of course, USBSUPT1 isn't the only program that may need the

functions contained in the "sub-support" programs, which is why they are

not "embedded" into USBSUPT1.  The Support Programs can be called by any

other program that wants to use them, not necessarily just the ones

contained in this DOS USB package.

The Support Programs are written as stand-alone DOS programs, rather

than simple "data files" or in some "special" format that can only be

read or executed in some unique way.  This allows you, if you want, to

run the Support Programs as a separate, stand-alone programs, and

discern all sorts of interesting things about the USB structure and

organization.

In addition, to Support Programs, there are also several Test Programs

included in this package.  A Test Program is designed to test a specific

Device/Interface Driver to make sure it is operating properly.  An

example of a Test Program is PS2MTEST (PS2 Mouse TEST), which is

specifically designed to test USBMOUSE (the USB Mouse Driver).  Test

programs are originally written as part of the program-writing process,

in order to test and troubleshot all of the features and compatibilities

of a Device/Interface Driver.  You can use them to test and troubleshoot

things as well, or you can just "play around" with them to see how

things work.

DOS USB DRIVER INTRODUCTION            Page 45 of 186

THE DOS PATH

������������

This DOS USB package contains many different programs (more than 20 of

them).  As noted above, some of the programs are interdependent on each

other to some degree (Device/Interface Drivers call Support Programs,

which may in turn call other Support Programs).  Because of this, I

recommend that you put all of the DOS USB related programs in the same

directory (I personally use C:\UTIL\USB), and that you put that

directory in your DOS PATH.

As a program is trying to locate the Support Program it needs, it first

searches in the same location (same directory) that it is in, and then

searches all directories in the PATH.  If it can't locate the correct

Support File, it will not crash, but will simply display some sort of

error message indicating that it couldn't find the Support Program it

was looking for.  Keeping all the files in the same directory keeps

things running smoothly and efficiently, and also makes it easier when

the time comes for program updates.

DOS USB DRIVER INTRODUCTION            Page 46 of 186

GENERAL OPTION SWITCHES

�����������������������

All programs will accept option switches from the DOS command line.  A

complete list of option switches is usually obtained with a "?" option

switch, e.g.:

USBUHCIL ?

Following are details of some of the "common" option switches, that are

available for most of the programs.

ALIASES

�������

Many programs have several aliases for each option switch (e.g., "Help"

is usually an alias for "?").  If the program supports aliases, there

will be an alias option switch listed somewhere in the details obtained

from the "?" option.

INSTALLATION INTO LOW MEMORY

����������������������������

If a program is listed as a Device/Interface Driver, it is also a TSR

(Terminate-but-Stay-Resident).  By default, the TSR's will automatically

try to load themselves into upper memory, if there is any upper memory

available (which there should be if you have EMM386.EXE installed).  All

of the TSR's have an option switch to override this behavior, and if the

appropriate option switch is selected they will install themselves into

Lower (Conventional) Memory even if Upper Memory is available.  For

example, to force installation of USBUHCI into Low Memory, do the

following:

USBUHCI LowMemory:Yes

This only has an effect when the TSR is first installed into memory.

After it has been installed, the option switch will not "move" the TSR

from Upper to Lower Memory (or vice-versa).  You can, however, "move"

the TSR from Upper to Lower Memory by Uninstalling it from memory, and

then re-installing it using the appropriate option switch.

DOS USB DRIVER INTRODUCTION            Page 47 of 186

UNINSTALL FROM MEMORY

���������������������

All TSR's have an option switch to Uninstall themselves from memory.

For example, to uninstall USBKEYB from memory, you would do the

following:

USBKEYB Uninstall

A TSR will not Uninstall itself from memory unless it is "safe" for it

to do so.  In most cases, what makes it unsafe is for another TSR to be

installed after it was, which is "blocking" it.  All of the TSR's in

this package use the IBM Interrupt Sharing Protocol, which prevents them

from "blocking" each other.  TSR's from other sources generally do not

use the IBM Interrupt Sharing Protocol; therefore, it is usually better

to install TSR's from other sources first, and install the USB TSR's

last.  That will allow you almost complete control of which TSR's are in

memory at any particular time.

"BAD" DEVICES

�������������

As discussed previously, the USB architecture is "flaky", and

Device/Interfaces do not always initialize or enumerate properly.  When

this happens, the Logical Device is declared to be "Bad" by the

software.  This does not necessarily mean that the Physical Device needs

to be scolded or replaced (though it could mean that).  Sometimes,

resetting the Physical Device (either by physically unplugging it and

then re-plugging it back in again, or by issuing a software Reset) will

fix the problem.

The Device/Interface Drivers (TSR's) all operate in the background,

while you are doing other things in the foreground.  In DOS, it is

usually not safe for a background process (TSR) to "write" anything to

the screen, since doing so could possibly screw up something that is

happening in the foreground.  Therefore, the only way safe way a

Device/Interface Driver has of informing you that something "bad" has

happened is to beep the speaker.

DOS USB DRIVER INTRODUCTION            Page 48 of 186

By default, all of the Device/Interface Drivers will beep the speaker

when they encounter a "bad" Device/Interface.  If you hear the beep, the

Device/Interface Driver programs have option switches to help you figure

out which Device/Interface is "bad", and show you details as to where

and why during the complex initialization process the problem occurred.

Note, however, that the details as to where and why the problem occurred

are somewhat cryptic, and will require you to scan through the even more

cryptic source code of the Device/Interface Driver to do any effective

troubleshooting.  The source code for all of the Device/Interface

Drivers is available if you want it, but if you are not familiar with

Assembly Language programming, it may confuse you more than it helps

you.

If you have a Physical Device that always seems to fail for some reason,

and is always beeping and annoying you, but you still want to keep

plugged in, you can turn off the beeping.  All of the Device/Interface

Drivers have an option switch to disable beeping for "bad" Devices.  For

example, to do it with USBUHCIL, you would do the following:

USBUHCIL Beep:No

In the Host Controller Drivers (USBUHCI & USBUHCIL), "bad" Logical

Devices are assigned a "fake" Address.  Therefore, to view details about

the "bad" Logical Devices, you simply tell USBUHCI or USBUHCIL that you

want to view details about all of the Logical Devices, and the "good"

ones and "bad" ones will all appear together.  For example:

USBUHCI AddressData

As noted above, however, even if a Logical Device makes it through the

initialization and enumeration process of the Host Controller Driver, it

may not necessarily make it through the configuration process of the

specific Device/Interface Driver.  In the specific Device/Interface

Drivers, the "bad" Device/Interfaces are stored separately from the

"good" Device/Interfaces.  There will be an option switch to view the

"bad" Device/Interfaces in the specific Device/Interface Driver.  For

example:

USBJSTIK Bad

DOS USB DRIVER INTRODUCTION            Page 49 of 186

VIEWING DESCRIPTORS

�������������������

All Device/Interface Drivers (TSR's) have switch options that allow you

to view the Descriptors that were downloaded from a particular

Device/Interface.  Something to keep in mind, however, is that any

particular Device/Interface Driver will usually only let you view the

Descriptors of the last Device/Interface that was enumerated or

configured.  In order to use as little memory as possible, the TSR will

only contain a buffer large enough to store the Descriptor data for the

Device/Interface it is currently enumerating or configuring.  When it is

done working with the Device/Interface, it will keep the data in the

buffer until a new Device/Interface is detected.  The old

Device/Interface data will then get overwritten by the new

Device/Interface data, and the old data is forgotten.

All Device/Interface Drivers have an option switch that will show you

the data that is contained in the buffer of the TSR, which will only be

valid for the last Device/Interface that was configured.  If you want to

view the details for a particular Device/Interface, and it was not the

last configured Device/Interface, you will need to Reset the Physical

Device you're interested in so that it becomes the last

Device/Interface.  You can Reset a Physical Device either by physically

unplugging it and then re-plugging it back in again, or by issuing a

Reset through the software.

For example, to view the General Descriptors for the last mouse that

USBMOUSE configured, you would do a:

USBMOUSE Descriptors

In addition to the General Descriptors, many Device/Interfaces also have

class- or vendor-specific Descriptors that are downloaded during the

configuration process.  If this is the case, the Device/Interface Driver

will have an option switch that will allow you to view the specific

Descriptors for the last configured Device/Interface.  For example,

mice, keyboards, and joysticks are all part of a larger "class" of

Device/Interfaces called Human Interface Devices or HID's.  HID's have a

special, class-specific Descriptor called a Report Descriptor that

"describes" details about the Device/Interface (numbers and types of

buttons, wheels, keys, axes, LED's, etc.).  To view the Report

Descriptor of the last keyboard configured by USBKEYB, for example, you

would do a:

USBKEYB ReportDescriptor

DOS USB DRIVER INTRODUCTION            Page 50 of 186

ENVIRONMENT VARIABLES & REDIRECTED INPUT

����������������������������������������

Depending on the complexity and use of a particular program, in addition

to accepting option switches from the command-line, it may also accept

option switches from an environment variable or a redirected input file.

The name of the environment variable is the same as the program name

(e.g., the environment variable USBMOUSE is associated with the

USBMOUSE.COM executable program).  The format of the options in the

environment variable is exactly the same as the format on the command

line.  For example, performing a:

SET USBMOUSE=Resolution:8

before running USBMOUSE would accomplish the same thing as:

USBMOUSE Resolution:8

Programs that use environment variables read the option switches from

the environment variable first, before reading them from the command-

line.  This allows environment variables to store the "default" option

switches you normally use, but you can "override" the environment

defaults at any time by using command-line option switches.

The format option switches in a redirected input file are similar to

those in an environment variable or on the command-line, but allows

organization (such as blank lines and indents) and comments (anything on

a line following a semicolon ";" is considered a comment).  For example,

the following lines could be in an ASCII text file called USBMOUSE.IN:

;This is USBMOUSE.IN, an example of a redirected input file

Resolution:8 ;Set Mouse Resolution to 8 Counts/mm

;This is the end of USBMOUSE.IN

If you then did the following:

USBMOUSE < USBMOUSE.IN

you would accomplish the same thing as:

USBMOUSE Resolution:8

The main reason for allowing redirected input files is to allow many

options switches to be entered at the same time, when there is simply

not enough space on a single DOS command line (limited to 126

characters) to do everything that needs to be done.  If a program allows

option switches from a redirected input file, it reads the environment

variable first, then the redirected input file, and finally the command-

line.  Options switches on the command-line are always the "final word".

DOS USB DRIVER INTRODUCTION            Page 51 of 186

INDIVIDUAL PROGRAMS

�������������������

Below are brief descriptions of each of the programs in the DOS USB

"package".  These descriptions are not complete, but are simply intended

to provide enough information for you to be able to use the program

effectively (this document is a "ReadMe" file, not complete

documentation).  This document will simply explain some of the more

"obscure" or difficult-to-understand options, and discuss major

interactions or idiosynchrocies related to the program's use.  Most of

the "general" stuff you need to know about USB, program interactions,

and "common" switch options are discussed above, and will not be

repeated.

The programs are in various stages of completion.  Some are considered

"complete", which means that there are no current ideas or plans to

update or upgrade them any further.  Some are essentially complete and

fully functional, but have a few minor "tweaks" and adjustments that are

planned for them.  Others are functional, but still have several major

items that need to be changed or adjusted.  Comments are welcomed on any

of the programs, no matter what stage of "completion" they are

considered to be in.

I would welcome some help from anybody interested in updating or adding

to any or all of these programs.  You can freely download the source

code for the programs, and as long as you continue to abide by the

license (never charge anyone in any way for the programs, source code,

or anything derived from them), you can do as you wish.  However, if you

decide to update or create your own drivers, I would like you to PLEASE

notify me of your intentions, and allow me to host your completed

program on my web site.  This will be especially critical if more than

one person decides to update the same program at the same time, which

(for obvious reasons) should be coordinated.

All of the programs are considered in alpha or beta.  It is expected

that anyone using these programs is a "beta tester", and already has a

basic understanding and mastery of common DOS and hardware mechanisms

(BIOS configuration, booting, batch files, PATH, environment variables,

TSR's, IRQ's, I/O Addresses, etc.).  If you do not already have a pretty

good handle on these things, you will probably get confused very

quickly.

DOS USB DRIVER INTRODUCTION            Page 52 of 186

One thing to keep in mind about the Device/Interface Drivers is that

because they are TSR's, they are constantly doing things in the

background while you are doing things (running other programs) in the

foreground.  Therefore, when you have one of the programs show you a

"status report" of some sort, it is simply giving you a "snapshot" of

what is happening in the TSR at the particular moment in time that you

ask it.  If you ask it to show you the status again a short time later

(even a single millisecond), it may show you something completely

different.  In USB, the Schedule is constantly being modified and

processed, data packets are constantly being sent back and forth,

Physical Devices are being inserted and removed and performing their

dedicated functions, etc.  The whole concept of something running in the

background where you can't "see" what's going on constantly may confuse

you at first, but you'll eventually get used to it.

USBUHCI & USBUHCIL

������������������

Version:     0.14

Prog Type:   Device/Interface Driver

Source Code: ASM (A386 by Eric Isaacson)

Status:      Complete, a few minor tweaks required

Environment: USBUHCI (for USBUHCI.COM)

USBUHCIL (for USBUHCIL.COM)

Redirection: Yes

Overlay:     USBUHCI.OVL (for USBUHCI.COM)

USBUHCIL.OVL (for USBUHCIL.COM)

Purpose:     Driver for Universal (UHCI) USB Host Controllers

TSR Memory:  USBUHCIL: 29.7 kB (first Instance)

12.5 kB (instances 2+)

USBUHCI:  42.6 kB (first Instance)

24.6 kB (instances 2+)

USBUHCI is the full USB Host Controller Driver program.  USBUHCIL is the

"Lite" or "Limited" version.  The difference between the two is that

USBUHCI can handle a full 126 Logical Devices (Addresses), and allows

Isochronous Transactions to be scheduled.  USBUHCIL can only handle 16

Logical Devices, and does not allow the scheduling of Isochronous

Transactions.  Both programs implement the full Application Programming

Interface (API) as described in USBAPI.DOC, and are equivalent in every

other way.

From now on, when we are referring to one or the other of the programs

(USBUHCI or USBUHCIL), we will use the name of the individual program.

When we are referring to a Host Controller Driver in general (and can be

actually referring to either program), we will use USBUHCI{L}.

DOS USB DRIVER INTRODUCTION            Page 53 of 186

USBUHCIL uses substantially less memory than USBUHCI, and should be

sufficient for most applications.  Most computers only have a few

Physical Devices plugged into any particular USB bus, and there are no

Drivers available yet for any type of Isochronous Device/Interface (such

as a Video Camera).  Note, however, that because there has not yet been

a Device/Interface Driver developed for a "real" Isochronous Device, the

Isochronous capabilities of USBUHCI have not been tested with an actual

Physical Device and may not work correctly.

Both USBUHCI & USBUHCIL require an overlay file to operate properly.

The overlay file for USBUHCI is called USBUHCI.OVL, and the overlay file

for USBUHCIL is called USBUHCIL.OVL.  The overlay file must be in the

same directory as the main executable file (USBUHCI.COM or

USBUHCIL.COM).  If USBUHCI{L} cannot find the overlay file (or it is the

wrong version of overlay file), the program will not work.

PCI Search Index

����������������

A modern computer will typically have more than one UHCI Host

Controller installed.  USBUHCI{L} will only control one UHCI Host

Controller.  However, more than copy (instance) of USBUHCI{L} can be

installed in memory at the same time, each controlling a different

UHCI Host Controller.  When you have more than one instance of

USBUHCI{L} in memory at the same time, you need to be able to

identify which one is which, and which one is associated with which

physical Host Controller.  This is done with an identifier called a

"PCI Search Index".

A PCI Search Index is a 0-based index number that is assigned by the

PCI BIOS to enumerate and identify the different UHCI Host

Controllers.  The "first" Host Controller is given PCI Search Index

0, the "second" is given Index 1, etc.  If you only have one UHCI

Host Controller, it is assigned PCI Search Index 0.  The USBHOSTS

program (below, page 82) will show you the details of all the

different Host Controllers on your computer, including the PCI Search

Indexes.

Keep in mind, however, that you do not necessarily need install an

instance of USBUHCI{L} for each and every physical UHCI Host

Controller you have.  You only need to install USBUHCI{L} for the

Host Controllers that have USB Devices attached that you actually

want to be able to use and control in DOS.  For example, to issue

commands for the UHCI Host Controller with PCI Search Index 2 (the

third one), you would do a:

USBUHCIL Index 2 {other Options here}

DOS USB DRIVER INTRODUCTION            Page 54 of 186

If you do not give USBUHCI{L} an Index number with an option switch

(either in the USBUHCI{L} environment variable, redirected input

file, or command-line), it assumes you want to use Index 0.  Also

note that the first instance of USBUHCI{L} you install into memory

(if you install more than one) does not necessarily needed to be

associated with Host Controller Index 0.  You can install the

different instances of USBUHCI{L} in any order you want.

Memory Usage

������������

The first instance of USBUHCI{L} that gets installed into memory uses

quite a bit more memory than the later instances.  This is because

all of the program code it takes to manage a Host Controller

(including the sophisticated Application Programming Interface or

API) is installed in the first instance.  In addition to the code,

the first instance also contains all of the data required to manage

the "first" Host Controller (data such as the Schedule and "map" of

the attached Devices).  After the first instance is installed, any

other instances that are installed "reuse" the program code that was

installed in the first instance.  The only memory that the later

instances use is the memory needed to store the data that is unique

to the particular Host Controller -- they do not need another copy of

the program code.  This allows the Host Controller Drivers to use as

little total memory as possible.

However, this can also cause you some problems when you want to

Uninstall the first instance of USBUHCI{L}.  Because the first

instance is the one the contains all of the program code (including

all of the code for the API), it cannot be Uninstalled unless all the

other programs that depend on it are Uninstalled first.  This

obviously includes all further instances of USBUHCI{L}, but also

includes any USB Device Drivers you have installed (which access the

API).  The program architecture normally lets you install and

Uninstall Device Drivers at any time and in any order, but that is

not true for the first Host Controller Driver.  It is the

"foundation" that all of the other programs depend on, and therefore

must the last program that is Uninstalled.  Remember also that the

first instance of USBUHCI{L} installed into memory does not

necessarily need to be associated with the "first" UHCI Host

Controller (the one with PCI Search Index 0).  The two concepts of

"first" are very different from each other.

DOS USB DRIVER INTRODUCTION            Page 55 of 186

IRQ & I/O Resources

�������������������

Each UHCI controller requires two hardware resources from the

computer: an IRQ (Interrupt ReQuest line, a.k.a. hardware interrupt

number), and 32 bytes of I/O (Input/Output) space.  Normally, these

are assigned and configured during the boot process by the BIOS.  If

they are not assigned by the BIOS, USBUHCI{L} will try to assign them

automatically.  For the IRQ, USBUHCI{L} will use IRQ #9.  For the I/O

space, USBUHCI{L} will start searching at I/O address 400h, and look

for a 32-byte section of I/O space that appears to be unused.  Again,

USBUHCI{L} will only do this if the IRQ & I/O space have not already

been assigned by the BIOS.

If the IRQ and I/O space were assigned by the BIOS, they will usually

(but not always) work just fine and you will not have any conflicts.

If USBUHCI{L} automatically assigns them, they will also probably

also work just fine, but it's possible that they won't.  For example,

if USBUHCI{L} is using IRQ #9, but IRQ #9 is already being used by

some other port or device on the computer, and is not configured as a

Sharable IRQ, trouble can result.  When USBUHCI{L} starts using IRQ

#9, the computer will either crash or the other device will just stop

working.  If there is no other device currently using IRQ #9, or it

is set up as a Sharable IRQ, there should not be any problems.

There is one particular IRQ number that will almost always cause you

problems: IRQ #12.  On all modern computers, IRQ #12 is reserved for

the PS2 mouse, and should NEVER be used for anything except the

mouse.  Some BIOS s set up IRQ #12 to be used for other things

besides (or in addition to) the mouse, including using IRQ #12 for a

USB host controller.  This is almost always guaranteed to cause you

problems, especially if you intend to use a USB mouse in DOS.  If one

(or more) of your USB host controllers is using IRQ #12, and you ever

intend to use a USB mouse on that computer, you should change the IRQ

to something else.

You are less likely to have a conflict in the I/O space than you are

with the IRQ, but a conflict can happen there as well.  Unlike IRQ's,

I/O space cannot be shared.  As USBUHCI{L} is scanning through the

I/O space to find an empty range, it is possible (though very

unlikely) that it will "screw up" some port or device that's already

using the space it's trying to scan.

DOS USB DRIVER INTRODUCTION            Page 56 of 186

There is also a possibility that the IRQ or I/O space as assigned by

the BIOS are actually wrong (that is, what the BIOS tells you it did

and what it actually did are two different things).  This actually

happened to me on a Sony laptop, and it took me quite awhile to

figure out what the problem was.  Like many modern computers, the

laptop uses highly integrated chip sets on the computer.  Rather than

having individual chips for each function (such as a separate chip

for each USB Host Controller), there is a single chip (manufactured

by Intel) that combines a whole bunch of separate functions onto a

single chip.  In my case, the chip contains several USB Host

Controllers, along with several other functions.

In the specification for UHCI Host Controllers (written by Intel),

there is a specific location in the UHCI configuration space where

the IRQ number is stored.  In a "regular" UHCI Controller, that is

also how you configure the IRQ that is actually used.  If you change

(write to) the IRQ number stored there, it literally changes the IRQ

line that the Host Controller uses.  Unfortunately, in the integrated

chip set used on the Sony laptop, that is not what happens.  The IRQ

number that is stored in the UHCI configuration space is simply there

"for show".  Changing it does not change anything -- the actual IRQ

number is stored somewhere else in the configuration space, and

exactly where it's stored is specific to that particular model of

Intel chip (it is not "standard" the way UHCI is supposed to be).  As

the Sony BIOS is setting up the USB Host Controllers during the boot

process, it is supposed to copy the "real" IRQ number (stored in the

proprietary location) into the (useless) UHCI configuration space.

Well, the Sony BIOS does in fact put a number into the UHCI

configuration space, but it puts the WRONG number.

As you might expect, it took me some time to track this problem down.

At first, I though there was something wrong with the USBUHCI{L}

program, and just blew it off for a while.  When I finally got around

to earnestly troubleshooting things, I discovered that the computer

was lying to me about what IRQ it was using, and that if I tried

changing the IRQ in the UHCI configuration space, it didn't actually

do anything.  Incidentally, that's why I'm including the IRQ program

(below, page 86).  It can be useful in helping you track down and

trace IRQ's for situations such as this.

DOS USB DRIVER INTRODUCTION            Page 57 of 186

USBUHCI{L} has option switches that let you configure the IRQ and I/O

space, if you want to do that.  Note, however, that if you use either

of these option switches, they will override (overwrite) the settings

of the BIOS, and USBUHCI{L} will not try to automatically configure

anything.  USBUHCI{L} considers the IRQ and I/O space numbers you

give it to be "gospel", and does no checking or testing of them.

USBUHCI{L} simply stores the number(s) that you give it into the UHCI

configuration space.  You can either use the switch options to change

or manually configure the IRQ and I/O resources (usually to avoid

conflicts), or, as in the case of my Sony laptop, to make sure that

what it says in the UHCI configuration space (which is what

USBUHCI{L} uses) matches how the computer hardware is actually set

up.

The IRQ number must be between 3 and 15 (though you should usually

avoid IRQ 12 if you intend to use a USB mouse), e.g.:

USBUHCI IRQ 11 {other options}

The I/O base address must be between 100h and FFE0h, and must be

evenly divisible by 32 (20h), e.g.:

USBUHCIL IOAddress 420h {other options}

Note that the IRQ and I/O space option switches only have an effect

when USBUHCI{L} is first being installed into memory.  After it has

been installed, you cannot change the IRQ or I/O resources without

Uninstalling USBUHCI{L} from memory completely and the re-installing

it with the new settings.

Other Host Controller Drivers

�����������������������������

There can be only one software Driver managing and scheduling each

USB Host Controller.  The Driver can be the BIOS (Legacy Support),

USBUHCI{L}, or another brand of DOS Driver (incompatible with

USBUHCI{L}).  As you are attempting to install USBUHCI{L} into memory

to start managing a particular UHCI-compatible Host Controller,

USBUHCI{L} will detect if there is another Driver already installed.

If there is, USBUHCI{L} will stop and ask you if you really want to

"disable" the other Driver in order to install USBUHCI{L}.  If you

tell USBUHCI{L} to continue with the installation, it will save the

status of the current Driver before installing itself into memory.

Later on, when you Uninstall USBUHCI{L} from memory, USBUHCI{L} will

attempt to "restore" the state of the original Driver.

Unfortunately, this doesn't usually return things back exactly to the

way they were before you installed USBUHCI{L}, at least not in the

way you think it would.

DOS USB DRIVER INTRODUCTION            Page 58 of 186

While USBUHCI{L} was installing, it had to Reset the Host Controller

hardware and any Physical Devices that were attached to the bus

(possibly multiple times).  While USBUHCI{L} is Uninstalling, it must

also reset the Host Controller hardware.  As a result of all of this

manipulation, the Host Controller hardware and the original (non-

USBUHCI{L}) Host Controller Driver have become "unsynchronized", and

the software doesn't actually know what the hardware looks like any

more.  If you want any of the Physical Devices to start working

again, you must usually tell the original Host Controller Driver to

"reset" itself (some Host Controller Drivers provide a way to do that

and some don't -- Legacy Support Drivers never do).  If you can't (or

don't know how to) reset the original Driver, you can try unplugging

and re-plugging each of the Physical Devices in an attempt to reset

them.  Of course, that will only help if the original Driver supports

plug-and-play, which most of them do not.

Instead of having USBUHCI{L} stop to ask you if it should continue

with the installation when it detects that another Driver is managing

the Host Controller, there are option switches you can use to tell

USBUHCI{L} to do it automatically.  If you want USBUHCI{L} to

automatically replace a BIOS (Legacy Support) Driver, you would do

the following:

USBUHCI DisableLegacySupport {other options}

If you want USBUHCI{L} to automatically replace another (non-BIOS)

Driver, you would do the following:

USBUHCIL DisableOtherProgram

Disabling the Companion EHCI Host Controller

��������������������������������������������

Most modern computers are capable of USB 2.0, which can transfer data

at much higher speeds than USB 1.0.  USB 2.0 is designed to be

backwards-compatible, which means that USB 2.0-capable devices can

operate on a USB 1.x bus (managed by a USB 1.x host controller), and

vice versa.  The mechanism used to perform this "sleight-of-hand" is

what USB calls "Companion Controllers".

A USB 2.0 host controller actually has two different "sections" in

it.  It has a high-speed (USB 2.0) section, which uses the EHCI

(Enhanced) protocol, and also has a full/low speed (USB 1.x) section,

which uses either the UHCI (Universal) or OHCI (Open) protocol.  The

two "companion" sections are integrated together as one unit, and

they interact with each other very closely.

DOS USB DRIVER INTRODUCTION            Page 59 of 186

Assuming all of the correct software is installed and everything is

working like it should, this is what happens when a Device is

attached to a port on the root hub (the root hub is also a separate

"section" of the host controller).  The EHCI (high-speed) section

detects that a Device is attached, and determines whether the Device

is USB 2.0-capable or not.  If it is, the EHCI section keeps control

of the Device by keeping control of the root hub port (it never lets

the full/low-speed section know that the Device is even there).  If

the Device is not high-speed-capable (if it is a USB 1.x Device), the

EHCI section "flips a switch" and transfers control of the Device to

the full/low speed section (UHCI or OHCI).

In most USB 2.0 implementations, the sole criteria used to determine

which host controller (high-speed or full/low speed) should control

the Device is the speed capability of the Device.  It doesn't matter

if you can actually do anything useful with the Device or not -- all

that matters is whether it is "fast".  This actually causes a problem

in some cases when you're using USBUHCI{L}.  I'll provide some

specific examples from my own experience.

I have a relatively new computer that is USB 2.0-capable, and does

not have any legacy PS2 ports for the keyboard or mouse (you must use

a USB keyboard and mouse).  Obviously, the BIOS is set up with Legacy

Support to handle the mouse and the keyboard, and also includes

support for USB disk drives.  However, the USB support in the BIOS

does not allow for any other types of USB Devices, like printers or

joysticks.  Unfortunately, I have USB printers and joysticks that I

want to use sometimes with my DOS programs, but that is impossible to

do with the BIOS support.  The BIOS manufacturers apparently expect

you to never run "real DOS" any more, but rather expect you to run

DOS emulators under Windows or Linux.  This is completely

unacceptable to me.

Because my printer is USB 2-0 capable, the BIOS of the computer takes

control of the printer (even though I don't want it to).  The BIOS

never relinquishes control of the printer to the full/low-speed

section of the host controller (UHCI), where I can install the

drivers that let me actually use the printer.  I also cannot

completely disable USB Legacy Support in the BIOS, because the USB

keyboard is required to be able to boot.  So, what I need is a way to

completely disable the EHCI (high-speed) section of the host

controller after the computer has booted up, which forces all of the

Devices to the full/low-speed (UHCI) section, so that I can install

the DOS drivers, so that I can actually use the printer.

DOS USB DRIVER INTRODUCTION            Page 60 of 186

On the same computer, I also have another time when I need to disable

the EHCI controller and force the USB devices onto the full/low-speed

(UHCI) controllers.  As noted before, the BIOS does support USB disk

drives, and also supports plug-and-play of the USB devices, but only

to a limited degree.  The BIOS supports USB disks in DOS (DOS assigns

a drive letter to them) only if a USB disk is plugged in when the

computer is booted.  After the BIOS is done and DOS has taken over,

it's too late to assign new DOS drive letters.  The BIOS does let you

insert and change USB disks after DOS has booted, but can't assign a

new drive letter to them.  So, instead of plugging in the USB disk

and rebooting, I instead simply disable the EHCI controller so that I

can install the DOS USB drivers contained herein and use the disk(s)

(the disk driver, USBDRIVE, can assign new drive letters after DOS

has booted).  The disadvantage to this is that the disks only operate

at full-speed (12 Mbps) rather than high-speed (480 Mbps), since an

EHCI driver has not yet been developed.  However, unless I'm needing

transfer a bunch of data, it's much more acceptable to me to put up

with the lower speeds for a little while than it is to wait for the

computer to reboot.

I have provided a way to disable an EHCI controller, which forces all

of the USB devices onto the UHCI controllers, thereby allowing you to

use the drivers contained herein.  The option to do this, however, is

part of the USBHOSTS program (below, page 82), rather than part of

this program (USBUHCI{L}).  You can read about the STOP option in

USBHOSTS below on page 83.

Debugging Program Installation

������������������������������

While USBUHCI{L} is installing itself into memory as a TSR, or while

it is uninstalling itself from memory, there may be problems that

keep it from being successful.  When this happens, USBUHCI{L} is

usually able to display some sort of error message telling you what's

wrong so that you can try and fix it.  Unfortunately, in rare cases,

USBUHCI{L} will simply "lock up" when a problem occurs during

installation or uninstallation.  The usual cause of this is a UHCI

(or companion EHCI) host controller that doesn't exactly comply with

the specifications, or a problem in the BIOS with the way it

implements USB Legacy Support for keyboards, mice, and/or disk

drives.

USBUHCI{L} has an option that helps to narrow down exactly where the

problem is occurring, to help you troubleshoot things.  What the

option does is display (on screen) each of the major steps that

USBUHCI{L} is currently performing during the installation or

uninstallation process.  If USBUHCI{L} "locks up" on you, try doing

the same thing again with the Program Debugging option turned on:

USBUHCIL ProgramDebug {other options}

DOS USB DRIVER INTRODUCTION            Page 61 of 186

This will let you see what USBUHCI{L} is trying to do when the "lock

up" happens, helping you to figure out what exactly might be

"broken".  On my most recent computer, I had a problem when trying to

uninstall USBUHCI{L}, and I used the ProgramDebug option to help

isolate the problem and create a patch for it.

You can also use the ProgramDebug option to slow down the

installation or uninstallation process, by adding a number after the

option.  The number must be between 0 and 2000 (with the default

being 0), and is the number of milliseconds to delay between each

major step.  For example:

USBUHCIL ProgramDebug:10 {other options}

This will cause USBUHCI{L} to delay 10 milliseconds between each of

the major steps, slowing down the installation or uninstallation

process slightly.  Slowing down the process may actually cause it to

work (to not "lock up" any more), though I've never seen that happen

in my limited experience.  Slowing things down also helps you see

what''s going on a little bit clearer, instead of having things flash

by on the screen so fast you can't make any sense out of it.

Please do not confuse this ProgramDebug option with the Debug option

discussed below (page 72).  This ProgramDebug option is designed to

help troubleshoot installation and uninstallation of the executable

program file itself (USBUHCI.COM or USBUHCIL.COM), while the Debug

option below enables the Debug/SingleStep mode that is built into the

UHCI host hardware.

DOS USB DRIVER INTRODUCTION            Page 62 of 186

Postponing Device Enumeration

�����������������������������

After USBUHCI{L} is installed into memory, it normally starts

immediately enumerating any Logical Devices it finds attached to the

bus.  However, there may be times when don't want the enumeration

process to begin right away.  The most likely reason for this would

be if you have a Physical Device that needs several seconds to

"stabilize" before it will enumerate properly.

I have a USB Printer attached to one of my older computers, and it is

constantly giving me trouble.  It only initializes and enumerates

properly about 10% of the time.  When it doesn't enumerate properly,

I have to reset it (usually multiple times) before it will finally

start working.  I don't use the computer (or the printer) very often,

so I just put up with it instead of trying to replace it.  I thought

that maybe if I just gave the printer more time to "stabilize" before

I tried to enumerate it, it may work a lot better.  So, I added an

option switch to USBUHCI{L} to wait a little while before starting to

enumerate things instead of doing it right away.  Well, as it turns,

out, it didn't help me at all with my printer -- and it still really

annoys me when I try to use it.

In spite of the fact that it didn't help me any, I've left the switch

option in USBUHCI{L} to delay the start of the enumeration process.

You may want to try it if you're having problems with a particular

Physical Device, or if you want to delay things for some other

reason.  In the option switch, you need to tell USBUHCI{L} how many

deciseconds to delay (10 deciseconds = 1 second).  The number of

deciseconds can be between 0 (no delay) and 255 (25.5 seconds).  For

example, to delay the start of the enumeration process for 3.5

seconds, you would do a:

USBUHCIL Pause 35 {other options}

You must use this option switch when you are first installing

USBUHCI{L} into memory.  After it has already been installed, it is

too late to tell it to delay anything.

While the implementation of this option does not require a lot of

memory, it does use some, and as a result the USBUHCI{L} TSR uses

more memory than it would otherwise.  Since the option does not

appear to do anything useful (at least to me), it may be

unnecessarily wasting memory.  If nobody provides feedback stating

that the option is somehow beneficial, it will probably be removed

when the official (non-beta) release of USBUHCI{L} is issued.

DOS USB DRIVER INTRODUCTION            Page 63 of 186

Bus Timing Manipulation

�����������������������

The general concept of USB bus Timing is discussed above in the "BUS

TIMING" section (page 32), and those details will not be repeated

here.

USBUHCI{L} provides an option switch that lets you temporarily be the

"Owner" of the bus Timing.  That is, you can slow down or speed up

the bus by a small amount (make it run at a slightly different speed

than the normal 12 Mbps).  You can do this either because of a

legitimate need, or just to "play around" and see what happens.

Note, however, that if there is already a program (such as an

Isochronous Device/Interface Driver) in control of the bus Timing,

the option switch will not do anything.  UHCI Host Controllers allow

you to control the bus Timing by setting the number of bits per Frame

to a number between 11,936 and 12,063.  For example, to change the

bus Timing from whatever it is to 12050 bits per Frame, you would do

the following:

USBUHCI Timing 12050

Device & Host Manipulation

��������������������������

There are several different ways you can "manipulate" Physical

Devices and Hosts in the USB architecture.  These are discussed in

various places throughout this document, particularly in the sections

called "DISABLING & SUSPENDING DEVICES" (page 21) and "STARTING &

STOPPING HOSTS" (page 24).  Those details will not be repeated here.

This section is simply intended to detail the option switches you may

use in USBUHCI{L} to perform those functions.

The way you specify which specific Logical Device you are trying to

manipulate is to provide a Device Address.  Recall that a "real"

Logical Device is always assigned an Address between 2 and 17 (in

USBUHCIL), or between 2 and 127 (in USBUHCI).  Address 1 is always

assigned to the Root Hub, which for our discussion here will be

synonymous with the Host Controller.  That is, Address 1 will

actually be referring to the logical entity called the Root Hub when

it is a Hub-related function (such as Suspending Devices), and will

actually be referring to the logical entity called the Host

Controller when it is a Controller-related function (such as

Resetting the Host Controller).

In addition, we will consider Address 0 to be synonymous with Address

1 (it will refer to the Root Hub/Host Controller).  Technically,

Address 0 is the Address temporarily assumed by a Logical Device that

is in the middle of the initialization & enumeration process, but in

these option switches we will not use it that way.

DOS USB DRIVER INTRODUCTION            Page 64 of 186

As a final point, an Address larger than the maximum allowable

Address (17 for USBUHCIL and 127 for USBUHCI) will usually perform

the appropriate function for ALL Host Controllers that are being

managed, not just the one that is identified with the PCI Search

Index as declared in the command-line switch.  For example:

USBUHCIL Index 2 Reset 500

will Reset all of the Host Controllers that are currently being

managed (including those with PCI Search Indexes 0 and 1).  This will

Reset and re-initialize every Physical Device you have.  Compare this

to:

USBUHCIL Index 2 Reset 1

which will just Reset and re-initialize the Host Controller with PCI

Search Index 2, and:

USBUHCIL Index 2 Reset 5

will just Reset the individual Logical Device that has Address 5 on

the bus associated with PCI Search Index 2.

Here are the options switches you can use with USBUHCI{L} and a brief

description of what the option switch does.

Enable

������

USBUHCI  Enable {0-1}

USBUHCIL Enable {0-1}

Starts the Host Controller running normally, whether it was

Stopped (paused), in Debug Mode, or the Root Hub was in Global

Suspend mode.

USBUHCI  Enable {128-65535}

USBUHCIL Enable { 18-65535}

Starts ALL Host Controllers running normally, whether they were

Stopped (paused), in Debug Mode, or the Root Hubs were in

Global Suspend mode.

DOS USB DRIVER INTRODUCTION            Page 65 of 186

USBUHCI  Enable {2-127}

USBUHCIL Enable {2- 17}

Enables the appropriate Physical Device if it was Disabled.  A

Physical Device that was Disabled will NOT start running

"normally" again by simply Enabling it, however.  It must

instead be Reset in order to be re-initialized and re-

enumerated.

Resumes the appropriate Physical Device if it was Suspended.  A

Device that was Suspended WILL start running normally again by

issuing an Enable (or Resume) command.

Disable

�������

USBUHCI  Disable {0-1}

USBUHCIL Disable {0-1}

Stops the Host Controller from generating frames and sending

data across the bus.

USBUHCI  Disable {128-65535}

USBUHCIL Disable { 18-65535}

Stops ALL Host Controllers from generating frames and sending

data across the busses.

USBUHCI  Disable {2-127}

USBUHCIL Disable {2- 17}

Disables the appropriate Physical Device.  To recover from a

Disable, a Physical Device must be Reset in order to start

running "normally" again.  Simply Enabling it will not cause it

to start working.

Suspend

�������

USBUHCI  Suspend {0-1}

USBUHCIL Suspend {0-1}

Causes the Root Hub to send a Global Suspend signal across the

bus, putting ALL Physical Devices on the bus into Suspend mode.

USBUHCI  Suspend {128-65535}

USBUHCIL Suspend { 18-65535}

Causes ALL Root Hubs to send Global Suspend signals across ALL

busses, putting ALL Physical Devices on ALL busses into Suspend

mode.

DOS USB DRIVER INTRODUCTION            Page 66 of 186

USBUHCI  Suspend {2-127}

USBUHCIL Suspend {2- 17}

Puts the appropriate Physical Device into Suspend mode.  A

Physical Device can "recover" from Suspend mode either by

issuing a Resume or an Enable command.

Resume

������

USBUHCI  Resume {0-1}

USBUHCIL Resume {0-1}

Causes the Root Hub to send a Global Resume signal across the

bus, Resuming ALL Physical Devices on the bus that are in

Suspend mode.

USBUHCI  Resume {128-65535}

USBUHCIL Resume { 18-65535}

Causes ALL Root Hubs to send Global Resume signals across ALL

busses, Resuming ALL Physical Devices on ALL busses that are in

Suspend mode.

USBUHCI  Resume {2-127}

USBUHCIL Resume {2- 17}

Sends a Resume signal to the appropriate Physical Device.  This

will cause a Physical Device to "recover" if it was in Suspend

mode.

DOS USB DRIVER INTRODUCTION            Page 67 of 186

Remote Wakeup

�������������

Some USB Devices support a function called Remote Wakeup.  This

allows the Device to indirectly issue a Resume signal on the bus

when the Device is manipulated.  This is discussed in more detail

above in the "DISABLING & SUSPENDING DEVICES" section (page 21).

Host Controllers do not provide Remote Wakeup (they are not

"remote"), only Devices.  Therefore, Addresses 0-1 (Host

Controller/Root Hub) and Addresses greater than the maximum

legitimate address (ALL Host Controllers/Root Hubs) are not

allowed in this option switch.

USBUHCI  RemoteWakeup {2-127} On|Off

USBUHCIL RemoteWakeup {2- 17} On:Off

By default, USBUHCI{L} enables the Remote Wakeup feature in all

Devices that support it.  This option allows you to manipulate

(enable or disable) the Remote Wakeup feature for an individual

Device.  You can view the current state of the Remote Wakeup

feature with the Addresses option, discussed below in "Attached

Devices" (page 73).

Reset

�����

USBUHCI  Reset {0-1}

USBUHCIL Reset {0-1}

Causes the Host Controller to perform a complete reset of

itself, and "reboot" into its default state.  All Physical

Devices attached to the bus will be Disabled while the Host

Controller is re-initializing.  After the Host Controller has

finished Resetting itself, all Physical Devices attached to the

bus will be re-initialized & re-enumerated.

USBUHCI  Reset {128-65535}

USBUHCIL Reset { 18-65535}

Causes ALL Host Controllers to perform a complete reset of

themselves, and "reboot" into their default states.  ALL

Physical Devices attached to ALL busses will be Disabled while

the Host Controllers are re-initializing.  After the Host

Controllers have finished Resetting themselves, ALL Physical

Devices attached to ALL busses will be re-initialized & re-

enumerated.

USBUHCI  Reset {2-127}

USBUHCIL Reset {2- 17}

Sends a Reset signal to the appropriate Physical Device.  This

will cause the Physical Device to be Disabled, and then go

through the entire initialization & enumeration process again.

DOS USB DRIVER INTRODUCTION            Page 68 of 186

Configuration Value

�������������������

By default, USBUHCI{L} will configure a Device to configuration

value 1 during the enumeration process.  Some Devices have more

than one possible configuration value (though I have personally

never seen one), and it is also possible to set a Device to

configuration value 0 (unconfigured).  Host Controllers do not

support multiple configurations, and cannot be unconfigured (at

least not in the same sense that Devices are).  Therefore,

Addresses 0-1 (Host Controller/Root Hub) and Addresses greater

than the maximum legitimate address (ALL Host Controllers/Root

Hubs) are not allowed in this option switch.

USBUHCI  ConfigurationValue {2-127} #

USBUHCIL ConfigurationValue {2-17}  #

The configuration value (#) must be a legitimate configuration

value for the Device, usually 0 (unconfigured) or 1 (default

configuration).  The Device must be Reset in order to change

the configuration value, so the reconfiguration process

inherently includes a Device Reset.  If the Device does not

accept the requested configuration value, USBUHCI{L} will use

the default configuration value (1).

PowerOn, PowerOff, & PowerReset

�������������������������������

UHCI Host Controllers (Root Hubs) do not support power control.

Therefore, Addresses 0-1 (Host Controller/Root Hub) and Addresses

greater than the maximum legitimate address (ALL Host

Controllers/Root Hubs) are not allowed in these option switches.

USBUHCI  PowerOn {2-127}

USBUHCIL PowerOn {2- 17}

If the Hub that the Physical Device is attached to supports

per-port-power-control, the Hub will turn on the 5 VDC bus

power to the Physical Device.  If the Hub does NOT support per-

port-power-control, this does nothing.

If the Physical Device is bus-powered, and the power is turned

off before this PowerOn command is issued, the Physical Device

will have been disconnected as a result of not having any

power.  After the Physical Device receives power and becomes

reconnected to the bus, it will go through the entire

initialization and enumeration process again.

If the Physical Device is self-powered, enabling power to the

Physical Device will have no effect.

DOS USB DRIVER INTRODUCTION            Page 69 of 186

USBUHCI  PowerOff {2-127}

USBUHCIL PowerOff {2- 17}

If the Hub that the Physical Device is attached to supports

per-port-power-control, the Hub will turn off the 5 VDC bus

power to the Physical Device.  If the Hub does NOT support per-

port-power-control, this does nothing.

If the Physical Device is bus-powered, turning off power will

cause the Physical Device to disconnect from the Bus (similar

to unplugging the Physical Device).  Restoring power to the

Physical Device (using the PowerOn option switch discussed

above) will "reconnect" it to the bus, causing it to go through

the initialization & enumeration process again.

If the Physical Device is self-powered, disabling power to the

Physical Device will have no effect.

USBUHCI  PowerReset {2-127}

USBUHCIL PowerReset {2- 17}

If the Hub that the Physical Device is attached to supports

per-port-power-control, the Hub will turn off the 5 VDC bus

power to the Physical Device, wait approximately 250

milliseconds, and then restore power to the Physical Device.

If the Hub does NOT support per-port-power-control, this does

nothing.

If the Physical Device is bus-powered, resetting power will

have the same effect as unplugging the Physical Device from the

bus and then re-plugging the Physical Device back in again

(causing it to go through the initialization & enumeration

process again).  This is a more "complete" Reset process than

the "software" Reset discussed a few paragraphs back, since the

Physical Device will actually lose power during the transition.

However, a Power Reset requires both that the Hub that the

Physical Device is attached to supports per-port-power-control,

and that the Physical Device itself is bus-powered.  Therefore,

it will only work in certain situations.

If the Physical Device is self-powered, resetting power to the

Physical Device will have no effect.

DOS USB DRIVER INTRODUCTION            Page 70 of 186

Test Mode

���������

USB Devices can support something called Test Mode, which (as the

name implies) allows the Device to be tested in special ways.

There is usually not a good reason to put a Device into Test Mode

unless you are the manufacturer of the Device in a special test

environment.  Host Controllers have a Debug Mode (discussed

directly below on page 72), but do not have a Test Mode the way

Devices do.  Therefore, Addresses 0-1 (Host Controller/Root Hub)

and Addresses greater than the maximum legitimate address (ALL

Host Controllers/Root Hubs) are not allowed in this option switch.

USBUHCI  TestMode {2-127} On|Off

USBUHCIL TestMode {2- 17} On:Off

You can view the current state of the Test Mode feature with

the Addresses option, discussed below in "Attached Devices"

(page 73).

Resetting "Bad" Devices

�����������������������

A noted in various places in this document, there are times when a

Device does not enumerate properly and is declared to be "bad".

Sometimes, a bad Device can be made "good" by simply resetting it,

causing it to go through the initialization and enumeration process

again for a second (or third or fourth or ... time).  It can get

really annoying.

Normally, to Reset a Device (whether it's good or bad), you must

provide USBUHCI{L} a specific device Address that you want it to

Reset, and USBUHCI{L} will Reset that individual Device.  Because

trying to figure out what the Address of the bad Device is can be a

little bit cumbersome and annoying, USBUHCI{L} provides a way to

simply Reset all of the bad Devices at the same time:

USBUHCIL ResetBadDevices

This will locate ALL of the bad Devices on ALL of the host

controllers, and issue Reset requests for them.  Hopefully, after the

Resets are complete, all Devices will be performing properly.

DOS USB DRIVER INTRODUCTION            Page 71 of 186

Note that this does NOT Reset devices that are considered bad by a

specific device driver (e.g., USBHUB), only those that are considered

bad by the host driver (USBUHCI{L}).  A device that is considered bad

by USBUHCI{L} is truly bad (at the USB level), while one that is

considered bad by a specific device driver may just be incompatible

with the driver.  For example, the current version of USBHUB will

reject (declare as "bad") a Hub that has more than 7 ports, even

though there may be nothing actually wrong with the Hub.

Debug Mode

����������

As discussed above in the section called "STARTING & STOPPING HOSTS"

(page 24), a UHCI Host Controller can be put into Debug Mode.  In

this mode, the Host Controller is essentially "paused", and is not

issuing frames or sending data across the bus.  This is actually the

same thing that happens when the Host is Stopped.

However, while in Debug Mode, the UHCI Host Controller can be Single-

Stepped (this cannot happen in Stopped Mode).  Single-stepping the

Host Controller causes it to generate a single Frame sequence on the

bus (reading from the Schedule, transmitting packets, generating

IRQ's as appropriate, etc.).  After the single frame has been

processed, the Host Stops and is in Debug Mode again.

To enter Debug Mode, you would do the following:

USBUHCIL Debug

To have the Host Controller process a single Frame while in Debug

Mode, you would do the following (you can do this as many times in a

row as you want):

USBUHCIL SingleStep

To exit Debug Mode, you must re-Enable the Host Controller:

USBUHCIL Enable 0

Please do not confuse this Debug option with the ProgramDebug option

discussed above (page 61).  This Debug option enables the

Debug/SingleStep mode that is built into the UHCI host hardware,

while the ProgramDebug option above is designed to help troubleshoot

installation and uninstallation of the executable program file itself

(USBUHCI.COM or USBUHCIL.COM).

DOS USB DRIVER INTRODUCTION            Page 72 of 186

Attached Devices

����������������

If you want to see a list of all of the different Logical Devices

that are attached to the bus, you would do the following:

USBUHCIL Addresses

This will list all of the Logical Devices, sorted by Address, along

with several details about the Device/Interface (the Device/Interface

"type" or "class", power requirements, manufacturer, which port of

which Hub the Physical Device is plugged into, etc.).  Most of the

details that are listed will be fairly self-explanatory, at least if

you're familiar with the USB architecture.  Some of them need further

explanation, however.

In the first column (ADRS - the Logical Device Address), the Address

may be followed by another character: "T", "r", or "R".  A Logical

Device Address that has a "T" after it is in Test Mode.  Test Mode is

used with special testing equipment to verify that everything is

operating correctly at the electrical level on the bus.  If you ever

see a Device in Test Mode, and you are not trying to test it with

special equipment, you (or one of your programs) has done something

very wrong.  A small R ("r") means the Device/Interface supports

Remote Wakeup (discussed in "DISABLING & SUSPENDING DEVICES" above,

page 21), but Remote Wakeup functionality is currently disabled.  A

capital R ("R") means that Remote Wakeup functionality is currently

enabled.  By default, USBUHCI{L} enables Remote Wakeup on all

Device/Interfaces that support it -- you will not normally see an

"r".

In the BUS POWR (Bus Power) column, there may be a small S ("s")

preceding the milliamp number.  The "s" means that the Physical

Device is self-powered (or at least claims that it is self-powered),

rather than bus-powered.

Between the columns labeled AltIn (Alternate Interface) & DESCRIPTION

(Device/Interface "Class" or "Type"), there are asterisks ("*") in

some, though not necessarily all, of the rows.  Interfaces (as

defined in the USB sense -- see the "DEVICES & INTERFACE" section

above, page 9) can have more than one "configuration", called an

"Alternate Interface".  Only one Alternate Interface (Interface

configuration) can be selected at a time for each Interface.  The

Alternate Interface that is currently selected for an Interface is

the one with the "*" next to it.  "Simple" Device/Interfaces like

mice and keyboards usually don't have more than one Alternate

Interface per Interface, but "complicated" Device/Interfaces (like

multi-function printers) usually do.

DOS USB DRIVER INTRODUCTION            Page 73 of 186

If the Owned column has a "Y", it means that there is a specific

Device/Interface Driver that "Owns" (is managing and controlling) the

Device/Interface.  A Device/Interface that is not Owned will not do

anything useful -- it will just sit there and take up space.  The

concept of Ownership is critical to the Application Programming

Interface (API), and is described in a separate document called

USBAPI.DOC.  It will not be discussed in detail here.

While a Device is in the middle of the initialization & enumeration

process, USBUHCI{L} is still trying to figure out all of the details

about a particular Logical Device (there is no detail yet to show).

In this situation, the display will just say "Enumerating".  You must

wait for the enumeration process to complete before any further

details will be available about the Logical Device.

If a Logical device was found to be "bad" during the enumeration

process (see "BAD" DEVICES above, page 48), details about the Logical

Device will not be shown.  Instead, cryptic details (codes) about

exactly where and why the enumeration process failed will be shown.

If you want to research further and possibly troubleshoot the

problem, you can certainly do that, though this is usually a time-

consuming, non-trivial process.  You will need to research the "Bad"

Device detail codes, and compare them to the appropriate sections of

the Assembly Language source code for USBUHCI{L}.  You can certainly

learn a lot by doing it, though I would not recommend that road for

most users.

In order to list the Address data, USBUHCI{L} calls the support

programs USBDEVIC (page 84 Below) and VENDORID (page 184 Below).

New Device Notifications

������������������������

New Device Notifications are one aspect of the Application

Programming Interface (API), which is described in detail in a

separate document called USBAPI.DOC.  Specific details will not be

discussed here.

A Device/Interface Driver can Register with the API to be

automatically notified when the Host Driver software on the computer

has successfully recognized and enumerated a new Logical Device.

Part of the information provided by the Device/Interface Driver

during the Registration process is a "filter" for which

Device/Interfaces it wants to be notified of.  For example, the

USBMOUSE Driver will Register with a Filter that essentially says,

"Only notify me if the New Device/Interface is a Mouse -- If it's any

other kind of Device/Interface, don't bother."

DOS USB DRIVER INTRODUCTION            Page 74 of 186

USBUHCI{L} keeps the New Device Notification details in memory (in

the TSR), and you can view the details as follows:

USBUHCIL NewDeviceStatus

The New Device Notification table is actually a feature of the API

itself, and is not directly associated with a particular instance of

USBUHCI{L} in memory.  That is, all instances of USBUHCI{L} (and any

other Host Controller Drivers that are developed in the future) must

share the same table.  There is only one copy of the table, no matter

how many instances of USBUHCI{L} you have in memory.

In order to list the New Device Notification details, USBUHCI{L}

calls the support program USBDEVIC (page 84 Below).

Host Controller Hardware Status

�������������������������������

There are many hardware level details associated with the

configuration and operation of a UHCI Host Controller.  These include

various memory & Input/Output resource addresses, Framing details,

Root Hub Physical Device connection details, Legacy Support

configuration, etc.  If you want to fully understand everything

that's involved in running a UHCI Host Controller, you can read the

UHCI Specification (available from Intel).  We are not going to

discuss any of those ugly details here.

However, if you want to view the current hardware configuration

details of a UHCI Host Controller, you would do the following:

USBUHCIL HostControllerStatus

Even without reading the UHCI Specification, some of the information

provided will probably make sense to you simply based on what you've

seen so far in this document.  As you might imagine, I've used this

option switch a LOT as I've been developing the USBUHCI{L} Driver.

You may find it useful as well, even if just as a simple learning

experiment to discover more about how USB really works.

In order to list the UHCI Host Controller hardware configuration

details, USBUHCI{L} calls the support program USBHOSTS (page 82

below).

DOS USB DRIVER INTRODUCTION            Page 75 of 186

Schedule Status

���������������

One of the main functions that any USB Host Controller Driver (such

as USBUHCI{L}) provides is a Schedule.  The Schedule is how the

software (USBUHCI{L}) tells the hardware (the UHCI Host Controller)

when and where to send each data packet.  In UHCI Host Controllers,

this is done with three different data structures: Frame List (FL),

Transfer Descriptor (TD), and Queue Head (QH).

The data contained in these data structures is rather cryptic and

uninteresting, unless you're doing some really "hard-core"

troubleshooting.  As a rule, you probably won't want to look at these

details unless you're having serious problems.  Of course, you may

also be amazed at what you learn if you start looking at these kinds

of details and try to figure out what it all really means.

Also keep in mind that when you view these kind of details, you are

simply looking at a "snapshot" of the data at one particular instant

in time.  You are not monitoring things on a "real-time" basis.  The

software and hardware are constantly running and updating the

Schedule in the background, so you will usually not see the same

display two times in a row.

Frame List (FL)

���������������

The basis of the Schedule is the Frame List (FL).  The FL is a

data structure located in memory (RAM), which contains 1024

entries (numbered 0-1023).  Each entry corresponds to a USB Frame.

USBUHCI{L} places a pointer to the data structure describing the

data that needs to get sent across the bus (TD or QH) into the

appropriate place in the FL.  The UHCI hardware reads the

information from the FL (placed there by the software), and sends

data packets across the bus accordingly.  When it is time to start

a new Frame (which happens 1000 times every second), the UHCI

hardware simply looks at the "next" FL entry, and does whatever

the Schedule tells it to do.  When the hardware gets to the end of

the FL (entry number 1023), it simply "rolls over" and starts at

the beginning again (entry number 0).

DOS USB DRIVER INTRODUCTION            Page 76 of 186

The Schedule is set up in the form of a linked list.  An FL entry

simply contains a pointer (Physical Memory Address) to the first

TD or QH that the Host hardware needs to process.  After the Host

hardware finishes processing the first TD/QH, the TD/QH may then

contain a Link Pointer to another (second) TD/QH, which the Host

hardware will process.  The second TD/QH may contain a Link

Pointer to a third TD/QH, etc.  The end of the linked list is

indicated by a special Termination flag, which essentially tells

the Host hardware, "the Link Pointer in this TD/QH is not valid --

we're at the end of the linked list -- you can stop now."

To view the contents of the Frame List (FL), you would do the

following:

USBUHCIL FrameList

This will display the contents of the Frame List (FL).  However,

rather than list all 1024 entries, it will only display the ones

that have a valid TD/QH entry in them.  If you don't have a lot of

Device/Interface Drivers installed in memory, you probably won't

see very many items when you display the FL.

Transfer Descriptor (TD)

������������������������

A Transfer Descriptor (TD) is a data structure located in memory

(RAM) that describes a single data packet that needs to get sent

across the bus.  The data structure contains enough information

that the Host Controller hardware knows exactly what to do with

the data.  This includes, among other things, where the data is

coming from and going to (a Physical Memory Address on one end,

and a Logical Device Address & End Point Number on the other end),

how many bytes of data to transfer, and a place for the hardware

to store the status (whether the data was successfully transferred

or not).

A TD data structure in RAM also contains information that the Host

Controller hardware doesn't care about, but that the Driver

(USBUHCI{L}) needs to know.  This includes the repeat rate

(Periodicity) if the packet is associated with an Isochronous or

Interrupt Transaction, a Timeout counter to keep track of a

"stalled" packet that is taking too long to respond, and what to

do when the Packet has been completed (which Device/Interface

Driver to inform).  There is actually quite a bit of information

associated with each TD.

DOS USB DRIVER INTRODUCTION            Page 77 of 186

To view the contents of all the "active" TD's (the TD's that are

actually doing something), you would to the following:

USBUHCIL TransferDescriptors

To view the contents of a particular TD, you would do one of the

following:

USBUHCI  TransferDescriptor {0-255}

USBUHCIL TransferDescriptor {0- 79}

where the number (0-255 or 0-79) is the index number of the

specific TD you want to view.  Figuring out the index number can

be tricky (remember this is really designed for "hard core"

troubleshooting), but once you figure out what it is, you can

"track" it.

Queue Head (QH)

���������������

A Queue Head (QH) is a data structure located in memory (RAM) that

describes a group of related data packets (TD's) that need to get

sent across the bus.  There is a strict limit to the number of

data bytes that can be sent in a single USB packet.  When there is

a large amount of data to be transferred, it must be split up into

a series of smaller, packet-sized "chunks".  The packets must be

processed in the correct sequence, and if one packet gets

"stalled" for some reason, the rest of the sequence is also

"stalled" until the problem gets fixed.  In addition, it may be

possible for the entire sequence of packets to take a long time to

transfer, and it is not possible for all of the packets to be sent

within a single Frame (1/1000 of a second).

A QH is specifically designed to handle all of these "special"

data requirements.  USBUHCI{L} uses QH's for Control Transactions

and Bulk Transactions.  USBUHCI{L} will also use QH's for

Interrupt Transactions, if the amount of data to be transferred

will not fit into a single packet.  USBUHCI{L} does not use QH's

for Isochronous Transactions, or for "small" Interrupt

Transactions.

To view the contents of all the QH's, you would do the following:

USBUHCIL QueueHeads

As a rule, most of the QH's will be empty (the Element Pointer

will not point at a TD), since they are generally only used when a

"large" Transaction is in progress.  Device/Interfaces that

continuously operate in the background (such as mice and

keyboards) do not generally use QH's.

DOS USB DRIVER INTRODUCTION            Page 78 of 186

Descriptors

�����������

One very fundamental concept in the USB architecture is Descriptors.

All USB Devices have Descriptors, which is how a Device "describes"

itself to the computer.  Descriptors are how the software (device

drivers) in the computer know whether the Device is a mouse or a

keyboard or a printer, how much power it is supposed to use, who

manufactured the Device, etc.  All Devices have at least 4

Descriptors, and many Devices have even more than 4.  Unfortunately,

the Descriptors are all encoded in special formats (numbers), so

looking at them in their encoded format would not be very useful.

USBUHCI{L} allows you to look at the Descriptors for any Device

attached to the bus in a "translated" format that you can understand.

USBUHCI{L} internally stores the Descriptors of the last Device that

was enumerated (it needed to download them in order to enumerate the

Device).  If you want to see the Descriptors of the last Device that

was enumerated on the bus (usually, but not always, the one with the

highest address), you simply do:

USBUHCI Descriptors

USBUHCI{L} will also download and let you view the Descriptors of any

Device attached to the Bus, even if it was not the last one

enumerated.  To do this, you simply follow the Descriptors option

switch with a number indicating the assigned Address of the Device:

USBUHCI  Descriptors {2-127}

USBUHCIL Descriptors {2- 17}

What's displayed with the Descriptors option switch are the general

Descriptors, which all USB Devices must have.  In addition, some

Devices have what are called class-specific Descriptors.  These only

apply to certain types (classes) of Devices.  For example, there are

certain items and parameters that you might use to describe a video

camera, which would mean absolutely nothing if you tried to use them

to describe a keyboard.

DOS USB DRIVER INTRODUCTION            Page 79 of 186

There is a rather large class of Devices that USB calls "Human

Interface Devices" or HID's, which end up getting used a lot in the

USB world.  Pretty much anything that you manipulate with your hands

(mice, keyboards, joysticks, most anything with buttons or wheels,

etc.) are classified as HID's.  All HID's have a class-specific

Descriptor called an HID Report Descriptor, which is how the Device

describes to the computer how many buttons, wheels, axes, etc., it

has.  Because HID's are so common, and because they can be very

difficult to write software for, USBUHCI{L} will download and let you

view the HID Report Descriptor for any HID that is attached to the

Bus.  To do this, you must provide USBUHCI{L} with both an assigned

Address number and an Interface number (I#):

USBUHCI  Report {2-127} I#

USBUHCIL Report {2- 17} I#

You can determine the interface number (I#) by using the Address

option switch (see "Attached Devices" above, page 73).  In many

cases, the Interface number will be 0 (zero), though that is

certainly not always true.

Note that in both cases (general Descriptors and HID Report

Descriptors), even though the Descriptors are "translated" from raw

numbers into a meaningful format, it does not necessarily mean you

will understand what you are seeing.  This is especially true of HID

Report Descriptors, which are difficult to understand even when you

are familiar with them.  USB is extremely complicated, and you may

not understand what all of the different terms mean or why they are

important.  You will need to read the appropriate USB specifications

for deeper analysis and understanding.

In order to display the Descriptors, USBUHCI{L} will use various

support programs, including USBSUPT1 (page 178), HIDSUPT1 (page 180),

and VENDORID (page 184).

Miscellaneous Notes

�������������������

USBUHCI{L} only works with UHCI (Universal Host Controller Interface)

Host Controllers, which are manufactured by Intel and Via

Technologies.  The other alternative (for USB version 1.x) is called

OHCI (Open Host Controller Interface).  OHCI controllers are made by

all of the other manufacturers except Intel & Via Technologies.  If

you don't know what kind of USB Host Controller(s) you have (which

you probably don't), use the USBHOSTS program (page 82 below) to tell

you.  There are plans to eventually make an OHCI Driver, but it will

not happen any time soon.

DOS USB DRIVER INTRODUCTION            Page 80 of 186

The USB specifications state that there can be no more than 5 levels

of Hubs between a Physical Device and the Host Controller.  However,

USBUHCI{L} does not actually track or enforce the number of Hub

levels.  If there are more than 5 levels of Hubs, but the Physical

Device still works correctly, USBUHCI{L} will let it continue to

work.  In practice, there are rarely more than a couple levels of

Hubs, so this scenario will probably not occur except in some sort of

special test environment.

It is commonly recommended by manufacturers that you place their

Physical Devices as close to the Root Hub as possible (with as few

"levels" of Hubs between it and the Host as you can).  If you're

having problems with a particular Physical Device, that is one aspect

of "troubleshooting".  As a side note, that indicates to me that not

all Physical Devices are truly USB compatible, nor are they tested

thoroughly.

As USBUHCI{L} is assigning Addresses to Logical Devices, it does it

in a "first available" fashion.  For example, let's say you have a

Logical Device that is currently assigned Address 5.  If you unplug

the Physical Device and replug it back in again, or if you issue a

Reset to the Physical Device, it will go through the complete

initialization and enumeration process again.  As a result, it will

be assigned a new Address.  USBUHCI{L} will only assign the same

address (5) again if addresses 2, 3, and 4 are unavailable (already

assigned to some other Logical Device).  Many times a Logical Device

will end up with the same address again after it is re-plugged or

Reset, but sometimes it won't.  It is never safe to assume that the

address will remain the same across a re-plug or a Reset.

DOS USB DRIVER INTRODUCTION            Page 81 of 186

USBHOSTS

��������

Version:     0.06

Prog Type:   Support

Source Code: ASM (A386 by Eric Isaacson)

Status:      Complete

Environment: None

Redirection: No

Purpose:     Shows Details of One or All USB Host Controllers

TSR Memory:  0 kB (not a TSR)

Showing Host Controller Status

������������������������������

The main purpose of USBHOSTS is show details about the different Host

Controllers that are installed on your computer (UHCI, OHCI, and

EHCI).  If you run USBHOSTS with no option switches at all, it will

show you a summary of ALL the Host Controllers that are installed.

This summary only includes "high-level" (PCI bus related) details of

the Host Controllers, such as the PCI Search Index (discussed above

on page 54), manufacturer, IRQ number, PCI base memory or I/O

address, etc.  Again, to see the summary, you run USBHOSTS with no

options switches, as follows:

USBHOSTS

You can also have USBHOSTS show you virtually ALL of the details for

an individual Host Controller.  This is includes all of the "high-

level" information indicated above, as well as Framing details, Root

Hub Physical Device connection details, Legacy Support configuration,

etc.  The exact details of what it shows you are VERY different for

each type of Host Controller (UHCI, OHCI, or EHCI), and we will not

try to explain any of it here.  You can download and peruse all three

specifications (they are freely available) if you're interested in

all of the details.

To show all of the details for a specific Host Controller, you must

specify to USBHOSTS exactly which Host Controller you want to see.

You do this by specifying two pieces of information: a single letter

indicating the type of Host Controller (U = UHCI, O = OHCI, E =

EHCI), followed by a number indicating the 0-based PCI Search Index.

For example:

USBHOSTS U0

will show you the details of the first UHCI Host Controller (the one

with PCI Search Index 0).

DOS USB DRIVER INTRODUCTION            Page 82 of 186

You can also have USBHOSTS show you the details of more than one Host

Controller at a time.  For example:

USBHOSTS U2 E0

will show you details of the third UHCI Host Controller, followed by

details of the first EHCI Host Controller.

You can also view all of the details for all of the Host Controllers

as follows:

USBHOSTS D

Depending on the exact type of data element that is being shown,

USBHOSTS will either show you a short word or phrase, a number, a

"Y", a ".", or a "�".  A word, phrase, or number generally pretty

self-explanatory.  Here is what the "Y", ".", and "�" indicate:

Y = Yes, On, True, Enabled

. = No, Off, False, Disabled

� = Not Applicable, Unknown

In order to write the company name of the Host Controller

manufacturer, USBHOSTS calls the support program VENDORID (page 184

below).

Disabling EHCI Host Controllers

�������������������������������

As discussed in "Disabling the Companion EHCI Host Controller" (page

59), there are times when it is necessary or desirable to unconfigure

(disable) a particular high-speed (EHCI) Host Controller.  This in

turn forces all of the Devices attached to the EHCI Controller to be

routed to the companion low -speed (UHCI or OHCI) Controller.  This

is required in certain situations to force the USB hardware to do

what you want it to do, instead of having it act the way the BIOS or

hardware manufacturer wants it to.  To unconfigure (Stop) a

particular EHCI Host Controller, you must provide USBHOSTS with the

PCI Search Index of the EHCI Host Controller you want to Stop.  E.g.:

USBHOSTS Stop 0

will Stop (unconfigure) the first high-speed (EHCI) Host Controller

(the one with PCI Search Index 0), forcing all of the Devices to

route to the companion low-speed (UHCI or OHCI) Controller(s).

DOS USB DRIVER INTRODUCTION            Page 83 of 186

A single high-speed (EHCI) controller is usually associated with more

than one low-speed (UHCI or OHCI) companion Controller.  On the Sony

laptop I currently use, for example, there is one EHCI Controller and

two companion UHCI Controllers.  On the Dell Desktop I currently use,

there are two EHCI Controllers each with three companion UHCI

Controllers (a total of six UHCI Controllers).  What you do to a

single EHCI Controller usually affects several UHCI (or OHCI)

Controllers.

It is not possible to reliably force a single EHCI port to the

companion low-speed controller using USBHOSTS.  Attempting to do so

could potentially "break" the EHCI software (in the BIOS) and wreak

havoc on the entire USB bus.  Rather, what USBHOSTS does is

unconfigure the entire EHCI host, forcing ALL ports to the companion

Controller(s).  In addition, once an EHCI Controller is unconfigured

(Stopped), it is potentially unsafe to simply try and "turn it on"

(configure it) again.  Therefore, USBHOSTS does NOT provide a way to

re-configure an EHCI Host Controller that has be unconfigured

(Stopped).  Once it has been Stopped with USBHOSTS, the only way to

safely re-start an EHCI Host Controller is to reboot the computer.

The Stop (unconfigure) option for USBHOSTS should no longer be needed

once an EHCI Host Controller Driver is completed, though that is not

expected to happen in the near future.

USBDEVIC

��������

Version:     0.05

Prog Type:   Support

Source Code: C++ (Borland Turbo C++ for DOS 3.0)

Status:      Complete, a few minor tweaks required

Environment: None

Redirection: No

Purpose:     Show details of attached Physical Devices

TSR Memory:  0 kB (not a TSR)

USBDEVIC shows details about all of the different Device/Interfaces that

are attached to the USB busses.  The details include the assigned

Address, the Device/Interface "type" or "class", power requirements,

manufacturer, which port of which Hub the Physical Device is plugged

into, etc.

If you run USBDEVIC with no option switches, it will show you the

details of ALL Device/Interfaces that are attached to ALL busses:

USBDEVIC

DOS USB DRIVER INTRODUCTION            Page 84 of 186

You can also have USBDEVIC show you all Device/Interfaces that are

attached to a single bus.  To do this, you must provide USBDEVIC the

Host Index number (0-15) of the Host Controller you are wanting to view.

For example, to view the details of all Device/'Interfaces attached to

Host Index 2:

USBDEVIC 2

Note that what must be provided is the Host Index number, and NOT the

PCI Search Index number.  The Host Index number is associated with the

Host Controller Driver (USBUHCI{L}) and the Application Programming

Interface (API), not with the PCI bus hardware.

You can also have USBDEVIC show you the Device/Interface details for

more than one bus at a time, but not necessarily all of them.  To do

this, you just provide more than one Host Index number.  The show the

Device/Interface details for Host Index 2 followed by Host Index 1, for

example, you would do:

USBDEVIC 2 1

Most of the Device/Interface details shown by USBDEVIC are fairly self-

explanatory, at least if you're familiar with the USB architecture.

Some of them need further explanation, however.  The explanations are

provided above under "Attached Devices" (page 73), and will not be

repeated here.

In addition to showing you details about the Device/Interfaces attached

to the bus, USBDEVICE will also show you the contents of the New Device

Notification table.  The concept of New Device Notifications is a

complicated aspect of the Application Programming Interface (API), which

is described in detail in a separate document called USBAPI.DOC.  There

is also a brief description given above in the "New Device

Notifications" section (page 74).  Those details will not be repeated

here.

To display the contents of the New Device Notification table (stored in

memory as along with the first instance of USBUHCI{L}), you would do the

following:

USBDEVIC NewDeviceTable

In order to write the company name of the Device/Interface manufacturer,

USBDEVIC calls the support program VENDORID (page 184 below).  In order

to write a description of the "type" of Device/Interface, USBDEVIC calls

the support program USBSUPT1 (page 178 below).

DOS USB DRIVER INTRODUCTION            Page 85 of 186

IRQ

���

Version:     1.00

Prog Type:   Support

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete

Environment: None

Redirection: No

Purpose:     IRQ (Hardware Interrupt) troubleshooter

TSR Memory:  0 kB (not a TSR)

There are a total of 16 IRQ's (Interrupt ReQuests) on your computer.  An

IRQ is also called a Hardware Interrupt, because it is "caused" by a

hardware device of some sort.  In some contexts, the word interrupt has

a negative connotation, and is something you usually want to avoid.  In

a computer, though, an Interrupt is usually a good thing.

Computers have 16 IRQ's, numbered 0-15.  Some IRQ's are "hard-wired" to

certain hardware Devices, such as the clocks inside the computer, the

keyboard, and the math coprocessor.  Others are there for "general use"

(not hard-wired for a specific purpose), and can be used by most any

hardware device, including USB Host Controllers.  For an IRQ to be

useful, it must be "wired" to a hardware device of some sort, and it

must also be enabled.

The view the status of the 16 IRQ's on your computer, you run IRQ with

no option switches:

IRQ

This will show you which of the 16 IRQ's are enabled, which are hard-

wired (and consequently not available for general-use), and what each of

the general-use ones is commonly (but not always) used for.  For

instance, IRQ 3 is usually used for serial ports COM2 & COM4.  However,

IRQ 3 is not REQUIRED to be used for COM2 & COM4 -- it is actually a

general-use IRQ and can be associated with any type of hardware device.

Many newer computers only have one serial port (COM1) if they have any

at all, and modern laptops usually don't have any built-in serial ports

at all.  An IRQ that is enabled was put that way by the BIOS as the

computer was booting up, and does have some kind of hardware device

"wired" to it.  An IRQ that is disabled by the BIOS is probably not

wired to any kind of hardware device (at least not one that is "turned

on"), and is available to be used by a hardware device like a USB Host

Controller.

DOS USB DRIVER INTRODUCTION            Page 86 of 186

In addition to just showing you which IRQ's are currently enabled, the

IRQ program also lets you turn the IRQ's on and off (enable and disable

them).  This is actually a useful troubleshooting tool in certain

situations, such as the one I describe above in the "IRQ & I/O

Resources" section above (page 56).

To enable a particular IRQ, simply give the IRQ program an IRQ number

(0-15) as an option switch.  For example, to enable IRQ 4, you would do

the following:

IRQ 4

To disable a particular IRQ, you must give the IRQ program an IRQ number

followed by any character (any character at all).  For example, to

disable IRQ 4, you could do either of the following:

IRQ 4 Disable

IRQ 4 *

While the IRQ program will let you enable and disable any of the 16

available IRQ's (numbered 0-15), you should generally avoid disabling

IRQ 0 (system timer), 1 (keyboard), and 2 (PIC controller #2).

Disabling any of these will usually screw up your computer beyond repair

(you'll need to reboot using the power switch to recover).  Disabling

(or enabling) any of the other IRQ's will USUALLY not screw anything up

too badly, but exactly what happens will depend on what hardware (if

any) is wired to the IRQ.

As described above in the "IRQ & I/O Resources" section (page 56), I was

having a problem with the USB Host Controller on my laptop.  Basically,

the software (as configured by the BIOS) was lying to me about which IRQ

the USB Host Controller was using.  The software said it was using IRQ 5

when it was actually using IRQ 7.  The way I found the real IRQ was to

attach a Legacy Support Device (in my case, a USB Mouse) to the Root

Hub, which would work as soon as the computer turned on.  I then

disabled the IRQ's one at a time (using the IRQ program), until I found

out when the USB Mouse (the USB Host Controller) stopped working.  After

I figured out what the problem was, I "fixed" it in my AUTOEXEC.BAT file

so I don't need to remember how to fix it each time.

DOS USB DRIVER INTRODUCTION            Page 87 of 186

USBHUB

������

Version:     0.06

Prog Type:   Device/Interface Driver

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete, a few minor tweaks required

Environment: USBHUB

Redirection: Yes

Purpose:     Driver for up to 16 USB generic version 1.x Hubs

TSR Memory:  5.7 kB

USB Hubs are the most basic of Device/Interfaces that you can attach to

your computer.  In fact, Hubs are so basic to USB busses that they are

completely defined in the base USB specification -- there isn't a

separate document needed to specify them the way there is for all other

Device/Interfaces.

A USB Hub is a special Device/Interface that effectively "expands" a USB

bus by creating additional ports.  If the Root Hubs (integrated with the

Host Controllers) do not have enough ports to support all of the

Physical Devices you want to plug in, you will need a USB Hub to create

new ports.  USBHUB is the software driver it takes to enable and manage

any USB Hubs you have.  If you do not have any USB Hubs installed on

your computer, or if you do not want to use the Physical Devices you

have attached to those Hubs (the only Physical Devices you want to use

are attached to Root Hubs), you do not need to install USBHUB.

To install USBHUB into memory, you can simply run it with no option

switches.  Note, however, that at least one instance of USBUHCI{L} must

be installed in memory before USBHUB can be installed.  For example:

USBHUB

USBHUB will enable and manage up to 16 USB Hubs, with up to 7 ports on

each Hub.  USBHUB will not manage a Hub with more than 7 ports (USBHUB

will classify a Hub with more than 7 ports as a "bad" Device).

To have USBHUB display the General Descriptors (Device, Configuration,

Interface, End Point) of the last Hub that was attached, you would do

the following:

USBHUB Descriptors

In addition to displaying the contents of the General Descriptors, this

option switch will also display the contents of the class-specific Hub

Descriptor.  The Hub Descriptor includes specific details about the Hub,

such as the number of ports and power control characteristics.

DOS USB DRIVER INTRODUCTION            Page 88 of 186

To have USBHUB show you the current status of the Hubs that it is

managing, you would to the following:

USBHUB Status

This will display all of the Hubs that are being controlled, as well as

details about each of the ports on each Hub (power usage, which ports

have Physical Devices connected, etc.).  The details can be a little bit

cryptic, so you may want to read the appropriate parts of the USB

specifications if you need further explanation of what the different

data items are actually telling you.

While a New Device is being detected, initialized, and enumerated, there

is a large amount of interaction that takes place back and forth between

the Host Controller Driver (USBUHCI{L}) and USBHUB.  If a Logical Device

fails to enumerate properly for some reason, and it is attached to a Hub

being managed by USBHUB, you can have USBHUB tell you where and why the

failure occurred.  You do this as follows:

USBHUB ExternalStage

As with most troubleshooting details in these programs, the results are

rather cryptic.  You will need to correlate the error information

provided with the Assembly Language source code for USBHUB to figure out

exactly what happened.  The source code for USBHUB is available if you

want it, but if you are not familiar with Assembly Language programming,

it may confuse you more than it helps you.

Just as a point of clarification, it is not possible to electrically

"split" or "tap" the USB bus wires to go to two (or more) different

Physical Devices.  Everything on a USB bus is carefully managed through

the Host Controller Driver (software), and any "splits" in the bus to go

to multiple Devices MUST go through a Hub.  Each Physical Device must

plug into a port that is on either a Root Hub or a managed USB Hub.

In order to write the contents of the Descriptors, USBHUB calls the

support program USBSUPT1 (page 178 Below).

DOS USB DRIVER INTRODUCTION            Page 89 of 186

USBKEYB

�������

Version:     0.09

Prog Type:   Device/Interface Driver

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete, a few minor tweaks required

Environment: USBKEYB

Redirection: Yes

Purpose:     Driver for up to 4 USB Keyboards

TSR Memory:  9.2 kB

USBKEYB is a Device/Interface Driver program for up to four USB

Keyboards or Keypads ("miniature" Keyboards).  USBKEYB is very unique --

you cannot find a Keyboard Driver for any other type of DOS USB

architecture at all.  In the other DOS USB architectures, if you want to

use a USB Keyboard, you must use the Legacy Support in the BIOS, which

has certain limitations that USBKEYB does not.

USBKEYB allows you to have up to four USB Keyboards or Keypads installed

on the computer at the same time, and any or all of them can be used at

any time.  You can also have a Legacy (PS2) keyboard installed and use

it was well.  It might be useful, for example, to have two separate

keyboards in a game where two players are "shooting" at each other, and

normally requires both of you to share the same keyboard.  Or, there are

many accountants and bookkeepers that like a separate Keypad (with just

numbers and basic math functions) to do their data entry.  Or, it may be

easier to train someone on the computer if you each have your own

keyboard, so you're not constantly needing to slide people or chairs or

keyboards back and forth as you interact.  In short, there are certain

times when having more than one Keyboard can actually be quite useful.

Before USBKEYB can be installed into memory, you must first have at

least one USB Host Driver (USBUHCI{L}) installed in memory.  To install

USBKEYB into memory, you can simply run it with no option switches:

USBKEYB

Typing Methods

��������������

As you type things on a USB Keyboard, it sends special codes, called

USB Key Codes, to the computer.  When you type things on a regular or

Legacy (PS2) keyboard, it sends special codes called Scan Codes to

the computer.  USB Key Codes and Scan Codes are completely different

from each other.  Of course, what DOS programs are expecting to see

are Scan Codes, not USB key codes.

DOS USB DRIVER INTRODUCTION            Page 90 of 186

Also, it is important for you to know that Scan Codes are not the

same thing as ASCII characters, which are what you normally see on

the screen as you are typing things.  There is another "translation"

that goes on in the computer to convert Scan Codes (and Scan Code

combinations) into ASCII characters.  Details of this level of

translation will not be discussed in this document, but will

definitely add another level of confusion and complexity into the mix

as you are trying to figure things out.  You can read the

documentation for my SCANCODE program if you want more information

about this.

What USBKEYB does is take the USB Key Codes as they are coming in

from the USB keyboard, "translate" them into Scan Codes, and then

simulates "typing" them into the computer so that they look like they

came from a Legacy (PS2) keyboard.  If you are familiar with some of

my other programs (SCANCODE, JOYKEYS, MOUSKEYS), you will probably

already know what this involves.

Unfortunately, simulating the typing of keystrokes into the computer

is a very difficult task, and does not work correctly in all

circumstances.  Therefore, there are certain times and certain

programs where you will not be able to use USBKEYB, and will need to

resort to using a real Legacy (PS2) keyboard OR USB Legacy Support in

the BIOS.  Hopefully, these situations will be rare (or even non-

existent) for you.  You should always try to have a Legacy (PS2)

keyboard laying around, though, just in case.

USBKEYB has three different Methods of simulating keystrokes into the

computer, simply called Methods 1, 2, & 3.  Method 1 uses a special

function built in the keyboard BIOS (hardware) of many modern

computers.  In other words, for it to work properly, the hardware on

your computer (motherboard) must be compatible with Method 1.  Of the

three Methods, Method 1 is usually BY FAR the best and most reliable

Method to use, and can simulate keystrokes in situations that the

other two Methods can't.

Method 2 uses a combination of hardware and software to simulate

keystrokes.  Similar to Method 1, it requires that the keyboard BIOS

hardware on the motherboard be compatible with Method 2.  Of the

three Methods, it is the least likely to work on your computer (it

has never worked reliably on any of my computers).  Method 2 also has

the disadvantage that if your hardware is not compatible, attempting

to simulate keystrokes with Method 2 will sometimes, but not always,

lock up (crash) the computer.

Method 3 strictly uses software to simulate keystrokes.  It will work

on all computers, and does not require any special or compatible

hardware on the motherboard.  The main disadvantage to Method 3 is

that it may not work in certain environments, such as if your

computer switches into DPMI (DOS Protected Mode Interface).

DOS USB DRIVER INTRODUCTION            Page 91 of 186

As USBKEYB is installing itself into memory, it performs a relatively

simple test to determine which of the three Methods it should use to

simulate keystrokes.  The first test it performs is to see if you

have provided it an option switch to tell it which Method you want it

to use.  To have USBKEYB simulate keystrokes using Method 1, for

example, you would do:

USBKEYB Method 1

If you do not provide an option switch when USBKEYB is first

installing itself into memory, it (internally) attempts to simulate a

single keystroke using Method 1.  If it works, USBKEYB uses Method 1.

If Method 1 doesn't work, USBKEYB attempts to simulate a keystroke

using Method 2.  If it works, USBKEYB uses Method 2.  If Method 2

doesn't work, USBKEYB simply assumes Method 3 will work (it does not

actually test it).

There are four problems that I have seen occur during the testing.

The first is that your computer crashes while Method 2 is being

tested (the computer locks up as soon as you try to install USBKEYB

into memory).  This means that your computer appears to be

incompatible with Method 1 (if it appeared to be compatible with

Method 1, USBKEYB would not have tried to test Method 2).  The

solution is to force USBKEYB to use Method 3, as follows:

USBKEYB Method 3

Note that you must do this as USBKEYB is first being installed into

memory, so it will not try to test Method 2.

The second problem is that after USBKEYB is installed into memory,

the USB Keyboard still won't type anything.  This means that the BIOS

keyboard software on your computer is incompatible with Method 3.

The solution for that is to replace the BIOS software with some new

software that is compatible with Method 3.  The easiest way to do

that is to installed the KEYB program that comes with later versions

of Microsoft DOS.  Assuming you are using a United States QWERTY

keyboard, you would do the following for "regular" DOS:

KEYB US,,C:\DOS\KEYBOARD.SYS

or the following for DOS 7.x (DOS95 / DOS98):

KEYB US,,C:\WINDOWS\COMMAND\KEYBOARD.SYS

You should make the appropriate adjustments to the installation based

on your keyboard layout, DOS version, and directory structure.  There

may be other programs besides Microsoft's KEYB program that will do

the same thing, but the point is that you must COMPLETELY replace the

BIOS keyboard software on the computer.

DOS USB DRIVER INTRODUCTION            Page 92 of 186

The third problem that can occur is for your computer hardware to be

compatible with Method 1, but your BIOS keyboard software to not be.

The "patch" to fix this problem is the same as discussed directly

above -- to replace the BIOS keyboard software.  If Method 1 doesn't

appear to work on your computer, try installing the Microsoft KEYB

program.  If this fixes your problem, you're all set.

The fourth problem is that Method 1 or Method 2 can appear to work OK

during simple testing, but actually turn out to be unreliable.

Usually, what will happen with an unreliable Method is that sometimes

the wrong keystroke will be simulated, or some keystrokes seem to

simply be "ignored".  Installing the KEYB program will sometimes fix

these reliability issues as well, but many times you will end up

needing to force USBKEYB to use Method 3.

As you are first figuring out how USBKEYB works, you will probably

need to experiment with different permutations of Methods and

installation of the KEYB program to find out what works best on your

computer.  Once you figure it out, edit your AUTOEXEC.BAT and

environment variables and batch files to automate things

appropriately so that you don't need to worry about the details every

time you use your computer.

Keyboard Mapping

����������������

As discussed in the previous section, one of the functions that

USBKEYB must provide is to "translate" USB Key Codes into Scan Codes

before it can simulate a keystroke.  USB Key Codes and Scan Codes are

COMPLETELY different from each other, and consequently USBKEYB

maintains an internal table that contains translation information for

every single key on the keyboard (more than 100 of them).

To add further complication, USB actually has two different

"categories" of Key Codes.  There are "regular" Key Codes (letters &

numbers, function keys, arrow keys, etc.) and there are "system" Key

Codes (Sleep, Power, Wakeup, etc.).  In the USB architecture, regular

keys and system keys are handled completely differently from each

other, even when they come from the same keyboard.

The default translation table that is already built into USBKEYB will

perform translations as appropriate for a standard United States

QWERTY keyboard.  This includes many of the "multimedia" keys that

are common on newer keyboards (speaker volume controls, CD player

start/stop/pause, etc.), as well as the sleep/power/wakeup system

keys that many newer keyboards also have.

DOS USB DRIVER INTRODUCTION            Page 93 of 186

However, if you are not using a US QWERTY keyboard, or if you have

"unusual" multimedia keys, or just for whatever reason want to change

the way USBKEYB translates things, there are several option switches

related to doing that.  Unfortunately, using the option switches

involves using lots and lots of numbers (as opposed to using

descriptive words), so it can be very confusing.  Once you figure out

the option switches to use to do what you want, you should set up

some batch files or configuration files to automate things and keep

yourself "un-confused".

Translating USB Key Codes into Scan Codes

�����������������������������������������

To tell USBKEYB to translate a particular USB Key Code into a

particular Scan Code, you do the following:

USBKEYB Translate U#, S#

where U# is the USB Key Code and S# is the Scan Code.  The USB Key

Code is a single number between 4 and 255, and corresponds to

Usage Page 7 (Keyboard/Keypad Codes) in the USB Human Interface

Device (HID) Specification.  You can download the USB Human

Interface Specification if you want, but you don't actually need

it to use USBKEYB.  To display a complete list of the defined USB

Key Codes on the screen to use as a reference, just do the

following:

USBKEYB USBKeyCodes

The S# (Scan Code) can be a single number, two numbers, or three

numbers, separated by dashes.  The reason it can be multiple

numbers is very complicated and has lots history behind it, which

we won't discuss here.  Following are the different ranges of

numbers you can use for S#:

1 thru      95

98 thru     127

224- 1 thru 224- 95

224-98 thru 224-127

225-29-69

To display a complete list of the defined Scan Codes on the screen

to use as a reference, just do the following:

USBKEYB ScanCodes

DOS USB DRIVER INTRODUCTION            Page 94 of 186

Now, let's do some examples.  When you press the <A> key on a USB

Keyboard, it generates USB Key Code 4.  By default, USBKEYB

translates this to Scan Code 30, which is the Scan Code that is

generated when you press the <A> key on a regular (PS2) keyboard.

Let's say that, for some reason, when you press the <A> key on the

keyboard, you actually want the keyboard to type a <Q> (Scan Code

16).  To have USBKEYB "translate" an <A> into a <Q>, you would do

the following:

USBKEYB Translate 4, 16      (4 = USB <A>, 16 = PS2 <Q>)

To have USBKEYB translate the <A> key on the USB Keyboard into the

<Enter> key located on the number pad on the right side of the

keyboard (as opposed to the "regular" <Enter> key in the

alphanumeric portion of the keyboard), you would do the following:

USBKEYB Translate 4, 224-28

If you are only temporarily wanting to perform a translation for

some special purpose, but then later want to restore the

translation back to the default US QWERTY translation, you simply

provide a Scan Code of 0.  For example, assuming you did one of

the above translations to the <A> key on the USB keyboard, and

wanted to restore it back to the default translation, you would do

the following:

USBKEYB Translate 4, 0

In the first example above, translating <A> into <Q> probably

wouldn't be all that useful (you would have two keys that typed

<Q> and none that typed <A>).  However, actually swapping the <A>

and <Q> keys might (the <A> becomes <Q> and the <Q> becomes <A>):

USBKEYB Translate 4, 16 Translate 20, 30   (A->Q, Q->A)

DOS USB DRIVER INTRODUCTION            Page 95 of 186

Translating USB System Codes into Scan Codes

��������������������������������������������

To tell USBKEYB how to translate a USB System Key Code (Sleep,

Power, Wakeup, etc.), as opposed to a "regular" USB Key Code, you

do the following:

USBKEYB TranslateSystemCode Y#, S#

where Y# is the USB System Key Code and S# is the Scan Code.  The

USB System Key Code is a single number between 129 and 255, and

corresponds to values in the Generic Desktop Usage page of the USB

Human Interface Device (HID) Specification.  To display a complete

list of the defined USB System Key Codes on the screen to use as a

reference, just do the following:

USBKEYB USBSystemKeyCodes

The S# (Scan Code) can be a single number, two numbers, or three

numbers, separated by dashes.  Following are the different ranges

of numbers you can use for S#:

1 thru      95

98 thru     127

224- 1 thru 224- 95

224-98 thru 224-127

225-29-69

To display a complete list of the defined Scan Codes on the screen

to use as a reference, just do the following:

USBKEYB ScanCodes

For example, if wanted to the <Sleep> key on the USB keyboard (USB

System Key Code 130) to do the same thing as the <Pause> key, you

would do the following:

USBKEYB TranslateSystemCode 130, 225-29-69   (Sleep->Pause)

DOS USB DRIVER INTRODUCTION            Page 96 of 186

Setting Up a Keyboard Map

�������������������������

If you want to or need to change the keyboard mappings for a USB

keyboard, you will probably become confused very quickly.  You

must provide USBKEYB a bunch of numbers that have no real

significance as you are looking at them, and even a few minutes

after you do it you probably won't understand what it means any

more.  I recommend that any time you do keyboard mappings with

USBKEYB, you do it using a "configuration file" that you load as a

redirected input file (as discussed above in the "ENVIRONMENT

VARIABLES & REDIRECTED INPUT" section on page 51).  This will

allow you to add comments and notes to yourself about what you did

so that make sense of it all again at some point in the future.

Figuring out all of the codes it takes to set up the keyboard

mapping in the first place can be quite challenging and confusing.

USBKEYB provides some tools to help you figure it all out, and

once you do figure it out, you should automate the process as much

as possible with batch and configuration files.

We have already discussed three of the tools that USBKEYB provides

to help you figure out how to set up the keyboard mappings.  Here

they are again:

USBKEYB KeyCodes

displays all of the defined USB Key Codes, along with their

description (the descriptions are only valid for US QWERTY

keyboards, however).

USBKEYB SystemKeyCodes

displays all of the defined USB System Key Codes, along with their

descriptions.  The descriptions should be valid for all USB

keyboards.

USBKEYB ScanCodes

displays all of the standard Scan Codes associated with a

"regular" (PS2) keyboard along with their descriptions.  The

descriptions are only valid for US QWERTY keyboards, however.

In addition to simply displaying lists of the Codes along with

their standard (US QWERTY) definitions, USBKEYB also has an option

switch that lets you view the actual "raw" USB Key Codes that are

being sent by the USB Keyboard.  To view the raw keyboard data,

you would do the following:

USBKEYB RawData

DOS USB DRIVER INTRODUCTION            Page 97 of 186

The display will show a total of 16 columns of keyboard data,

labeled "Shift Codes", "N/A", and "Key 1" through "Key 14".  The

keyboard has several "Shift" keys (sometimes called "modifier

keys"), which are bit-mapped into the "Shift Codes" column as

follows:

1 = <Left Control>    16 = <Right Control>

2 = <Left Shift>      32 = <Right Shift>

4 = <Left Alt>        64 = <Right Alt>

8 = <Left Windows>   128 = <Right Windows>

If the "Shift Codes" column reads  1", for example, it means that

<Left Control> is pressed.  If the  Shift Codes  column reads  5",

it means that <Left Control> and <Left Alt> are both pressed at

the same time.

The "N/A" column is not used, and should always read "0".  "Key 1"

through "Key 14" are the  regular  (non-shift) USB Key Codes.  As

a key is pressed, its USB Key Code will appear in one of the

fourteen columns, and remain there until you release the key.

According to the USB keyboard specification, USB keyboards are

supposed to allow up to 6 non-shift keys to be pressed

simultaneously (the  Key 1" through  Key 6" columns can all have

legitimate keypress entries in them at the same time).

Unfortunately, not all keyboards are created equal, and the actual

number of keys that can be pressed at the same time will vary.

I ve personally tested keyboards that only support four

simultaneous key presses (instead of 6 like they re supposed to),

and have seen documentation for keyboards that claim to support up

to 14 (though I ve never personally tested any of those).

In addition to the option switches in USBKEYB, there is also

included a separate test program called SCANTEST (discussed below

on page 105).  This is actually an updated version of the SCANTEST

program I have distributed before with some of my other keyboard-

related programs.  SCANTEST will display the Scan Codes, along

with their US QWERTY descriptions, as you press the keys on the

keyboard.  If you are using a "real" (PS2) keyboard, SCANTEST

displays the Scan Codes that are coming directly from the

keyboard.  If you are using a USB Keyboard, SCANTEST displays the

Scan Codes after they have been "translated" by USBKEYB (or by the

BIOS, if you are using the keyboard in Legacy Support mode).

Hopefully, the various tools provided here will allow you to set

up your USB Keyboard to work exactly as you want it to.

DOS USB DRIVER INTRODUCTION            Page 98 of 186

Miscellaneous Notes about Keyboard Mapping

������������������������������������������

The RawData option switch discussed in the previous section allows

you to view the "raw" USB Key Codes as they are coming from your

USB Keyboards.  However, there is no option switch that allows you

to view the raw USB System Key Codes as they are coming from the

USB Keyboard.  There are very few USB System Key Codes, and they

are sent using a completely different mechanism and protocol than

the regular USB Key Codes are.  USB System Keys should be much

easier to figure out and process, and therefore no special option

switch was provided.

To view the current "custom" keyboard mapping status (which USB

Key Codes and USB System Key Codes are mapped to which Scan

Codes), you would do the following:

USBKEYB Status

This shows you several details about how USBKEYB is currently

configured, including any special keyboard mapping sequences you

have told USBKEYB to use.  Note that this does not display the

"default" keyboard mappings -- it only displays the ones you have

told USBKEYB to use with the Translate or TranslateSystemCode

option switches.

USBKEYB allows you to enter special keyboard mappings for up to

100 USB Key Codes, and up to 16 USB System Key Codes.  This is

enough to remap virtually the entire keyboard.

If you have multiple USB Keyboards attached to the computer, all

of them must use the exact same keyboard mapping.  There is a

single mapping table that is used for all Keyboards.  If you are

in the unusual situation of having more than USB Keyboard and they

have different layouts, you will need to load a new configuration

file if you switch keyboards.

The USB documentation is very unclear (at least to me) on how

keyboard layouts other than US QWERTY are supposed to work.  I've

never used (or even seen) a USB keyboard other than US QWERTY, so

I'm not sure how to even begin to set up a translation table for

one of them.  If you have a USB Keyboard layout for some kind of

foreign keyboard, and get the translation table figured out, you

can send a copy of the configuration file to me and I will make it

available for distribution from my web site.  If you do this,

please include LOTS of comments (including your name and contact

information) in the configuration file to make it as useful to

other people as possible.

DOS USB DRIVER INTRODUCTION            Page 99 of 186

Processing Special Keys

�����������������������

There are certain keys and keystroke combinations on a keyboard that

are processed differently in the BIOS than most of the other keys

are.  Most of the time, the way that they are processed is perfectly

acceptable, and there is no need to change it.  However, USBKEYB can

replace the default (BIOS) processing for some particular keystrokes

to allow them to work "better" in certain situations.

The first keys that USBKEYB can process are the "Lock Keys" (<Caps

Lock>, <Num Lock>, and <Scroll Lock>).  Through experimentation, I

have discovered that sometimes when USBKEYB is installed in memory,

and you press one of the "Lock Keys", the "real" (PS2) Keyboard

becomes disabled and stops working while the USB Keyboards continue

to work just as they should.  I have no idea why this happens, or why

it is only a problem on certain computers.  By default, USBKEYB will

replace the default (BIOS) processing for the "Lock Keys" so that

this problem does not occur.  However, USBKEYB lets you turn this

feature off if you want to for some reason, to allow the BIOS to

process the "Lock Keys" itself.  To disable the "Lock Key" processing

in USBKEYB, you would do the following:

USBKEYB ProcessLockKeys: No

To turn the USBKEYB processing back on again, you would do the

following:

USBKEYB ProcessLockKeys: Yes

There is normally not a good reason to turn this feature off, unless

you have a program that performs some special processing of the "Lock

Keys", or if you just want to experiment to see if your particular

computer has the problem or not.

The second keystroke combination that USBKEYB will process for you is

<Control>-<Alt>-<Delete>, which normally reboots the computer.

<Control>-<Alt>-<Delete> does not work correctly on some computers,

and will sometimes lock up the computer instead of rebooting it.

USBKEYB's handling of the reboot combination is usually "better" than

the default (BIOS) handling, but is still not perfect (it can

sometimes not work properly, either).  By default, USBKEYB will

replace the default (BIOS) processing for the <Control>-<Alt>-

<Delete> combination.  To disable the <Control>-<Alt>-<Delete>

processing in USBKEYB, you would do the following:

USBKEYB ProcessReboot: No

DOS USB DRIVER INTRODUCTION            Page 100 of 186

To turn the USBKEYB processing back on again, you would do the

following:

USBKEYB ProcessReboot: Yes

There is normally not a good reason to turn this feature off, unless

you have a program that performs some special processing of the

<Control>-<Alt>-<Delete> keystroke combination (such as certain

network control programs).

The third keystroke that USBKEYB will process for you is the <Pause>

key.  Normally, when you press the <Pause> key, the computer will

temporarily stop (pause) whatever it's doing until you press another

key on the keyboard.  When you press the second key, the computer

will un-pause and start running again.  With the default (BIOS)

processing in place, the second (un-pause) key can be any key except

another <Pause> key.  This is actually very inconvenient, since the

way you usually use the <Pause> key is to temporarily stop and start

a continuous dump of data that keeps scrolling off the top of the

screen before you can read any of it.  With the default processing,

you have to keep moving your hand back and forth between the <Pause>

key and a key located somewhere else on the keyboard.

When USBKEYB is processing the <Pause> key, the <Pause> key can be

used as the second (un-pause) key.  That way, you can keep your hand

in the same place and just keep pressing the same key to stop and

start the screen scrolling.  To disable the <Pause> key processing in

USBKEYB, you would do the following:

USBKEYB ProcessPause: No

To turn the USBKEYB processing back on again, you would do the

following:

USBKEYB ProcessPause: Yes

Changing the Typeamatic Rate

����������������������������

USBKEYB does not provide any option switches to control the

typeamatic delay and repeat rates of the USB Keyboard (how fast the

keyboard types a repeating keystroke if you continue to hold the same

key down for a long time).  Rather, USBKEYB responds to the standard

BIOS requests that are designed to control the keyboard.  The most

common way of controlling the typeamatic rate of the keyboard is by

using the MODE utility that comes with DOS.  For example, to set the

typeamatic rate to the fastest it can be (repeat rate = 30 cps, delay

= � second), you would do the following:

MODE CON: RATE=32 DELAY=1

DOS USB DRIVER INTRODUCTION            Page 101 of 186

There is one issue you need to be aware of concerning the typeamatic

rate, however.  The keyboard BIOS on all computers supports the "Set

Typeamatic Rate" function, which is what the MODE utility uses.  All

computers are also SUPPOSED to support the "Get Typeamatic Rate"

function, but unfortunately not all of them do.  As USBKEYB is

installing itself into memory, it asks the keyboard BIOS what the

typeamatic rate setting of the PS2 keyboard is by issuing a "Get

Typeamatic Rate" request.  If the keyboard BIOS supports the request,

USBKEYB will set its typeamatic rate to be the same as the PS2

keyboard.

However, if the keyboard BIOS does not support the "Get Typeamatic

Rate" function, USBKEYB has no idea how the PS2 keyboard typeamatic

rate is configured.  In that case, USBKEYB simply uses the same

default settings as a PS2 keyboard normally uses (repeat rate = 10

cps, delay = � second).  Of course, most people change the keyboard

typeamatic settings in the CMOS/BIOS startup settings of the

computer, so computers rarely boot up with the default typeamatic

settings still intact.  So, if your computer does not support the

"Get Typeamatic Rate" function (which it probably doesn't), your USB

keyboards and PS2 keyboard will not be typing at the same speed.

The remedy for this situation is relatively simple, though is a bit

of a hassle to set up.  What you need to do is run the MODE utility

to change the typeamatic rate AFTER you install USBKEYB into memory.

If you do this, the PS2 keyboard and all of the USB Keyboards will be

configured the same way.  For instance, you could do the following

(presumably, you would do this in a single batch file which would

automate the entire process):

USBKEYB

MODE CON RATE=32 DELAY=1

Incidentally, while USBKEYB is installed in memory, it fixes the "Get

Typeamatic Rate" function so that it works properly in case other

programs need to use it.

Enabling & Disabling Keyboards

������������������������������

By default, all USB keyboards are enabled (turned on) by USBKEYB.

However, USBKEYB will let you enable (turn on) and disable (turn off)

each of the individual USB keyboards.  You may want to disable a

particular keyboard, for example, while you are not using it but it

is in a location where it accidentally gets bumped all of the time.

To Disable a particular keyboard, you must provide a USB Keyboard

Index number (between 0 and 3).  For example, to Disable the first

USB Keyboard (Index 0), you would do the following:

USBKEYB Disable 0

DOS USB DRIVER INTRODUCTION            Page 102 of 186

To re-enable the second USB Keyboard (Index 1), you would to the

following:

USBKEYB Enable 1

You can also Disable or Enable all four USB Keyboards at the same

time, by specifying Keyboard Index 255 (instead of 0-3).  For

example, to Disable all four USB Keyboards, you would do the

following:

USBKEYB Disable 255

It is extremely important that you have some sort of keyboard working

on your computer, or else you won't be able to do much of anything

useful.  Therefore, you should not even attempt to Disable your USB

Keyboard unless you have "real" Keyboard (PS2 or USB in Legacy Mode)

in place that you can use instead.  Therefore, if USBKEYB doesn't

think that you have a "backup" keyboard installed, it will not allow

you to Disable any of the USB Keyboards.

By default, USBKEYB assumes that you DO have a "real" keyboard and

that it is OK to Disable the USB Keyboards.  However, there is no way

that USBKEYB can tell for sure if you have a "real" keyboard or not.

Therefore, if you do not have a "real" keyboard, and you want to make

sure that you cannot accidentally turn off your only (USB) Keyboard,

you would do the following:

USBKEYB RealKeyboard:No

To reverse this setting and tell USBKEYB to start assuming that you

have a "real" Keyboard again, you would do the following:

USBKEYB RealKeyboard: Yes

Descriptors

�����������

To display the General Descriptors (Device, Configuration, Interface,

etc.) of the last keyboard that was configured by USBKEYB, you would

do the following:

USBKEYB Descriptors

DOS USB DRIVER INTRODUCTION            Page 103 of 186

A USB Keyboard is also part of a larger USB classification called

Human Interface Devices (HID's), and therefore has a special

Descriptor called an HID Report Descriptor.  The HID Report

Descriptor "describes", among other things, how many buttons and

LED's the keyboard actually has.  To display the HID Report

Descriptor of the last keyboard that was configured by USBKEYB, you

would to the following:

USBKEYB ReportDescriptor

Miscellaneous Notes

�������������������

To have USBKEYB show you the current status of the USB keyboards that

it is managing, you would to the following:

USBKEYB Status

This will display "general" information about the USB keyboards (such

as typing method and typeamatic settings).  It will also display any

custom key mapping you have in place for either the "standard" USB

Keys or the USB System Keys.  In addition, it will also display

details for each of the individual USB keyboards you may have

installed (such as Host Index & Device Address).  If something is not

working quite right with one of the keyboards, running USBKEYB with

the Status option switch will usually be where you start your

troubleshooting path.

While USBKEYB is installed in memory, it provides a BIOS-level

Application Programming Interface (API) that allows other programs to

simulate keystrokes (Scan Codes) the same way that USBKEYB does.

Details of the API are rather lengthy and complicated, and will not

be discussed here.  They will be discussed in the official

documentation for USBKEYB, to be released some time in the future.

If you want to review how the API works before the official

documentation is released, you can freely download and peruse the

source code for USBKEYB.

USBKEYB is not a "perfect" keyboard driver -- there are certain

situations and environments and programs where it does not work

correctly.  Hopefully, it will work in every situation where you need

it to work.  However, if it does not, you will probably need to leave

the USB keyboard in Legacy Support mode, and let the BIOS manage the

keyboard instead of USBKEYB.  Legacy Support has many limitations

that USBKEYB does not (and vice versa), and which one is the most

appropriate for you to use will depend on your particular hardware

and software needs and capabilities.

DOS USB DRIVER INTRODUCTION            Page 104 of 186

Keyboards sometimes send certain keys (multimedia keys and "system"

keys) in unusual ways that USBKEYB does not understand.  In the

future, the DOS drivers will be improved to add support for these

"unusual" keyboards.  In the meantime, you should be able to use all

of the "regular" keys (compatible with standard 101/102 PS2

keyboards) with no problem.

SCANTEST

��������

Version:     1.10

Prog Type:   Test

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete

Environment: None

Redirection: No

Purpose:     Keyboard tester

TSR Memory:  0 kB (not a TSR)

SCANTEST is a low-level keyboard testing program.  It's included here as

a "companion" program to help you test and configure USBKEYB (discussed

above on page 90), but you can use it test any kind of keyboard.  This

version of SCANTEST is actually a slightly updated version of the

SCANTEST that was included with one of my other programs (SCANCODE) that

was last released several years ago.  The updates added to this version

include descriptions for several of the "multimedia" keys on the

keyboard (volume controls, CD player play/pause, etc.), an indicator for

"repeating" keystrokes (typeamatic repeats), and support for Long File

Names in the output.  If you've used earlier versions of SCANTEST, the

differences are relatively minor and will probably not be all that

obvious to you.

When you run SCANTEST, it "intercepts" every keystroke you type from the

keyboard, and displays the Scan Code of the keystroke (a number or a

series of numbers), along with a description of the keystroke.  Note

that the description is only valid for United States QWERTY keyboards,

so if that's not the kind of keyboard you're using, the descriptions

will probably be different than what you're expecting.  The Scan Code

numbers displayed will always be correct, no matter what kind of

keyboard you have.  To exit the SCANTEST program, simply press <Escape>

on the keyboard.

DOS USB DRIVER INTRODUCTION            Page 105 of 186

It is important that you realize that SCANTEST is a low-level program,

and what is displays are the Scan Codes that are coming from the

keyboard, and are NOT the ASCII characters that usually appear on the

screen as you are typing.  Scan Codes and ASCII characters are

COMPLETELY different animals, and there is no direct relationship

between the two.  You can read the documentation for my SCANCODE program

if you want to investigate this further -- we will not discuss it in

detail here.

The only option switch SCANTEST provides is the ability to "tee" the

displayed output to both the screen and to another location (DOS file or

device) at the same time.  For instance, let's say you intend to have a

long SCANTEST "session", and you would like to remember the entire

session by sending everything that gets displayed on the screen to the

printer (LPT1:).  To accomplish this, you would start SCANTEST the

following way:

SCANTEST LPT1:

If you do this, everything that SCANTEST displays on the screen will

also be sent to LPT1:.

You can "tee" the output to almost any DOS device or file you want to,

not just the printer.  Here are a some examples:

SCANTEST C:\SCANTEST.TXT

SCANTEST "C:\My Documents\ScanTest.Out"

Obviously, the second example above will only work if you're currently

working in an environment that supports Long File Names (LFN's).

The one Device that you cannot "tee" the output to is the CON: Device,

because it is the same thing as the screen.  That is, you cannot send

the output to the screen (CON:), and then try to "tee" the output to the

same place (the screen or CON:).  I can't imagine anybody trying to do

this (at least not on purpose), but if you do SCANTEST will generate an

error.

DOS USB DRIVER INTRODUCTION            Page 106 of 186

USBMOUSE

��������

Version:     0.10

Prog Type:   Device/Interface Driver

Source Code: ASM (A86 by Eric Isaacson)

Status:      In progress, several major items to address

Environment: USBMOUSE

Redirection: Yes

Purpose:     Driver for up to 8 USB Mice

TSR Memory:  7.9 kB

USBMOUSE is a Device/Interface Driver program for up to eight USB Mice.

Most other DOS USB architectures do not provide any mouse support at

all, or if they do, it is very limited and virtually useless.  If you

want to use a USB mouse with the other DOS USB architectures, you must

usually use the Legacy Support in the BIOS, which has certain

limitations that USBMOUSE does not.

USBMOUSE allows you to have up to eight USB Mice installed on the

computer at the same time, and any or all of them can be used at any

time.  You can also have a Legacy (PS2) mouse installed and use it was

well.  On a "shared" computer that many different people use, for

example, it might be useful to have left-handed and right-handed mice,

trackballs, touchpads, etc., instead of forcing every user to use the

exact same mouse.  Or, if there is a training session going on, it might

be useful to have both the trainee and the trainer each have their own

mouse so that they're not constantly shifting things back and forth to

be able to both use the same mouse.  Having more than one mouse on a

computer is actually quite useful in certain situations.

Before USBMOUSE can be installed into memory, you must first have at

least one USB Host Driver (USBUHCI{L}) installed in memory.  To install

USBMOUSE into memory, you can simply run it with no option switches:

USBMOUSE

DOS Mouse Drivers

�����������������

In order to use a mouse with most DOS programs, you must install a

program called a DOS Mouse Driver.  The most common one is probably

Microsoft's MOUSE.EXE, which comes with many later versions of DOS.

MOUSE.EXE is certainly not the only DOS Mouse Driver available, and,

at least in my opinion, is certainly not the best one, either.

DOS USB DRIVER INTRODUCTION            Page 107 of 186

I think the best DOS Mouse Driver available is CTMOUSE, which you can

download for free (http://cutemouse.sourceforge.net).  As far as I

know, it is the only DOS Mouse Driver that is still being updated,

and uses much less memory (RAM) than any other mouse driver I have

seen.  All of the "big names" in the mouse business (Microsoft,

Logitech, Mouse Systems, etc.) stopped updating their memory-

intensive DOS Mouse Drivers a long time ago.

The main purpose of a DOS Mouse Driver (like CTMOUSE) is to provide a

common interface to DOS programs that want to use a mouse.  Even in

the early days of computing, there were are least two different ways

to attach a mouse to the computer: with a special card that plugged

into one of the expansion slots (called a Bus Mouse), or with a

serial port like an external modem uses (called a Serial Mouse).  A

little later on, PS2-attached mice were developed, and eventually

that became the most popular way, by far, to attach mice.  The DOS

Mouse Driver understands how each of the three different hardware

protocols works (Bus, Serial, and PS2), and provides a simple

interface that DOS programs can use to control the mouse.  The DOS

program doesn't know (or care) how the mouse is attached to the

computer or which hardware protocol is being used -- it just needs to

understand how the DOS Mouse Driver works.

In the time since the mouse manufacturers stopped updating their DOS

Mouse Drivers, new ways of attaching mice have emerged.  The most

popular ways now are USB and Bluetooth, which use completely

different protocols than each other and than any of the previous

attachment methods (Bus, Serial, and PS2).  In the future, there may

be even more new ways of attaching mice that will eventually replace

USB and Bluetooth.  The DOS Mouse Drivers that stopped development

before USB and Bluetooth became popular (all of them except CTMOUSE)

will never be able to work natively with the newer mouse hardware.

USBMOUSE is NOT a DOS Mouse Driver, and is NOT a replacement for a

program like CTMOUSE.  Even if you have USBMOUSE installed, you still

need a DOS Mouse Driver like CTMOUSE in order to actually make the

mouse do something useful.  However, even CTMOUSE (the most modern

DOS Mouse Driver available) does not understand how USB mice work --

it only understand Bus, Serial, and PS2 mice.  That's where USBMOUSE

comes in.

What USBMOUSE does is "talk" to the USB Mice using the native USB

protocol, and then emulates a PS2 mouse in the computer's BIOS.

USBMOUSE makes CTMOUSE think it is talking to a PS2 mouse, when it's

actually talking to a USB mouse.  Because CTMOUSE is still being

developed and updated, it may eventually include native support for

USB mice.  However, even if it doesn't, it will still be possible to

use USB mice in DOS thanks to USBMOUSE.

DOS USB DRIVER INTRODUCTION            Page 108 of 186

PS2 Mouse BIOS

��������������

In the early days of computers, there were two kinds of mice: serial

(which plugged into the same kind of serial port that a modem uses)

and bus (which required a special card that plugged into the

expansion bus).  Later on, when IBM developed the PS2 interface for

mice (and keyboards), it also developed a standard BIOS-level

(software) interface for the mouse.

Development of a standard software interface for the mouse should

have been a huge step forward for mouse developers, since they could

now use almost any proprietary hardware mechanism they wanted and

still have programs be able to work with the mouse.  The DOS Mouse

Drivers simply need to "talk" to the mice using the PS2 BIOS software

interface, and it doesn't matter how the mouse actually works at the

hardware level.

Unfortunately, every mouse manufacturer I know of developed their DOS

Mouse Driver to interact directly with the PS2 mouse hardware, rather

than interacting with the PS2 BIOS software.  As a result, those DOS

Mouse Drivers are now virtually useless, since they cannot work with

the new mice (like USB and Bluetooth and whatever else develops in

the future) that don't use the PS2 hardware interface.

CTMOUSE is the only DOS Mouse Driver I know of that interacts with a

PS2 mouse at the BIOS level rather than the hardware level.

Therefore, CTMOUSE is also the only DOS Mouse Driver I know of that

is even capable of working with newer mice, like USB and Bluetooth.

One thing to keep in mind, however, is that you must use CTMOUSE

version 1.9 or earlier, or version 2.1 or later.  For a while,

CTMOUSE got "off track", and version 2.0 only interacts directly with

the PS2 mouse hardware.  With version 2.1 they are back to using the

BIOS which is the "right" way to do things (as far as I'm concerned,

anyway).

USBMOUSE emulates a PS2 mouse in the BIOS of the computer.  Any DOS

Mouse Driver that interfaces with the PS2 mouse at the BIOS

(software) level instead of at the hardware level should work with

USBMOUSE.  Unfortunately, the only DOS Mouse Driver I know of that

actually does that is CTMOUSE.  If there is another one out there, I

would certainly like to know about it.

DOS USB DRIVER INTRODUCTION            Page 109 of 186

IntelliMouse Protocols

����������������������

Back in the early days, mice were different than they are today.

Most mice only had two buttons, though some had three buttons.  None

of the mice had the scrolling wheels that nearly all modern mice

have.  Back then, the protocol that was used to transfer data between

the mouse and the computer only allowed for at most three buttons,

and did not have enough room to transfer the extra data required by a

wheel even if the mouse had one.

Somewhere along the line, somebody thought it would be a good idea to

add a vertical scrolling wheel to a mouse.  Of course, the original

protocol didn't have enough room for the data associated with the

wheel, so Microsoft created the IntelliMouse protocol.  Later yet

again, somebody decided that even more would be better, and thought

that mice should have two scrolling wheels (a horizontal as well as a

vertical) and five buttons (instead of just two or three).  As a

result, Microsoft created the IntelliMouse2 protocol (actually, I

don't know what the official name is, but I call it IntelliMouse2).

The three protocols (PS2, IntelliMouse, and IntelliMouse2) are

completely incompatible with each other.  That is, the software (DOS

Mouse driver, like CTMOUSE) must know which kind of mouse (which

protocol) is being used by the mouse.  All PS2 mice, whether they

support the IntelliMouse(2) protocols or not, initialize themselves

in PS2 mode (the protocol that cannot support any wheels).  If the

software (DOS Mouse Driver) understands how the IntelliMouse protocol

works, it will then "ask" the mouse hardware if it supports the

IntelliMouse protocol.  If the mouse hardware does, the software and

hardware will then use the IntelliMouse protocol to communicate with

each other.  If the software understands how the IntelliMouse2

protocol works, it will then "ask" the mouse hardware if it supports

the IntelliMouse2 protocol.  If the mouse hardware does, the software

and hardware will use the IntelliMouse2 protocol to communicate with

each other.

By progressing this way (from PS2 to IntelliMouse to IntelliMouse2),

the software (DOS Mouse Driver) controls how many wheels and buttons

are actually available to be used.  So, even if the mouse has a

wheel, if the DOS Mouse Driver doesn't understand the IntelliMouse

protocol, you'll never be able to use the wheel to do anything

useful.  The popularity of scroll wheels on mice happened well after

all of the mouse hardware manufacturers stopped updating their DOS

Mouse Drivers, so none of the OEM DOS Mouse Drivers have support for

scrolling wheels.  The only DOS Mouse Driver I know of that includes

support for any wheels at all is CTMOUSE, and it only understands the

IntelliMouse protocol.  It does not understand the IntelliMouse2

protocol (at least not yet), so does not support a second wheel or

more than 3 buttons.  But this is still far better than any other DOS

Mouse Driver available, which don't support any wheels at all.

DOS USB DRIVER INTRODUCTION            Page 110 of 186

Even if you use CTMOUSE, which understands and supports a scrolling

wheel, it still doesn't mean you'll be able to do anything with the

wheel.  In addition to the mouse hardware and the DOS Mouse Driver,

the particular DOS program you're using must also understand and

support scrolling wheels.  There aren't very many new DOS programs

being written or updated these days, and there are only a few that

are able to do anything with a scrolling wheel.  The CTMOUSE web site

(http://cutemouse.sourceforge.net) lists some programs that are able

to use the scrolling wheels.

Also note that even if a mouse supports the IntelliMouse protocol, it

doesn't necessarily mean it actually has a scrolling wheel, or even

that it actually has three buttons.  It just means that it MAY have a

scrolling wheel.  Likewise, support of the IntelliMouse2 protocol

doesn't necessarily mean the mouse actually has two wheels and five

buttons, it just means that the mouse MAY have that many.  The

IntelliMouse and IntelliMouse2 protocols don't provide a way for the

mouse to tell the computer how many buttons or wheels the mouse

actually DOES have, only how many it MAY have.

PS2 Mouse Hardware Emulation

����������������������������

In spite of the previous statements about only emulating the mouse at

the BIOS (software) level, USBMOUSE is also able to emulate the PS2

mouse hardware under certain circumstances.  Unfortunately, even

though the hardware emulation does in fact work, there are still many

problems with it and I do not recommend using mouse hardware

emulation.

PS2 keyboards and PS2 mice actually use the exact same control

hardware inside the computer.  The PS2 mouse is treated as sort of an

"extension" of the keyboard, and is not really a truly separate

device.  When USBMOUSE is attempting to emulate the PS2 mouse

hardware, for some reason, it also affects the PS2 keyboard.  That

is, when USBMOUSE is in hardware emulation mode, the mouse usually

works just fine, but your keyboard doesn't work correctly any more.

On my computers, sometimes two keystrokes are generated when you only

pressed the key one time, some keystrokes are simply ignored when you

type them, sometimes the keyboard beeps annoyingly at you when you

press keys, and the keyboard seems especially sluggish and

unresponsive.  I have no idea why any of this happens, and have not

been able to resolve it.

DOS USB DRIVER INTRODUCTION            Page 111 of 186

Even though the mouse hardware emulation has "bugs" in it, I am

leaving it installed in USBMOUSE.  You may find it useful in certain

situations, such as in a program that uses the mouse but doesn't use

the keyboard.  Hardware emulation could also, at least theoretically,

let you use your USB Mouse in a program that "bypasses" the DOS Mouse

Driver and interacts directly with the PS2 mouse hardware (there are

a few of those kinds of programs around).

In addition, when USBMOUSE is using hardware emulation instead of

BIOS (software) emulation, you cannot control the PS2 mouse.  For

instance, the options discussed below allow you to disable and enable

(turn off and on) individual mice, exchange the left & right mouse

buttons, view the "raw" data that is coming from the mouse, etc.  In

BIOS (software) emulation mode, USBMOUSE also controls the "real" PS2

mouse data, and can do all of these things with the PS2 mouse

(disable, exchange buttons, etc.).  However, in hardware emulation

mode, USBMOUSE does not control the PS2 mouse, and the PS2 mouse will

always do what it wants to do instead of what USBMOUSE wants it to

do.

I encourage you to experiment with the hardware emulation just to see

what happens, though I suspect in the end you will end up deciding to

use the faster, more reliable, and more manageable BIOS emulation

instead.

By default, USBMOUSE uses BIOS (software) emulation.  To have

USBMOUSE use hardware emulation instead of software emulation, you

would do the following:

USBMOUSE BIOSEmulation: No

To have USBMOUSE switch back again to using software emulation, you

would do the following:

USBMOUSE BIOSEmulation: Yes

Again, keep in mind that CTMOUSE (http://cutemouse.sourceforge.net)

is the only DOS Mouse Drive I know of that will work with USBMOUSE in

BIOS emulation mode.  If you know of any others, I would be very

interested in finding out what they are.

DOS USB DRIVER INTRODUCTION            Page 112 of 186

Hardware Emulation Methods

��������������������������

USBMOUSE has three different Methods of emulating mouse movements

and button presses at the hardware level, simply called Methods 1,

2, & 3.  These are essentially the same thing as the three

"typing" Methods discussed above for USBKEYB (page 90).

Method 1 uses a special function built in the BIOS (hardware) of

many modern computers.  In other words, for it to work properly,

the hardware on your computer (motherboard) must be compatible

with Method 1.  If you hardware is compatible with Method 1, it is

the best and most reliable of the three Methods.

Method 2 uses a combination of hardware and software to emulate

the mouse hardware.  Much like to Method 1, it requires that the

BIOS hardware on the motherboard be compatible with Method 2.

Unlike Method 2 with USBKEYB (which I've never gotten to work on

any of my computers), however, I have gotten Method 2 with

USBMOUSE to work on some of my computers.

Method 3 strictly uses software to emulate the mouse hardware.

Similar to USBKEYB, as USBMOUSE is installing itself into memory,

it performs a relatively simple test to determine which of the

three Methods it should use to simulate keystrokes if it is using

hardware emulation rather than BIOS (software) emulation.  The

first test it performs is to see if you have provided it an option

switch to tell it which Method you want it to use.  To have

USBMOUSE simulate keystrokes using Method 1, for example, you

would do:

USBKEYB Method 1

If you do not specifically provide USBMOUSE with a Method to use,

it will try Method 1, and if unsuccessful, will then try Method 2.

If neither Method 1 nor Method 2 works, USBMOUSE will simply

default to using Method 3.  Note that USBMOUSE does not actually

test Method 3, but simply assumes that it should work if neither

Method 1 nor Method 2 does.

DOS USB DRIVER INTRODUCTION            Page 113 of 186

Enabling & Disabling Mice

�������������������������

USBMOUSE will let you use up to 8 USB Mice, along with a "real" PS2

mouse, all at the same time.  Even if you have more than one mouse

attached to the computer, you will usually only want to use one of

them at a time.  If you have extra mice, and want to disable some of

them so that nothing happens when you accidentally bump them, you can

do that.  For instance, my laptop has a touch-pad "mouse" on it

located directly underneath the keyboard.  I am constantly "bumping"

the touch-pad by accident with my palms or my thumbs as I am typing,

and really get annoyed as the cursor moves all over the screen and I

end up typing things in the wrong places.  So, by default, I disable

(turn off) the touch-pad on my laptop and plug in an external USB

mouse (when I even want to use a mouse).

By default, all mice (USB and PS2) are enabled (turned on) in

USBMOUSE.  However, USBMOUSE will let you enable (turn on) and

disable (turn off) each of the individual USB mice, as well as the

PS2 mouse.  To Disable a particular mouse, you must provide a Mouse

Index number (between 0 and 8).  Index 0 is the first USB mouse, and

Index 7 is the last (eighth) USB mouse.  Index 8 is the PS2 mouse.

For example, to Disable the first USB mouse (Index 0), you would do

the following:

USBMOUSE Disable 0

To re-enable the PS2 mouse (Index 8), you would to the following:

USBMOUSE Enable 8

Note that if USBMOUSE is in hardware emulation mode, disabling and

enabling the PS2 mouse does not actually do anything.  You must be in

BIOS (software) emulation mode before USBMOUSE can control the PS2

mouse hardware.

Exchanging Left & Right Buttons

�������������������������������

A mouse can be configured either as a right-handed mouse or a left-

handed mouse.  Right-handed is the most common, and the only

difference between the two is which mouse button is considered the

"main" selector button.  In both cases, it is the one you normally

press with your index finger (the left button in a right-handed

mouse, and the right button in a left-handed mouse).  To the mouse

hardware itself, it doesn't make any difference -- a button is simply

a button, and none of them are "more important" than any of the

others.  It's only in the software that it matters.

DOS USB DRIVER INTRODUCTION            Page 114 of 186

USBMOUSE lets you exchange the left and right buttons on the mice,

changing them from right-handed to left-handed mice (and vice versa).

You can exchange the buttons of an individual mouse, or exchange the

buttons on all of the mice at the same time (including the PS2

mouse).  To indicate which mouse you want to exchange buttons on, you

must provide a Mouse Index number (between 0 and 8).  Index 0 is the

first USB mouse, and Index 7 is the last (eighth) USB mouse.  Index 8

is the PS2 mouse.  For example, to exchange buttons on the first USB

mouse (Index 0), you would do the following:

USBMOUSE Exhange 0

If you later wanted to return the buttons back to their normal (un-

exchanged) state, you would just exchange them again:

USBMOUSE Exhange 0

You can also exchange the left and right buttons for all mice at the

same time by specifying Mouse Index 255:

USBMOUSE Exchange 255

Note that when you tell USBMOUSE to exchange the buttons for all mice

at the same time, it is a truly "global" button exchange, as opposed

to 9 individual exchanges.  That is, if you do a global exchange

(Index 255), and then also do an individual exchange (say, Index 0),

the end result will be that the individual device will not be

exchanged at all (exchanging something twice is the same as doing

nothing).

Also note that some DOS Mouse Drivers (like CTMOUSE) allow the left

and right buttons to be exchanged, as do some individual DOS

programs.  If you mess around and try to exchange buttons in too many

different places, you will find yourself totally confused and may

still have things backwards from what you want them to be.

Note that if USBMOUSE is in hardware emulation mode, attempting to

exchange the left and right buttons (either globally or individually)

does not exchange the buttons on the PS2 mouse.  You must be in BIOS

(software) emulation mode before USBMOUSE can control the PS2 mouse

hardware.

DOS USB DRIVER INTRODUCTION            Page 115 of 186

Descriptors

�����������

To display the General Descriptors (Device, Configuration, Interface,

etc.) of the last mouse that was configured by USBMOUSE, you would do

the following:

USBMOUSE Descriptors

A USB Mouse is also part of a larger USB classification called Human

Interface Devices (HID's), and therefore has a special Descriptor

called an HID Report Descriptor.  The HID Report Descriptor

"describes", among other things, how many buttons and wheels the

mouse actually has.  To display the HID Report Descriptor of the last

mouse that was configured by USBMOUSE, you would to the following:

USBMOUSE ReportDescriptor

Displaying Raw Mouse Data

�������������������������

USBMOUSE will also let you look at the Raw (USB) data as it is coming

from the mice.  This can be used as a troubleshooting aid, or just

lets you "investigate" what's going on in the background if you're

interested.  To view the raw mouse data, you would do the following:

USBMOUSE RawData

The display will show a total of 8 columns.  The first column is

labeled "Buttons, and the remaining columns are labeled "Byte 2" thru

"Byte 8".  USB Mice can send up to 8 bytes of data to the computer,

though they usually only send 4 or 5 bytes.

The first byte (8 bits) of data sent by a standard USB Mouse is a

bit-map of the buttons that are currently pressed on the mouse.

Using this format, a mouse can have up to 8 buttons, though they

typically only have 3 to 5.  The RawData screen labels the first

three buttons L (Left), R (Right), and C (Center).  The remaining

buttons are labeled with the numbers 4 thru 8.

The remaining bytes (2 thru 8) are used to indicate mouse or wheel

movements, with each byte dedicated to a specific direction or wheel.

The byte is simply a number between -127 and +127, with a positive

number indicating movement in one direction (usually right or down),

and a negative number indicating movement in the other direction

(usually left or up).  The absolute value of the number (1 thru 127)

indicates "how much" or "how fast" the movement was, with 1 being

slow and 127 being very fast.

DOS USB DRIVER INTRODUCTION            Page 116 of 186

The first movement byte (labeled "Byte 2") is usually the horizontal

(X) movement of the mouse, and "Byte 3" is usually the vertical (Y)

movement of the mouse.  The remaining bytes (4 thru 8) are usually

for any scrolling wheels that the mouse may have, with one byte

dedicated to each wheel.

Note that the format of the raw data shows you what is coming from

the USB mice.  This is COMPLETELY different than the format of the

data that comes from a PS2 mouse.  USBMOUSE takes the data coming

from the USB mice, translates it into the PS2 format, and then

"sends" (emulates) it into the DOS Mouse Driver so that the Driver

thinks it is "seeing" a PS2 mouse instead of a USB mouse.  The

PS2MTEST program below (page 121) allows you to look at the data

coming from the mice in PS2 format.

When you are through viewing the raw data, you simply press any key

on the keyboard and USBMOUSE will stop.

Controlling Mouse Sensitivity

�����������������������������

PS2 mice let you have a certain amount of control over how

"sensitive" they are to movement.  That is, you can adjust the mouse

to react to the slightest touch of your hand, or you can adjust it so

that you have to drag it halfway across the table before you see much

of anything happen on the screen.  Unfortunately, DOS does not

provide an easy way to control these settings, so most people just

leave them at the default values.

USBMOUSE provides a way for you to control the sensitivity settings

of the PS2 mouse, even when USBMOUSE is not installed in memory.

That is, if you want to, you can simply use USBMOUSE to control the

sensitivity of your PS2 mouse even if you never use a USB mouse.

USBMOUSE does not require any memory if the only thing you're using

it for is to control the mouse sensitivity.

However, remember that the main purpose of USBMOUSE is to control USB

mice.  By default, USBMOUSE assumes that you are wanting to use it to

control USB mice, and therefore, by default, will try to install

itself into memory as a TSR (Terminate-but-Stay-Resident) program.

If you do not want USBMOUSE to try and install itself into memory,

you can do the following:

USBMOUSE Install:No   {other options}

DOS USB DRIVER INTRODUCTION            Page 117 of 186

The most basic sensitivity setting for a PS2 mouse is called the

Resolution.  The Resolution is measured in Counts Per Millimeter

(CPM), and can be set to either 1, 2, 4, or 8.  If the Resolution is

set at 1 CPM (the least sensitive setting), and you move the mouse 1

millimeter, the mouse will send a signal to the computer saying "the

mouse moved 1 Count."  If the Resolution is set at 8 CPM (the most

sensitive setting), and you move the mouse only 1/8 of a millimeter,

the mouse will send a signal to the computer saying "the mouse moved

1 Count."  When a PS2 mouse "boots up", it has a default Resolution

of 4 CPM.  To set the Resolution of the PS2 mouse (and all of the USB

mice, if USBMOUSE is installed in memory) to 8 CPM, you would do the

following:

USBMOUSE Resolution:8

If you are only using a PS2 mouse, and do not want USBMOUSE to try

and install itself into memory as you changed the resolution, you

would do the following:

USBMOUSE Install:No Resolution:8

The next setting that changes the sensitivity of PS2 mice is called

the Scaling Factor.  The Scaling Factor can either be 1 (the default)

or 2.  When the Scaling Factor is set at 2, and you move the mouse

quickly, it accelerates the mouse movement even more (the cursor on

the screen moves twice as fast as it normally does).  If you move the

mouse slowly, however, the cursor moves across the screen at the

normal (un-doubled) speed.  To set the Scaling Factor to 2, you would

do the following:

USBMOUSE ScalingFactor:2

The last setting related to mouse sensitivity is called the Sampling

Rate.  Technically, the Sampling Rate does not really affect the

"sensitivity" of the mouse as such, but if set incorrectly can make

the mouse appear sluggish and unresponsive (it will appear "jerky").

The PS2 mouse is actually being polled by the computer several times

a second (rather than sending data in an "ad hoc" fashion), and the

Sample Rate controls how may times every second the mouse is polled.

It is measured Samples Per Second (SPS), and can be set to 10, 20,

40, 60, 80, 100, or 200, with the default setting at 100 SPS.  The

default setting of 100 SPS is usually very adequate, and there is

rarely a reason to change it (unless you just want to experiment to

see what happens).  I have personally experimented with the settings

to some degree, and don't see much practical difference between any

of the settings that are at least 60 SPS.  To set the Sampling Rate

to 60 SPS, you would do the following:

USBMOUSE SamplingRate:60

DOS USB DRIVER INTRODUCTION            Page 118 of 186

I should also point out that most USB mice cannot be sampled at more

than 100 SPS, and USBMOUSE does not try to poll any USB mouse at more

than 100 SPS.  Therefore, even if you set the Sampling Rate to 200,

the USB mice will not actually be polled that often (the PS2 mouse

will be, but not the USB mice).

Resetting the PS2 Mouse

�����������������������

There are times when the PS2 mouse gets "gummed up" and stops working

correctly for some reason.  The most likely scenario for this to

happen is when a program tries to access the PS2 mouse hardware

directly, instead of using the DOS Mouse Driver or the PS2 Mouse

BIOS.  If the PS2 mouse has stopped working correctly, and you need

to "reset" it, you should do the following:

USBMOUSE Reset

Under normal circumstances, you should not need to use the Reset

option switch.

Miscellaneous Notes

�������������������

To have USBMOUSE show you the current status of the USB and PS2 mice

that it is managing, you would to the following:

USBMOUSE Status

This will display "general" information about the USB mice (such as

emulation method and global button exchange status).  In addition, it

will also display details for each of the individual USB and PS2 mice

you may have installed (such as Host Index, Device Address, and

button exchange status).  If something is not working quite right

with one of the mice, running USBMOUSE with the Status option switch

will usually be where you start your troubleshooting path.  Keep in

mind, however, that the data shown for the PS2 mouse will not be

valid unless USBMOUSE is in BIOS (software) emulation mode.  When

USBMOUSE is in hardware emulation mode, it is not actually managing

the PS2 mouse hardware.

DOS USB DRIVER INTRODUCTION            Page 119 of 186

As USBMOUSE is installing itself into memory, it must perform a

hardware reset of the PS2 mouse, which may take several seconds to

perform.  Once this has happened, USBMOUSE has inserted itself "on

top of" the PS2 mouse hardware and BIOS software, and the lengthy

delay that occurs during mouse resets will no longer be required.

However, if you set USBMOUSE to hardware emulation mode, that will no

longer be the case.  If USBMOUSE is in hardware emulation mode, it is

not controlling the PS2 mouse hardware, and lengthy mouse resets will

be required again.  Mouse resets are required at the beginning and

end of every program that uses a mouse, so using hardware emulation

mode will probably slow your programs down considerably.

USBMOUSE performs the necessary steps to make sure all of your mice

appear to be the same (to have the same number of wheels and buttons)

to the DOS Mouse Driver.  For example, let's say you are using

CTMOUSE, which includes support for one scrolling wheel and three

buttons.  Let's also say you have an "old" PS2 mouse with just two

buttons, and a "new" USB mouse with two scrolling wheels and 5

buttons.  USBMOUSE will perform the necessary adjustments and

translations to make sure everything works together as it should (at

least if you are using USBMOUSE in BIOS emulation mode).  That is,

USBMOUSE will make the DOS Mouse Driver (CTMOUSE) believe that the

PS2 mouse has a scrolling wheel and three buttons (even though it

actually doesn't), and will make the DOS Mouse Driver believe that

the USB mouse only has one wheel and three buttons (even though it

actually has more than that).  If CTMOUSE is updated in the future to

support more than one wheel and more than three buttons, USBMOUSE is

already set up to automatically handle that transition as well.

As discussed above in  IRQ & I/O Resources  (page 56), on all modern

computers IRQ #12 is dedicated to the PS2 mouse, or to any software

(like USBMOUSE) that emulates the PS2 Mouse.  IRQ #12 must not be

used for any other purpose, including a USB host controller.  If you

have some other device that is using IRQ #12, you should probably

change it to to use a different IRQ or risk serious problems &

crashes when you use a mouse.  USBUHCI{L} has options to let you

change the IRQ (see page 56) if the BIOS happens to set your USB Host

Controller to use IRQ 12 and you want to use USBMOUSE.

DOS USB DRIVER INTRODUCTION            Page 120 of 186

PS2MTEST

��������

Version:     1.00

Prog Type:   Test

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete

Environment: None

Redirection: No

Purpose:     PS2 mouse BIOS tester

TSR Memory:  0 kB (not a TSR)

PS2MTEST is a program designed to test your PS2 mouse.  It does this by

displaying the "raw" data that is coming from the mouse through the PS2

mouse BIOS (software).  It's included here as a "companion" program to

help you test USBMOUSE (discussed directly above on page 107), but you

can also use it to simply test your PS2 mouse.

What you will see on the screen when you run PS2MTEST will be four words

(eight bytes) of data on the left, labeled "Raw PS2 Mouse Data".  The

PS2 Mouse BIOS always transfers eight bytes of data, even though only

three or four of the bytes actually contain any useful data.  This is

where the raw data (numbers) will be displayed as you move the mouse and

press the buttons.

To the right of the raw data on the screen are three columns, labeled

"Buttons", "Motion", and "Wheels".  This is where the raw data is

translated into meaningful descriptions (button presses, mouse

movements, and scrolling wheel clicks).  Exactly how the data gets

translated depends on which protocol is currently being used by the

mouse (PS2, IntelliMouse, or IntelliMouse2, described above on page

110).  Only three bytes of actual data are transferred when the mouse is

in PS2 mode, and four bytes are transferred when the mouse is in

IntelliMouse or IntelliMouse2 mode.  Exactly which bits of data

correspond to which buttons and movements changes based on the protocol

currently in use, and those details will not be discussed here.

Information about that is available at various places on the internet if

you want to search for it.

By default, PS2MTEST will use the most sophisticated protocol supported

by the mouse hardware and BIOS (IntelliMouse2 is the first choice,

followed by IntelliMouse, with PS2 as the last choice).  If you are

using USBMOUSE in BIOS (software) emulation mode, it will always support

the IntelliMouse2 protocol.  If you are not using USBMOUSE at all, or

are using USBMOUSE in hardware emulation mode, the most sophisticated

protocol supported will depend on the PS2 mouse hardware (how many

wheels and buttons it has).

DOS USB DRIVER INTRODUCTION            Page 121 of 186

Although the default is for PS2MTEST to use the most sophisticated

protocol, you can force it to use a less sophisticated protocol if you

want to for testing or educational purposes.  To do this, you must

provide to PS2MTEST a number which corresponds to the maximum number of

wheels supported by the protocol (0 = PS2, 1 = IntelliMouse, and 2 =

IntelliMouse2).  For example, to provide the same functionality as the

default, you would do the following:

PS2MTEST 2

This will test the mouse using the IntelliMouse2 protocol (maximum of 2

wheels and 5 buttons) if the mouse supports it.  If the mouse does not

support the IntelliMouse2 protocol, but does support the IntelliMouse

protocol, this will test the mouse using the IntelliMouse protocol.  If

the mouse only supports the PS2 protocol, this will test in using the

PS2 protocol.

To test the mouse using the IntelliMouse protocol, even if it supports

the IntelliMouse2 protocol, you would do the following:

PS2MTEST 1

If the mouse does not support the IntelliMouse protocol, PS2MTEST will

test the mouse with the PS2 protocol.

To force PS2MTEST to test the mouse with the PS2 protocol, no matter how

many wheels it actually has, you would do the following:

PS2MTEST 0

After you are through viewing the raw PS2 mouse data, simply press a key

on the keyboard and PS2MTEST will stop.  After PS2MTEST stops, it will

attempt to restore the PS2 mouse BIOS and DOS Mouse Driver to the same

exact state it was in before you ran PS2MTEST.  While the restoration

process usually works just fine, there are times when it does not.  If

the restoration process does not seem to work correctly, you will need

to reset or reinstall the DOS Mouse Driver again.

DOS USB DRIVER INTRODUCTION            Page 122 of 186

USBJSTIK

��������

Version:     0.11

Prog Type:   Device/Interface Driver

Source Code: ASM (A386 by Eric Isaacson)

Status:      Complete, a few minor tweaks required

Environment: USBJSTIK

Redirection: Yes

Purpose:     Driver for up to 8 USB Joysticks

TSR Memory:  7.4 kB

USBJSTIK is a Device/Interface Driver program for up to eight USB

joysticks or gamepads.  USBJSTIK emulates a "regular" joystick (attached

to a game port) at the BIOS (software) level.  Unfortunately, most DOS

programs that use a joystick do not interact with the joysticks at the

BIOS level (even though they should at least have that as an option).

Rather, most DOS programs interact with the joysticks directly at the

hardware level.

USBJSTIK can also emulate joysticks at the hardware level, rather than

the BIOS (software) level, using a process called I/O Virtualization.

Emulating a joystick at the hardware level allows you to use a USB

joystick with almost any DOS program you can imagine.  However, USBJSTIK

cannot perform I/O Virtualization on its own.  In order Virtualize I/O,

you must have an appropriate memory manager installed via your

CONFIG.SYS file.  Appropriate memory managers include Microsoft's EMM386

version 4.46 or later, and 386MAX.  As far as I know, other memory

managers (such as QEMM or the EMM386 program that comes with FreeDOS) do

not support I/O Virtualization, and cannot be used with USBJSTIK if you

want to emulate joysticks at the hardware level.

In general, you cannot have both a regular (game port) joystick and a

USB joystick (emulated by USBJSTIK) at the same time.  There are certain

unique situations (discussed below) where it is possible, but these

situations are rare.  If you have a joystick attached to a regular game

port, you are generally better off using it rather than attempting to

use a USB joystick.

Before USBJSTIK can be installed into memory, you must first have at

least one USB Host Driver (USBUHCI{L}) installed in memory.  To install

USBJSTIK into memory, you can simply run it with no option switches:

USBJSTIK

DOS USB DRIVER INTRODUCTION            Page 123 of 186

Joystick Protocols

������������������

Game ports are very simple devices.  There are only 8 bits of data

that can be transferred from the joystick to the computer.  There are

four digital bits, which are associated with buttons (they are either

on or off at any particular instant), and four analog bits, which are

normally associated with joystick axes (their value is variable at

any particular instant).

In the original game port specifications provided by IBM in the early

1980's, there were four buttons.  These were either separated as two

buttons on each of two separate joysticks, or one button on each of

four separate paddles.  The four analog values could either be

separated as two axes (horizontal & vertical) each on two separate

joysticks, or one axis on each of four separate paddles.  As far as I

know, nobody makes paddles any more (at least I've never seen one).

As time passed, joystick manufacturers figured out ways of

manipulating the digital and analog inputs in ways that IBM hadn't

imagined.  The first thing that joystick manufacturers did was to put

all eight of the inputs on a single joystick, instead of dividing

them among two different joysticks.  This allowed a single joystick

to have as many as four buttons and as many as four axes.  Usually,

the two extra axes are in the form of sliders or gas pedals or

wheels, and do not usually resemble what you normally think of as an

"axis".

A further advancement came when one of the joystick manufacturers

decided to have more than four buttons on a joystick.  There are two

possible ways of using the four bits of digital data associated with

the buttons.  The first way is to have four independent buttons,

where any or all of them can be pressed at the same time (this is the

original use of the bits).  We will call this the 4-button protocol.

The second way allows you to have as many as 15 buttons, but only one

of them can be pressed at a time.  If you do press more than one at

the same time, the joystick will consider one of the buttons to be

"more important" than the other one, and only the most important one

will be sent to the computer.  We will call this the 15-button

protocol.  If you want USBJSTIK to emulate the 15-button protocol,

you would do the following:

USBJSTIK FifteenButtons: Yes

To have USBJSTIK emulate the 4-button protocol (the default), you

would do the following:

USBJSTIK FifteenButtons: No

DOS USB DRIVER INTRODUCTION            Page 124 of 186

The joystick manufacturers also came up with a new way to manipulate

the analog values.  I believe that ThrustMaster was the company that

invented this, so I will call it the ThrustMaster protocol.  What

this protocol involves is taking one of the analog axis values and

"digitizing" it.  Rather than simply having two digital values,

however, it is digitized into five different values.  Each of the

five values is associated with the position of a four-way hat-switch

(centered, up, right, down, and left).  If you want USBJSTIK to

emulate the ThrustMaster protocol, you would do the following:

USBJSTIK ThrustMaster: Yes

To have USBJSTIK emulate the Standard (default) protocol, you would

do the following:

USBJSTIK ThrustMaster: No

USBJSTIK can be set up to emulate two separate joysticks (each with

two buttons), or can be set up to emulate a single joystick.  When

emulating a single joystick, USBJSTIK can emulate the buttons with

either the 4-button or 15-button protocol, and can emulate the axes

with either the Standard protocol or the ThrustMaster protocol.

However, I will admit that configuring USBJSTIK to do exactly what

you want it to do can be confusing, unless you need understand some

basic concepts about how analog and USB joysticks actually work.

Analog Joystick Entity Names

����������������������������

In order to talk about joysticks in more detail, we need to come up

with a way of naming the entities (axes and buttons) that the

joystick has.  Our naming convention will be primarily based on the

idea of having two separate joysticks, each having a single two-

dimensional handle and two buttons.  The first joystick will be

called "A" and the second will be called "B".  The left-right

(horizontal) axis of the handle will be called "X", and the up-down

(vertical) axis of the handle will be called "Y".  The first button

(usually located on the front of the handle that you press with your

index finger) will be called "1", and the second button (usually

located on top of the handle that you press with your thumb) will be

called "2".  Therefore, with two standard joysticks, you will have

the following entities:

DOS USB DRIVER INTRODUCTION            Page 125 of 186

AX = First Joystick, Left-Right Axis

AY = First Joystick, Up-Down Axis

A1 = First Joystick, First Button

A2 = First Joystick, Second Button

BX = Second Joystick, Left-Right Axis

BY = Second Joystick, Up-Down Axis

B1 = Second Joystick, First Button

B2 = Second Joystick, Second Button

If you have only one joystick and it uses the 4-button protocol, it

is essentially "stealing" the B1 and B2 buttons from what would be

the B joystick.  We will call these buttons A3 & A4, but they are

actually synonymous with B1 and B2, respectively.  That is, A3 is

actually the same button as B1, and A4 is the same as B2.

A3 = Single Joystick, Third Button (same button as B3)

A4 = Single Joystick, Fourth Button (same button as B4)

If you have only one joystick and it uses the 15-button protocol, the

additional buttons will simply be called A5-A15.  While we will call

the first four buttons A1-A4 just like we do when we use the 4-button

protocol, they do not act the same way (you can only press one of

them at a time when using the 15-button protocol).

A5  = Single Joystick, Fifth Button

A6  = Single Joystick, Sixth Button

...

A15 = Single Joystick, Fifteenth Button

If you are using a single joystick with the standard axis protocol,

the third and fourth axes are still called BX and BY, even though

they are not on the second (B) joystick.  Usually, the third and

fourth "axes" on a single joystick do not resemble axes in the

traditional sense, but are usually implemented instead as sliders,

gas pedals, or wheels.

BX = Single Joystick, Third Axis

BY = Single Joystick, Fourth Axis

If you are using a single joystick with the ThrustMaster protocol,

the four-way hat-switch is treated as if it is the second (B)

joystick handle (the hat-switch axes are called BX & BY).  The

remaining axis (usually implemented as a slider or gas pedal) is

called AZ.

BX = Single Joystick, Hat-switch Left-Right Axis

BY = Single Joystick, Hat-switch Up-Down Axis

AZ = Single Joystick, Fifth Axis

DOS USB DRIVER INTRODUCTION            Page 126 of 186

USB Joystick Control Names

��������������������������

The section directly above declares a naming convention for the

analog joystick entities.  The purpose of USBJSTIK is to use a USB

joystick (or joysticks) to emulate an analog joystick (or joysticks).

USB joysticks don't work the same way that analog joysticks do, so we

need to come up with a different naming convention.  Calling them

different things will also make it easier to understand when we later

start talking about how to map the USB joystick buttons and axes to

the analog joystick buttons and axes.

The first thing we will do is to call each button or axis on the

joystick a Control, rather than calling it an Entity as we did with

the analog joysticks.  A USB joystick control is mapped to and

controls an analog joystick entity.

USBJSTIK allows you to have as many as 8 USB joysticks attached to

the computer, which we will refer to with 0-based index numbers (0-

7).  The "first" USB joystick (or gamepad) is called joystick 0, and

the "last" is called joystick 7.

Joystick 0 = First USB joystick or gamepad

Joystick 1 = Second USB joystick or gamepad

...

Joystick 7 = Last USB joystick or gamepad

On each USB joystick there will be multiple controls (buttons and

axes).  Almost every USB joystick is made differently than all of the

others, so the numbers and types of controls will vary considerably

by manufacturer and model.  The first type of control are the

Buttons, which will simply be numbered starting from 0.

Button  0 = First Button on USB joystick or gamepad

Button  1 = Second Button on USB joystick or gamepad

...

Button 99 = Hundredth Button on USB joystick or gamepad

In the USB architecture, buttons are simply numbered, and most USB

joysticks have a lot of buttons (although it is very unlikely that

they will have 100).  The numbering of the buttons (which one is

"first" and which one is "last") is determined by the joystick

manufacturer, and the numbering does not always seem to "flow" in a

logical fashion (left to right, for example).  You will usually need

to experiment with each of your joysticks to figure out what the

numbering scheme is.

DOS USB DRIVER INTRODUCTION            Page 127 of 186

The next category of controls are the Direction Pads (DPads) and Hat-

switches (Hats).  These can either be 4-way (up, right, left, down)

or 8-way (up, up and right, right, right and down, etc.).  DPads and

Hats are always spring-loaded, and when you are not pressing them in

a particular direction they will automatically become centered.  In

USBJSTIK, we do not differentiate between DPads and Hats, nor do we

differentiate between 4-ways and 8-ways.  We classify them all as the

same "type" of control.  Like the buttons, they are number starting

from 0.

DPad  0 = First 4-way or 8-way Direction-Pad or Hat-switch

DPad  1 = Second 4-way or 8-way Direction-Pad or Hat-switch

..

DPad 99 = Hundredth 4-way or 8-way Direction-Pad or Hat-switch

Like the buttons, which one is "first" and which one is "last" is

determined by the manufacturer, and you will need to experiment with

each of your joysticks to figure out which is which.

The last three types of control fall under the general classification

of "axes" (which we will also classify as "analog controls", even

though they are not truly analog).  This means that at any particular

instant the value (number) they have can fall anywhere within a large

range (as opposed to buttons and DPads, which are simply on or off).

In the USB architecture, and in USBJSTIK, there are three different

types of analog controls: Axis, Throttle, and Slider.

The difference between the three types of analog controls is what

happens when you let go of them: an Axis is spring-loaded and

automatically centers itself, a Throttle is spring-loaded and

automatically moves itself to one of the extremes (left, right, up,

or down), and a Slider is friction-loaded and stays where it was when

you last let go of it.  Each of these is also numbered starting with

0.

Axis  0 = First Axis

Axis  1 = Second Axis

...

Axis 99 = Hundredth Axis

Throttle  0 = First Throttle

Throttle  1 = Second Throttle

...

Throttle 99 = Hundredth Throttle

Slider  0 = First Slider

Slider  1 = Second Slider

...

Slider 99 = Hundredth Slider

DOS USB DRIVER INTRODUCTION            Page 128 of 186

Joystick Controls Mapping

�������������������������

Now that we know what we will call the different buttons and axes in

the analog and USB world, we can talk about how to "map" the USB

Controls to the Analog Entities to have USBJSTIK do exactly what we

want it to do.  Rather than trying to explain how you do it, lets

just start by using a specific example.  If you want to map the first

button of the first USB joystick (Joystick 0, Button 0) to the first

button of the first analog joystick entity (A1), you would do the

following:

USBJSTIK Map A1 From Joystick 0, Button 0

To map the second slider of the fourth USB joystick (Joystick 3,

Slider 1) to the Y axis of the second analog joystick entity (BY),

you would do the following:

USBJSTIK Map BY From Joystick 3, Slider 1

In general, you will want to map USB analog controls (Axes,

Throttles, and Sliders) to analog axis entities, and USB button

controls to analog button entities.  However, USBJSTIK will also let

you do some "unusual" mappings as well.  For instance, while it would

not make any sense to map a single button to an axis, USBJSTIK will

let map a PAIR of buttons to an Axis.  For example:

USBJSTIK Map AX From Joystick 0, Button 3,4

will map USB joystick 0, buttons 3 & 4 to the AX analog entity.  That

is, if neither button 3 or 4 is pressed, or both buttons are pressed

at the same time, AX will be centered.  If Button 3 is pressed, AX

will be to the left.  If button 4 is pressed, AX will be to the

right.  For a horizontal (X) axis, the first button listed (3 in our

example) corresponds to moving the axis left, and the second button

(4 in our example) corresponds to moving the axis right.  For a

vertical axis (AY or BY), the first button corresponds to moving the

axis up, and the second to moving the axis down.

Mapping USB DPads and Hats is more complicated and difficult to

understand than are buttons and axes.  In USBJSTIK, a USB DPad is

treated as two separate Axes (an X and a Y), even though the two axes

are inextricably tied together.  So, when you are trying to map a

DPad to an analog entity, in addition to the control type (DPad) and

index (say, 3), you must also provide a direction (X or Y).  For

example:

USBJSTIK Map BX From Joystick 3, DPad 0 X

DOS USB DRIVER INTRODUCTION            Page 129 of 186

If you wanted the entire DPad to be used to emulate the entire

joystick B "handle", you would to do both the above line and:

USBJSTIK Map BY From Joystick 3, DPad 0 Y

As should be fairly obvious by now, it can take a lot of confusing

calculating and writing to get everything mapped the way you want it

to be.  If you want to combine several key mappings onto the same

command-line, you can certainly use aliases and shorthand codes to do

what you want (a list of Aliases can be viewed by issuing a "USBJSTIK

Alias").  For instance, the following lines are equivalent to each

other:

USBJSTIK Map AX From Joystick 0 Axis 0 Map AY From Joystick 0 Axis 1

USBJSTIK MAX0A0MAY0A1

While the two lines are equivalent (they accomplish the exact same

thing), when you look at the second line and try to figure out what

it is actually doing you will probably have absolutely NO IDEA.  When

you come back later to try to test or troubleshoot or change

something, you won't even know where to start.  If you're doing any

mapping that is very complicated at all, you will probably want to

put the mappings into a configuration file and use the USBJSTIK

redirection option to load it.  For example, to accomplish the above

example, you would write a configuration file (we'll call it

USBJSTIK.CFG) that looks something like this:

;This is an example configuration file for USBJSTIK

;To load these options, you must do a "USBJSTIK < FileName",

; where FileName is the name of this particular file.

Map AX From Joystick 0, Axis 0   ;AX = First Joystick, First Axis

Map AY From Joystick 0, Axis 1   ;AY = First Joystick, Second Axis

;This is the end of the example configuration file

and then load it as follows (you could do this in your AUTOEXEC.BAT

file if you wanted):

USBJSTIK < USBJSTIK.CFG

DOS USB DRIVER INTRODUCTION            Page 130 of 186

If you do not "override" USBJSTIK with any special mappings, it will

use the following defaults.  This is set up as a single joystick in

which all buttons and all axes come from the same joystick.  This is

the type of setup that would normally be used for a single-player

game.

AX = Joystick 0, Axis 0

AY = Joystick 0, Axis 1

BX = Joystick 0, Axis 2

BY = Joystick 0, Axis 3

AZ = Joystick 0, Axis 4

A1  = Joystick 0, Button 0

A2  = Joystick 0, Button 1

A3  = Joystick 0, Button 2

...

A15 = Joystick 0, Button 14

Special Notes About Controls Mapping

������������������������������������

If an analog entity is mapped from a USB joystick that does not

actually exist (such as Joystick 7 when you only have one USB

joystick), obviously it will be impossible for USBJSTIK to emulate

that particular analog entity.  Note that USBJSTIK does not

actually treat this as an error, however, because of the "plug-

and-play" nature of the architecture.  Just because a particular

joystick does not exist (is not attached) right now does not mean

things will be that way when you actually want USBJSTIK to start

emulating an analog joystick.  In the same vein, mapping an analog

entity from a specific USB joystick control that does not actually

exist (such as Axis 4 on a USB joystick that only has 2 axes) is

not an error, it just means that the analog entity will never be

emulated.

USBJSTIK does not change the default mapping even if the default

controls do not exist.  Therefore, you will usually need to change

the default mapping.  For instance, very few USB joysticks

actually have four axes -- most only have one handle (two axes)

and some combination of Hat-switches or Sliders that you will want

to use as your BX, BY, and AZ entities.  Likewise, USB gamepads

usually do not have any axes at all, but instead have Direction

Pads that are intended to be used as axes.

DOS USB DRIVER INTRODUCTION            Page 131 of 186

Even if every analog entity is mapped from a legitimate USB

joystick control, however, it does not necessarily mean that all

of the analog entities can be emulated.  In addition to the

correct mappings, what happens during the emulation also depends

on which emulation modes USBJSTIK is currently set to (15-button

vs. 4-button, and Standard vs. ThrustMaster).  This is discussed

above in "Joystick Protocols" (page 124).  For instance, the AZ

entity is never emulated unless USBJSTIK is using the ThrustMaster

protocol.

It is possible for you to map the same USB joystick control to two

different analog entities.  For example:

USBJSTIK Map A1 From J1 Btn 0    Map A2 From J1 Btn 0

USBJSTIK does not consider this an error, since there may be a

perfectly legitimate reason for you to do this.  In general,

however, this should be avoided since USBJSTIK will probably not

behave the way you're expecting it to.

USBJSTIK does not provide a direct way to map an analog entity

from "nothing".  If you purposely do not want a specific analog

entity to ever be emulated, you must map it from a USB joystick

control that you know will never actually exist (such as Joystick

7, Button 99).

You can map analog axis entities from any type of USB joystick

control (Axis, Throttle, Slider, DPad half, or Button pair).

However, you can only map analog button entities from USB button

controls.

Remember that analog button entity A3 is the same as B1 (the first

button on joystick B, when you have two joysticks), and A4 is the

same as B2.  The following two lines, for example, are equivalent:

USBJSTIK Map A3 From Joystick 0, Button 6

USBJSTIK Map B1 From Joystick 0, Button 6

To view the current mappings that USBJSTIK is using, you need to

use the Status option switch (discussed in "Miscellaneous Notes"

below, page 138).  In addition to showing you the current

mappings, the Status option switch will also tell you how many of

each kind of control (Buttons, Axes, Throttles, etc.) each USB

joystick has.

While the Status option switch will tell you how many of each kind

of control each USB joystick has, it will not tell you which

control is "first" (index 0), which is "second" (index 1), etc.

To figure out precisely which index is associated with which

control, you will need to use the JoystickStatus option switch,

discussed below in "Displaying Raw joystick Data" (page 137).

DOS USB DRIVER INTRODUCTION            Page 132 of 186

Analog <-> Digital Conversions

������������������������������

A USB analog control (Axis, Throttle, or Slider) produces a

continuously variable range of numbers (small numbers when it is left

or up, large numbers when it is right or down, and mid-range numbers

when it is centered).  However, USBJSTIK allows you to map an analog

control to a Hat-switch axis, which is a digital entity.  The

horizontal axis of a Hat-switch, for example, is either left, center,

or right -- it is not continuously variable nor is it ever "almost

centered".  USBJSTIK needs a special parameter that essentially

answers the question, "Exactly how far away from the center does the

USB analog control need to be before I should consider it un-

centered?".  The parameter that USBJSTIK uses to determine this is

called the PercentFromCenter.

For example, lets say the range of values that a horizontal (X) USB

analog control produces range from 0 (when it is all the way to the

left) to 200 (when it is all the way to the right).  When it is

centered, it will produce a value of 100 (or very close to 100).  If

the USBJSTIK PercentFromCenter is set at 50 (which is the default),

the range where the analog control will be "centered" will go from 50

(100 - 50%) to 150 (100 + 50%).  When the analog entity is far enough

to the left for the value to read less than 50, USBJSTIK will

"trigger" the appropriate entity in the analog joystick emulation

(the analog Hat-switch will be to the left).  When it reads more than

150, the Hat-switch will be right.

You can set USBJSTIK's PercentFromCenter parameter to be anything

from 20 to 80, with the default being 50.  Lower numbers make the

joystick more "touchy", while larger numbers may make the joystick

seem less responsive.  You should adjust this number based on how you

want the joystick to "feel".  To change the PercentFromCenter to 20%,

for example, you would do the following:

USBJSTIK PercentFromCenter: 20

Also note that the ThrustMaster protocol only emulates a 4-way Hat-

switch, not an 8-way Hat-switch.  A 4-way Hat-switch cannot, for

example, be both up and to the left at the same time.  However, when

you are mapping USB controls to the Hat-switch entity, it is quite

possible for you to have the USB controls be two directions (like up

and to the left) at the same time.  When something like this happens,

of course, USBJSTIK does not know which direction you really want it

to emulate, and therefore keeps the Hat-switch entity centered.

DOS USB DRIVER INTRODUCTION            Page 133 of 186

Normalizing Axis Entities

�������������������������

The standard way that USBJSTIK emulates analog joystick entities is

through a set of BIOS functions specifically designed around

joysticks.  The BIOS functions return simple on and off values for

the four button entities, and analog (continuously variable) values

for the four axis entities.  Unfortunately, there is no standardized

or normalized range of numbers that are considered "appropriate" for

the analog values to be.  The numbers are allowed to be anything from

1 to 65535, and the only thing that matters is that the values get

smaller as you move left or up, and get bigger as you move right or

down.  The main reason that you need to "calibrate" your joysticks

every time you start a program that uses them is because there is no

standard range of values.

The default range of values that USBJSTIK returns is 1-100 (1 when

left or up, 100 when right or down, and 50 when centered).  That is,

the output of all analog axis entities are normalized (adjusted) so

that they fall within the 1 to 100 range.  To change USBJSTIK to use

a different range, you simply specify the Center value you want

USBJSTIK to use, which can be any number between 10 and 200.

USBJSTIK always uses 1 as the lower value, and double the center

value as the upper value.  For example, to change the Center value

reported by the BIOS to 100 (setting the range to be 1-200), you

would do the following:

USBJSTIK Center: 100

Nearly all programs that use a joystick have some sort of joystick

calibration routine you run each time you start the program.  With

such programs, there is normally no need to change the Center value

in USBJSTIK, since the program will simply "adjust" itself to

whatever values USBJSTIK is sending.  The only time you should need

to change the Center value is when you are using a program that does

not have a joystick calibration routine, or does not somehow adjust

itself automatically to the joystick values.

Adjusting the Center value affects ALL of the analog axis entities

(AX, AY, BX, BY, and AZ) at the same time.  It is not possible to

have one entity report different values than the other entities.

DOS USB DRIVER INTRODUCTION            Page 134 of 186

I/O Virtualization

������������������

The primary way that USBJSTIK emulates joysticks is through a special

set of functions in the BIOS that are specifically dedicated to

joysticks.  Unfortunately, in the past, computer manufacturers's

support of the BIOS joystick functions has been very bad ("pathetic"

is actually much more descriptive than "very bad").  As a result,

many DOS programs that use joysticks do not use the joystick BIOS

functions at all, but instead interact directly with the joystick

hardware.  This is very unfortunate and short-sighted of the program

authors, since this precludes them from using any of the newer

"digital" joysticks or joysticks that are attached to things like USB

ports.  They have essentially programmed themselves into

obsolescence.

All is not lost, however.  It is possible for USBJSTIK to emulate an

analog joystick at the hardware level, rather than the BIOS level,

using a process called I/O Virtualization.  This allows USBJSTIK to

work with almost any DOS program that knows how to use a joystick,

even if it interacts with the joystick at the hardware level instead

of the BIOS level.  However, USBJSTIK cannot perform I/O

Virtualization by itself.  It needs the help of another program: a

memory manager like later versions of Microsoft's EMM386 or Qualitas'

386MAX.

I can only talk knowledgeably about Microsoft's EMM386, since I have

never owned or used 386MAX.  Also, from my understanding, none of the

other DOS memory managers (QEMM, NetRoom, FreeDOS' EMM386, etc.)

support I/O Virtualization.  You will need EMM386 version 4.46 or

later.  DOS 6.20 came with version 4.48, as did Windows for

Workgroups 3.11.  Versions of EMM386 that came with any versions of

DOS or Windows previous to those (including "basic" Windows 3.11) do

not support I/O Virtualization, and USBJSTIK will only be able to

emulate analog joysticks using the BIOS in such cases.  I also

suggest that you use either the NOEMS or FRAME=NONE option with

EMM386 to disable the Expanded Memory page frame, unless for some

reason you actually need Expanded Memory.  If you don't actually need

EMS, it just unnecessarily wastes memory (a LOT of memory).

You do not need to do anything to USBJSTIK to "turn on" I/O

Virtualization.  USBJSTIK will automatically enable I/O

Virtualization if it detects a compatible memory manager.  If you are

unsure whether or not USBJSTIK is able to Virtualize I/O or not, you

can use the Status option switch (discussed below in "Miscellaneous

Notes" on page 138).

DOS USB DRIVER INTRODUCTION            Page 135 of 186

Although you can't turn it on or off, there is one option switch that

does directly affect how I/O Virtualization works.  In the PC, there

is a range of 16 total I/O ports that are "reserved" for use with

game ports (joysticks): 200h - 20Fh.  A game port only requires one

port (not 16), and the default one that all computers use is 201h.

USBJSTIK will let you select any valid I/O port (between 201h and

20Fh) for the I/O Virtualization.  For example, to change the I/O

Port to 209h, you would do the following:

USBJSTIK IOPort: 209h

There are VERY few DOS programs that will let you use any port other

than 201h, so it is rarely a good idea to change it from the default

of 201h.  However, by changing it, it is possible for you to have a

regular (analog) joystick attached to your game port (using I/O Port

201h), and a USB joystick emulating a joystick on some other I/O Port

at the same time.

There is also an option switch that indirectly affects how I/O

Virtualization works.  When emulating an analog joystick using the

BIOS, the Center option (discussed above in "Normalizing Axis

Entities, on page 134) controls the axis values that are returned by

the BIOS.  When emulating an analog joystick using I/O

Virtualization, the axis values as returned by the BIOS are

"converted" into a duration of time.  The larger the Center value,

the longer it take for USBJSTIK to process the I/O Virtualization.

The details are too complicated too discuss here, and are unimportant

in most situations.  Just note that the Center value does affect how

I/O Virtualization works, and changing it (either up or down) may

help USBJSTIK work better (or worse).

Descriptors

�����������

To display the General Descriptors (Device, Configuration, Interface,

etc.) of the last joystick or gamepad that was configured by

USBJSTIK, you would do the following:

USBJSTIK Descriptors

A USB joystick is also part of a larger USB classification called

Human Interface Devices (HID's), and therefore has a special

Descriptor called an HID Report Descriptor.  The HID Report

Descriptor "describes", among other things, how many buttons and axes

and DPads the joystick actually has.  To display the HID Report

Descriptor of the last joystick that was configured by USBJSTIK, you

would to the following:

USBJSTIK ReportDescriptor

DOS USB DRIVER INTRODUCTION            Page 136 of 186

Displaying Raw Joystick Data

����������������������������

USBJSTIK will also let you look at the Raw (USB) data as it is coming

from a joystick.  This is virtually a necessity when you are first

setting up the mapping options for a joystick, since it is usually

the only way you can figure out which controls (buttons, axes,

sliders, etc.) are which.  This will also help you to troubleshoot

joysticks that you are having problems with, or it can just be fun to

experiment to see how different manufacturers do things with their

joysticks.

Because there is so much variation in how different joysticks and

gamepads are made, USBJSTIK will not let you view the Raw Data for

all of the USB joysticks at the same time (assuming you have more

than one).  Rather, you must provide USBJSTIK with a joystick index

number (between 0 and 7).  For example, to view the raw data for the

first joystick (index 0), you would do the following:

USBJSTIK RawData: 0

Exactly what the display looks like will depend on the exact

characteristics of the joystick (how many Buttons, Direction-Pads or

Hat-switches, Axes, Throttles, or Sliders it has).  The "raw" data

displayed for each control will be the data as it is coming directly

from the joystick.  Buttons are either on (pressed) or off

(released), while Axes, Throttles, and Sliders will vary as you move

them (usually, small numbers if you move left or up or counter-

clockwise, and large numbers if you move right or down or clock-

wise).  Direction-Pads and Hat-switches will usually display 0 if

they are centered, 1 if up, 2 if up and right, 3 if right, ..., 7 if

left, and 8 if up and left.

Keep in mind that the display and the numbers that appear in this raw

data format will probably not look anything at all like the data you

see from an regular analog joystick.  USBJSTIK takes the raw USB

data, and then filters, maps, and normalizes it to be able to emulate

an analog joystick.  This process will completely transform the raw

data so that it is no longer recognizable.

When you are through viewing the raw data, you simply press any key

on the keyboard and USBJSTIK will stop.

DOS USB DRIVER INTRODUCTION            Page 137 of 186

Miscellaneous Notes

�������������������

To have USBJSTIK show you the current status of the USB joysticks and

gamepads that are currently attached to the computer, you would to

the following:

USBJSTIK Status

The upper left portion of the display shows "general" information

about the USBJSTIK program, such as which protocols are currently

being used (4-button vs. 15-button, Standard vs. ThrustMaster) and

which (if any) I/O Port is currently being Virtualized.  The upper

right portion of the display shows details about the individual

joysticks that are attached, including the quantity of each type of

control (Axes, Throttles, etc.).  The bottom of the display shows a

complete listing of which analog joystick entities are being mapped

from which USB joystick controls.  The Status option switch tells you

pretty much everything you need to know about how USBJSTIK is

currently configured, and should generally be the first place you

look when you are troubleshooting something.

I'll admit that setting up the mapping from USB controls to analog

entities can be very confusing, at least at first.  Once you get it

figured out the first time, you should "automate" it by setting up

appropriate configuration files, batch files, environment variables,

modifying AUTOEXEC.BAT, etc., as needed.  Quite frankly, you're crazy

if you don't.  This stuff is just way too confusing, and you won't

remember what you did ten minutes after you did it, much less weeks

or months later.  You need to automate it as much as you can, with

lots of comments in the batch and configuration files.

I've tried to come up with a way to make the mapping more "automatic"

within USBJSTIK, but was unable to come up with an acceptable method.

You (the user) still needed some control over exactly how the

"automation" worked, and this ultimately (at least in my mind) became

more confusing, and was definitely less flexible, than the way the

mapping options work now.

I've seen several joysticks that declare themselves (in their

descriptors) to have more controls (Axes, Buttons, etc.) than they

really have.  Obviously, it does no good to map a control that does

not actually exist to an analog entity, so you should adjust your

mapping options accordingly if you have a joystick that does this.

DOS USB DRIVER INTRODUCTION            Page 138 of 186

Some buttons are actually "mode" buttons; that is, when you press the

button, the joystick or gamepad will change modes and start behaving

differently (usually, different axes, buttons, DPads, etc. become

enabled or disabled).  Mode buttons are more likely to appear on

gamepads than joysticks, though they can be on either.  Also, some

mode buttons appear as "regular" buttons and their status is reported

just like any other button.  Other times, the mode button is "hidden"

and is not reported through the USB architecture.  If your particular

USB joystick has a mode button, you will usually NOT want to use it

as a control that is mapped to an analog entity.  Just make sure you

set up the mapping in USBJSTIK so the joystick responds exactly like

you want it to.

There is no universal, extended joystick BIOS interface.  It would be

really nice if there was one, which would include all kinds of input

devices (everything from simple paddles and joysticks to full body

suits for virtual reality simulations), as well as all kinds of

output devices (various types of "force feedback").  I've actually

considered starting to create such an extended BIOS joystick

interface, but decided against it.  There have been a few half-

hearted attempts in the past (such as Gravis' GRIP), but none of them

were ever comprehensive enough or non-proprietary enough to be very

useful.  Even if an extended BIOS interface were developed, though,

there would probably never be a DOS program written that would take

advantage of it.  We will probably be forever stuck using the limited

analog joystick interface (four buttons, four axes, along with

various the various twists and permutations) that exists today.

THRUST

������

Version:     1.16

Prog Type:   Test

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete

Environment: None

Redirection: No

Purpose:     Joystick tester

TSR Memory:  0 kB (not a TSR)

THRUST is a program designed to test your joysticks.  It does this by

displaying the low-level, "raw" data that is coming from the joysticks,

either through the joystick BIOS (software) or directly from the

joystick (game port) hardware.  It's included here as a "companion"

program to help you test USBJSTIK (discussed directly above on page

123), but you can also use it to simply test your analog joysticks.

DOS USB DRIVER INTRODUCTION            Page 139 of 186

What you will see on the screen when you run THRUST is divided into

three sections.  At the left will be a description of how THRUST is

currently interacting with the joysticks: using the joystick BIOS

(software), or by directly using the hardware (in which case USBJSTIK

will show you an I/O port number).

The second section will show you the raw data (numbers) being returned

by each of the four axes (AX, AY, BX, and BY).  For each particular

axis, if you move the appropriate control (handle or direction-pad or

slider or whatever), the displayed number will change.  Usually, moving

the control left or up or counter-clockwise will make the number get

smaller, and moving it right or up or clockwise will make the number get

bigger.  However, if the joystick is using the ThrustMaster protocol

(described above in "Joystick Protocols" on page 124), the last (BY)

axis is displaying the status of the (digital) 4-way Hat-switch.  In

this case, the number in BY will be maximum when the Hat-switch is

centered, a little lower when up, a little lower yet when right, still

lower when down, and minimum when left.

The third and last section of THRUST will display the status of the

joystick buttons, whether the joystick is using the 4-button or 15-

button protocol (see "Joystick Protocols" above on page 124 if you don't

know what this means).  If the joystick is using the 4-button protocol,

the A1, A2, B1, and B2 columns will display the status of the four

buttons (whether they are each pressed or not).  If the joystick is

using the 15-button protocol, the column labeled "15" will display the

number of the button that is currently pressed (0 = no Button currently

pressed, 1 = Button #1 pressed, etc.).

THRUST can interact with the joysticks in two different ways, either

through the BIOS (software) or directly through the hardware.  The

default method that THRUST uses depends on whether or not you have a

digital joystick driver (like my SDWRGMPD or USBJSTIK programs)

installed in memory or not.  If you do have such a program, THRUST will

use the BIOS (software) method by default.  If you do not have such a

program installed, THRUST will use the direct hardware method by

default.

No matter which access method THRUST would use by default, you can force

it to use whichever method you want.  To force THRUST to use the BIOS

method, you would do the following:

THRUST B

DOS USB DRIVER INTRODUCTION            Page 140 of 186

To force THRUST to use the direct hardware method, you would do the

following:

THRUST D #

where the # (the I/O port number) is optional.  The I/O port number can

be any number between 200h and 20Fh, which is the range of I/O port

numbers reserved for game ports.  The default I/O port number that game

ports use is 201h, which is what THRUST will use if you don't specify a

number (if you simply do "THRUST D").  Virtually all computers use port

201h, so there is rarely a reason to specify any other port.  In

addition to the "D" (Direct), you can also use the alias "P" (Port) or

"I" (I/O port) to tell THRUST to use the direct hardware method.

After you are through viewing the raw joystick data, simply press a key

on the keyboard and THRUST will stop.

USBDRIVE

��������

Version:     0.18

Prog Type:   Device/Interface Driver

Source Code: ASM (A386 by Eric Isaacson)

Status:      In progress, several major items to address

Environment: USBDRIVE

Redirection: Yes

Purpose:     Driver for up to 8 USB Disk Drives

TSR Memory:  15.5 - 29.6 kB, depending on configuration

USBDRIVE is a Device/Interface Driver program for up to eight USB disk

drives.  Unlike other DOS USB disk driver programs, USBDRIVE is

installed as a TSR rather than a device driver in CONFIG.SYS.  That

means that you can load it and unload from memory at any time without

needing to reboot the computer.  This allows you to set up USBDRIVE so

that it does not use any memory except when you actually need access to

the USB disks.  Other DOS USB disk drive programs are installed as DOS

device drivers via your CONFIG.SYS file.  That means that in order to

"remove" the USB drives or add new ones, you need to edit your

CONFIG.SYS file and reboot the computer.  Admittedly, some of the other

disk drive programs work in certain situations when USBDRIVE does not

(at least in its current state), but not needing to reboot can save you

a lot of time and inconvenience.

USBDRIVE currently supports USB flash drives, media card readers,

external hard drives, and floppy drives.  It does not currently support

USB CD-ROM's, DVD's, etc.  Future upgrades to USBDRIVE, or additional

programs separate from USBDRIVE, will add those capabilities.

DOS USB DRIVER INTRODUCTION            Page 141 of 186

You can adjust the amount of memory that USBDRIVE uses by setting the

number of drives that you want USBDRIVE to be able to access at the same

time.  There are three separate "layers" of complexity that it takes in

DOS before you actually get to the point where you have a drive letter

(like C:) that you can actually use.  You need to tell USBDRIVE how many

"slots" in each level of complexity you want it to "reserve" for you to

insert USB disks.  This is complicated, and is explained in the next

section, "Devices, LUNs, and Drive Letters" (page 142).

Before USBDRIVE can be installed into memory, you must first have at

least one USB Host Driver (USBUHCI{L}) installed in memory.  To install

USBDRIVE into memory using the default settings, you can simply run it

with no option switches:

USBDRIVE

However, you will usually want to be able to control the amount of

memory that USBDRIVE uses, and therefore will usually want to install

USBDRIVE into memory with some option switches (at least /Devices:#,

/Disks:#, and /Drives:#).  This is discussed in the next section.

Devices, LUNs, and Drive Letters

��������������������������������

Installing new drives after DOS has already booted is a pretty

complicated process (much more complicated, than say, installing a

mouse or a joystick).  It's so complicated, in fact, that most other

DOS USB drivers don't even attempt to do it -- they make you load the

drivers in your CONFIG.SYS file instead of as a TSR.  There are

several "layers" to DOS drives that you should understand as you're

working with USBDRIVE.  If you don't understand these layers, when

things don't work like they're supposed to (and they rarely do when

you're dealing with USB), you won't be able to troubleshoot anything.

The DOS drive letter (e.g., C:) is simply the top-most layer for

accessing the disk, though admittedly it is the only one most people

are used to seeing at all.

DOS USB DRIVER INTRODUCTION            Page 142 of 186

Device/Interfaces (Devices)

���������������������������

The lowest layer for disks is the physical hardware interface

(bus) that the device uses to "talk" to the computer.  Types of

hardware interfaces include MFM & RLL (on really old computers),

SCSI, ATA/IDE, SATA, and, of course, USB.  You can usually have

more than one device attached to a single bus, and they don't

necessarily need to all be disk drives (also known as "mass

storage devices").  In the USB world, this is the

Device/Interface, and each USB Device/Interface will have a USB

Host Index, Device Address, and Interface number associated with

it.  In USB, a single Device/Interface is usually the physical

"thing" that plugs into a single port on a USB Hub.

You control the maximum number of USB Device/Interfaces that

USBDRIVE can manage at one time by using the /Devices:# option.

This option must be entered when USBDRIVE is first installed into

memory.  After USBDRIVE has been installed, the only way to change

the maximum number of Device/Interfaces is to uninstall USBDRIVE

from memory and then re-install it with a different /Drives:#

option.

The # in the /Drives:# option switch is the maximum number of

Device/Interfaces that you want USBDRIVE to be able to control at

one time.  This must be a number between 1 and 8, with the default

being 4.  For example, to set the maximum number of

Device/Interfaces to 2, you would install USBDRIVE as follows:

USBDRIVE Devices:2

Each additional Device/Interface adds 76 bytes to the total memory

requirements of USB (the amount of RAM used by the TSR), so this

should be set to the minimum number you think you might need.  For

instance, if you want to be able to copy or move data between two

different USB disks, you probably will want this set at 2 (or

perhaps even more).  If you only plug in one USB disk at a time

(for example, if you only have one available hub port, or you only

have one available flash drive, or you never transfer data

directly between USB drives), you will probably want to set the

maximum number of Device/Interfaces to 1.

DOS USB DRIVER INTRODUCTION            Page 143 of 186

LUNs (Disks)

������������

In USB, each Device/Interface can have multiple devices "inside"

it.  These are called Logical Unit Numbers (LUNs).  Flash drives,

external hard drives, and floppy drives usually only have one LUN

(Disk) per Device/Interface, but there are some kinds of

Device/Interfaces that have more than one LUN (like the media card

readers designed to read all of the different memory cards

commonly used in cameras).  In USBDRIVE, each LUN is assigned a

physical disk number (also called an "INT 13h Disk Number").  This

corresponds to a physical disk number that is assigned to the

"normal" internal disks your computer has (IDE/ATA, floppy, etc.).

Floppy disks are normally assigned physical disk numbers 00h and

01h, while hard drives are assigned numbers starting at 80h (80h

is the first hard drive, 81h the second, etc.).  Note that this is

NOT the same as a drive letter.  Physical disk 00h usually

corresponds to A:, 01h usually corresponds to B:, and 80h usually

contains C:, though that is not always true.  Assuming this is

true is where most people get confused and run into problems.

You control the maximum number of Disks (LUNs) that USBDRIVE can

manage at one time by using the /Disks:# option.  This option must

be entered when USBDRIVE is first installed into memory.  After

USBDRIVE has been installed, the only way to change the maximum

number of Disks is to uninstall USBDRIVE from memory and then re-

install it with a different /Disks:# option.

The # in the /Disks:# option switch is the maximum number of Disks

(LUNs) that you want USBDRIVE to be able to control at one time.

This must be a number between 1 and 16, with the default being 8.

For example, to set the maximum number of Disks (LUNs) to 5, you

would install USBDRIVE as follows:

USBDRIVE Disks:5

Note that a maximum number of Disks set to 5 would allow you to

control a regular flash drive (which usually has only one LUN) and

a media card reader (which usually has 4 physical slots and,

consequently, 4 LUNs) at the same time.

Each additional Disk adds 202 bytes to the total memory

requirements of USBDRIVE (the amount of RAM used by the TSR), so

this should be set to the minimum number you think you might need.

If you don't have a media card reader (which will usually have 4

LUNs/Disks per Device/Interface), you will probably want to set

the maximum number of Disks and the maximum number of Devices

(discussed in the section directly above this one) to the same

value.

DOS USB DRIVER INTRODUCTION            Page 144 of 186

Drive Letters (Drives)

����������������������

Each physical disk can be further divided into multiple

partitions.  Floppy disks are not divided into partitions, while

hard disks many times are.  Microsoft Windows usually tries to set

up the entire hard drive as one gigantic partition, even if you

don't want it to.  If you have multiple operating systems on the

same computer, you will almost always have multiple partitions.

If there are multiple partitions on a disk, DOS assigns drive

letters to the ones that are the correct type (FAT12, FAT16, and

FAT32 if the DOS is new enough), assuming the LASTDRIVE setting in

CONFIG.SYS is large enough to handle all of the Drives.

There is a maximum number of Drive Letters that can be assigned by

DOS, which is determined when DOS is booted.  Once DOS has booted,

it is impossible to change the number of available Drive Letters.

You control the number of available Drive letters with the

LASTDRIVE setting in CONFIG.SYS.  DOS itself requires 88 bytes for

each Drive Letter, so the amount of memory DOS uses is directly

affected by the LASTDRIVE setting.

USBDRIVE also has a /Drives:# option that lets you control how

many drive letters you want USBDRIVE to assign and manage.  Note,

however, that no matter what you set the /Drives:# value to, if

the LASTDRIVE setting in CONFIG.SYS is too small, it doesn't

matter.  In addition to the disks that DOS automatically assigns

Drive Letters to while it is booting, you can also consume drive

letters after DOS has booted with DOS-provided utilities like

ASSIGN and SUBST, CD-ROM & DVD utilities like MSCDEX, RAMDisk

utilities, and networking software like Novell Netware, LANtastic,

etc.  In short, it is a combination of LASTDRIVE, other software,

AND the /Drives:# option that determines how many drive letters

USBDRIVE can actually assign and manage.

Also note that you don't necessarily need USBDRIVE to assign a

drive letter at all to a disk partition in order for USBDRIVE to

be useful to you.  USBDRIVE also allows you to use physical-level

utilities like FDISK and various formatting programs to view and

modify USB disks, which don't work at the DOS level and therefore

do not need any DOS drive letters to be assigned.  USBDRIVE will

not let you select 0 as the number of drive letters it will assign

and manage, however, even if you don t actually need it to assign

any (because you will only be accessing a disk at the physical

level).

DOS USB DRIVER INTRODUCTION            Page 145 of 186

You control the maximum number of Drive Letters that USBDRIVE can

manage at one time by using the /Drives:# option.  This option

must be entered when USBDRIVE is first installed into memory.

After USBDRIVE has been installed, the only way to change the

maximum number of Drives is to uninstall USBDRIVE from memory and

then re-install it with a different /Drives:# option.

The # in the /Drives:# option switch is the maximum number of

Drive Letters that you want USBDRIVE to be able to control at one

time.  This must be a number between 1 and 16, with the default

being 8.  For example, to set the maximum number of Drives to 2,

you would install USBDRIVE as follows:

USBDRIVE Drives:2

Each additional Drive adds 192 bytes to the total memory

requirements of USBDRIVE (the amount of RAM used by the TSR), so

this should be set to the minimum number you think you might need.

For example, if you want to be able to copy or move data between

two different USB devices, or between two partitions on the same

disk, you probably will want this set at 2 (or perhaps even more).

If you only plug in one USB disk at a time, and it only has one

partition on it, you will probably want to set the maximum number

of Device/Interfaces to 1.

Viewing USBDRIVE Status

�����������������������

USBDRIVE allows you to view the details of the USB devices it is

controlling (capacity, FAT type, etc.) in two different ways.  The

first way is with the Status option, as follows:

USBDRIVE Status

This will show you the status of the USB devices from the perspective

of USBDRIVE.  That is, it will be sorted first by Device/Interface,

then by Disk/LUN (possibly with multiple Disks per Device), and

finally by Partition/Drive Letter (possibly with multiple Drive

Letters per Disk).  Device/Interface details include Host Index,

Device Address, and Interface Number.  Disk/LUN details include

Cylinder/Head/Sector totals for the disk.  Partition/Drive Letter

details include Drive Letters and Partition Capacities.

The second way for USBDRIVE to show you the details is with the

DriveStatus option, as follows:

USBDRIVE DriveStatus

DOS USB DRIVER INTRODUCTION            Page 146 of 186

This will show you the status of the USB drives from the perspective

of DOS Drive Letters.  This will show you the DOS Drive Letter,

followed by the physical disk number that the drive is on, followed

by the partition information associated with the drive letter

(including FAT type), followed by the capacity information.  Some of

the information contained in the /DriveStatus option is the same as

that contained in the /Status option, but they look very different

because of their perspective (low-level to high-level vs. high-level

to low-level).

The DRIVES program discussed below (page 164) shows you information

about ALL of the drive letters in DOS, not just the ones that

USBDRIVE is managing.  It is interesting (at least to me) to see how

DOS views disk drives, since it is not at all what I originally

expected it to be.  For instance, until DOS actually accesses (reads

from) a drive, DOS doesn't know the capacity of the drive, and DOS

doesn't access a drive until it actually needs to for some reason.

Therefore, what DOS "thinks" about a particular drive is usually

wrong until you force it to think correctly.

Assigning Drive Letters

�������������������������

Current Directory Structure (CDS)

���������������������������������

There is a table in DOS, called the Current Directory Structure

(CDS), which keeps track of the DOS drive letters.  The number of

entries in the table is determined by the LASTDRIVE setting in

your CONFIG.SYS file, and cannot be changed once the computer has

booted up.  During the boot process, DOS fills in the CDS with

information about your physical disks (floppies and hard drives)

and any special device drivers (like RAM disks) you have in your

CONFIG.SYS file.  If there are more drive letters available than

there are physical disks, some of the CDS entries are simply left

blank.  After DOS has booted, the available drive letters can be

used for other things (network drives, CD-ROMs, SUBST'd or

ASSIGN'd drives, etc.).

DOS USB DRIVER INTRODUCTION            Page 147 of 186

As USBDRIVE is installing itself into memory, it "reserves" some

of the drive letters in the CDS.  The number of drive letters it

reserves is determined by a combination of the /Drives:# option

and the number of available (empty) CDS entries.  That is, the

number of CDS entries it reserves may be less than what is in the

/Drives:# option if there aren't enough empty entries available.

Remember that when USBDRIVE is first installed into memory, it is

not actually controlling any USB drives at all -- that happens

later (USBDRIVE searches for and manages USB drives as a

background process).  As USB devices are plugged and unplugged

from the bus, USBDRIVE assigns and un-assigns the drive letters

that it reserved upon installation.

By default, the process that USBDRIVE uses to determine which

drive letter(s) to reserve is as follows.  USBDRIVE starts with

the third entry in the CDS table (the one associated with the C:

drive letter).  If it is unavailable (already used), it goes to

the next one (the D: drive letter).  When it finds an available

(unused) CDS entry, it "reserves" it.  USBDRIVE keeps searching

until it reaches the end of the table (corresponding to the

LASTDRIVE setting in CONFIG.SYS), or until the number of requested

drives (in the /Drives:# option) has been assigned.

USBDRIVE does not give you complete control over which drive

letters get assigned to the USB drives.  However, USBDRIVE does

have an option that gives you some control.  The /DriveLetter:#

option allows you to control where in the CDS table that USBDRIVE

will start searching.  That is, it allows you to tell USBDRIVE to

start searching somewhere other than the C: drive letter entry.

For example:

USBDRIVE DriveLetter:G

will cause USBDRIVE to start searching at the seventh CDS entry

(associated with the G: drive) instead of the third (the C: drive

letter).  When USBDRIVE reaches the end of the CDS table, it will

"wrap around" back to the C: drive again and continue searching.

You must provide a letter between C and Z to the /DriveLetter:#

option.  USBDRIVE will never assign USB devices to the A: or B:

(floppy) drive letters.  Assigning an A: or B: (floppy) drive

letter to a disk that is not actually a floppy can cause all sorts

of undesirable behavior in DOS, but assigning a hard disk drive

letter (C:-Z:) to a floppy drive is usually not a problem.

DOS USB DRIVER INTRODUCTION            Page 148 of 186

In addition to the /DriveLetter:# option, there is another way you

can gain additional control over which drive letters are reserved

by USBDRIVE.  You can use the DOS ASSIGN or SUBST utilities to

temporarily make certain drive letters unavailable while USBDRIVE

is being installed, and then make the drive letters available

again after USBDRIVE is done.  For example, if you do not want

USBDRIVE to "reserve" drives D: and E: for some reason, you could

do the following (you would probably want to automate this process

with a batch file):

SUBST D: C:\    Reserve D: with SUBST

SUBST E: C:\    Reserve E: with SUBST

USBDRIVE        Let USBDRIVE start at F:

SUBST E: /D     Restore E: to normal/unused

SUBST D: /D     Restore D: to normal/unused

In addition, after USBDRIVE has reserved its drive letters, you

can use ASSIGN or SUBST on the USB drives to create "alias" drive

letters for the various USB drives and directories.  Incidentally,

this points out another advantage of using TSR's (after DOS has

booted) rather than using DOS device drivers (CONFIG.SYS).  You

have no control whatsoever over the drive letters that get

assigned for drivers that are loaded via CONFIG.SYS (DOS assigns

them automatically).

FAT32 Partitions

����������������

USBDRIVE assigns drive letters to every partition it finds on a

disk that is designated as FAT12 or FAT16.  In addition, USBDRIVE

will assign drive letters to partitions designated as FAT32 if it

believes the version of DOS you are using can support FAT32

drives.  The first version of Microsoft DOS that supported FAT32

was 7.10 (DOS 95 OSR 2), so USBDRIVE assumes that your computer

will support FAT32 if it claims to be DOS version 7.10 or later.

Note that non-Microsoft versions of DOS do NOT report their actual

version numbers, but rather return the "closest" Microsoft version

in terms of compatibility.  For example, DR-DOS 7.03 claims to be

DOS version 6.00, and FreeDOS 1.0 claims to be DOS version 7.10.

There is a switch option to override USBDRIVE's default behavior

regarding FAT32 drives.  If you want USBDRIVE to assign drive

letters to USB partitions that are FAT32, no matter what version

your DOS claims to be, you would do the following:

USBDRIVE FAT32: Yes

DOS USB DRIVER INTRODUCTION            Page 149 of 186

Keep in mind that if you allow USBDRIVE to assign drive letters to

FAT32 drives when your DOS doesn't support them, and you try to

read something from the drive, you will probably see a bunch of

garbage that doesn't make any sense at all.  If you try to write

to the drive, you will probably screw it up (possibly beyond

repair).

If you do NOT want USBDRIVE to assign drive letters to FAT32

drives, even if your DOS claims to be version 7.10 or later, you

would do the following:

USBDRIVE FAT32: No

Legacy Support for Disks

������������������������

There are also potential issues with drive letters if the USB

drives are already configured using Legacy Support (in the BIOS).

This can be a particularly serious problem if the computer was

booted from a USB drive, or if the current drive (in DOS) is a USB

drive.

When the Host Controller Driver (USBUHCI{L}) takes control of a

bus, all of the Device/Interfaces currently attached to the bus

are reset and stop working (at least temporarily).  There is no

mechanism to ensure a "smooth" transition from the BIOS (Legacy

Support) drivers to the Operating System drivers for disk drives.

All of the entries in the Current Directory Structure that the

Legacy Support drivers were managing instantly become invalid when

the Operating System host driver (like USBUHCI{L}) starts working,

and the computer will probably crash if you try to access those

particular drive letters.  After USBDRIVE begins to manage the

disk Device/Interfaces, they will be assigned new drive letters,

which will be completely different than the original drive letters

that were assigned during the boot process.  After all is said and

done, you can end up with an ugly, complicated mess on your hands.

Things may get even worse if you decide to remove USBDRIVE and

USBUHCI{L} from memory and try to revert back to Legacy Support.

I personally do not boot from USB drives, nor do I use Legacy

Support for disk drives.  Therefore, at least in this version of

USBDRIVE, I have not tried to figure out a solution to the Legacy

Support issues.  I suspect there might be a "smooth" way to

perform the transition from Legacy Support to Operating System

support for disk drives, but the implementation will need to wait

for awhile.  In the meantime, you'll just need to be very careful

if you use Legacy Support for USB Disk Drives.

DOS USB DRIVER INTRODUCTION            Page 150 of 186

Assigning Physical Disk Numbers

�������������������������������

In a manner similar to the DOS drive letters discussed in the section

directly above ("Assigning Drive Letters"), DOS also has a mechanism

to keep track of physical disk numbers.  While it is installing,

USBDRIVE "reserves" physical disk numbers that it will assign and un-

assign to the Logical Unit Numbers (LUN's) of the USB Devices that

get plugged and unplugged.  The quantity of physical disk numbers

that USBDRIVE reserves is equal to the /Drives:# option switch

provided during installation.  Unlike the number of Drive Letters

which are limited by the LASTDRIVE setting in CONFIG.SYS, there

should always be enough available physical disk numbers for USBDISK

to use.

Note that you usually do not "see" the physical disk numbers in daily

operations with DOS, even though they are there lurking in the

background and are very important.  You usually only see the physical

disk numbers when you are working with low-level formatting and

partitioning utilities like FDISK.

When USBDRIVE "reserves" a physical disk number, it essentially

configures it as a "removable hard disk" (with physical disk number

starting at 80h), not as a floppy disk.  Unfortunately, Microsoft's

FDISK program has a serious bug in it, and doesn't work correctly

with "reserved" physical disk numbers (removable hard disks), even

though removable hard disks have been around since the early 1980's.

As Microsoft's FDISK is searching for all of the hard disks, as soon

as it comes across a "reserved" disk with no media, it immediately

crashes and terminates.  It won't even let you look at or configure

the disks that it considers "legitimate".

There are two ways around this problem with MS FDISK.  The first is

to always make certain the you have media inserted (USB Devices) for

all possible physical disk numbers (corresponding to the /Disks:#

option switch in USBDRIVE) before running MS FDISK.  If the removable

hard drive has media inserted, MS FDISK works OK.  The second way

around this problem is to not use MS FDISK, but to use another disk

partitioning program instead.  The FDISK program that comes with

FreeDOS, Ranish Partition Manager, and possibly other disk

partitioning programs do not have this problem.  I personally

recommend using Ranish Partition Manager (available at

http://www.ranish.com) for reasons described below in "Partitioning &

Formatting" (page 152).  I find it much easier to use than any of the

FDISK programs.

DOS USB DRIVER INTRODUCTION            Page 151 of 186

SCSI Inquiry Responses

����������������������

Nearly all USB disks use what is called the SCSI Transparent

Protocol.  This means that even though at the hardware level the disk

attached to a USB bus rather than a SCSI bus, at the software level

the disk responds to the SCSI command set.  Unfortunately, even

though all USB disks claim to be compatible with the SCSI command

set, many of them are only partially compatible.  Some of them do not

respond appropriately to all of the SCSI commands like they're

supposed to.  This "partial compatibility" is the main reason that

USBDRIVE, and other USB disk drive programs, do not work correctly

with all of the different disks they come in contact with.  If all of

the disk manufacturers truly followed the standards, the world would

be a much better place.

One of the SCSI commands that all USB disks are required to respond

to is called an Inquiry Request.  The data returned from the Inquiry

Request includes the manufacturer (vendor), model (product), and

version of the drive.  This information is both interesting and

useful.  It is interesting because many times it is not what you

expect it to be.  It is useful in situations where you have several

different USB disks on the computer at the same time, and need help

figuring out which one is which when you get confused.  To display

the Inquiry Response data for all of the attached USB disks, you

would do the following:

USBDRIVE InquiryData

Partitioning & Formatting

�������������������������

USBDRIVE should be compatible with most disk management and

manipulation programs you want to use with it.  There are only two

programs that I know of that do not work with USBDRIVE, though there

could possibly be others.

The first program I know of that does not work correctly is

Microsoft's FDISK partition management program (the one that comes

with MS-DOS).  As discussed above in "Assigning Physical Disk

Numbers" (page 151), MS FDISK does not work correctly (crashes) with

removable hard disks that currently have no media (like a USB Device

that is currently unplugged).  The other partition management

programs that I have used, FreeDOS's FDISK and Ranish Partition

Manager, do not have this problem.

DOS USB DRIVER INTRODUCTION            Page 152 of 186

The second program that does not work correctly is SFORMAT (Safe

Format) that comes with the Norton Utilities for DOS.  SFORMAT is

specifically designed to format floppy drives (not hard drives).

However, if you use USBDRIVE to mount a USB floppy drive in DOS, you

cannot use SFORMAT to format the floppy drive.  I'm not sure exactly

what it is that SFORMAT doesn't like about the USB floppies, but

suspect it has something to do with the fact that USBDRIVE sets up a

USB floppy drive to look like a removable hard disk rather than a

true floppy drive.  In any event, the MS-DOS FORMAT utility works

just fine on USB floppy drives.

One additional problem I have seen when trying to format USB floppy

drives is that you may not be able to change the size of the disk.

That is, you may not be able to re-format a floppy disk that was

originally 720kB into one that is 1.44MB, or vice versa.  At least I

am not able to do it on my computers using the USB floppy drive I

have that is manufactured by TEAC.  I don't know for sure if this is

a problem with USBDRIVE, USB floppy drives in general, with TEAC, or

just with the particular model of TEAC drive that I have.  I can tell

you, however, that I am not able to re-format the disk to a different

size in MS-Windows, either, so suspect the problem is in the USB

floppy drive itself rather than with USBDRIVE.

After you manipulate a USB disk at a low level (messing with

partitions or formatting), you should reset the disk to force USBDISK

to re-read the contents.  You can either unplug and replug the disk,

issue a USB software reset, uninstall and re-install USBDRIVE, or

reboot the computer.  If you don't reset the disk, there's a

potential for you to really screw up the data that is stored on the

disk.  Sometimes USBDRIVE can recognize that the disk has changed,

and sometimes it can't.  It's always safest to Reset.  Note that with

USB disks that are not managed by USBDRIVE (including those managed

by BIOS Legacy Support), you usually need to reboot the computer for

the disk to get reset properly.  With USBDRIVE, you do not

necessarily need to reboot when a disk changes (though you certainly

can if you want to).  USBDRIVE has a Reset option (discussed below on

page 155) that you can use to Reset a USB disk if you need to.

Low-Level Formatting

��������������������

There are two different types of formatting you can do to a disk: a

high-level format and a low-level format.  A high-level format simply

erases the data on the disk by re-writing the File Allocation Table.

The MS-DOS FORMAT utility only does a high-level format to hard

drives.  A low-level format actually re-creates the physical tracks

on the magnetic media, making sure that the read/write heads and the

magnetic tracks are perfectly aligned.  The MS-DOS FORMAT utility

does both a low-level format and a high-level format to floppy

drives.

DOS USB DRIVER INTRODUCTION            Page 153 of 186

It is relatively rare for a hard drive to need a low-level format,

because of the "rigid" nature of the disk mechanics (the heads and

tracks rarely get out of alignment).  When you do need to low-level

format a hard drive, you can't use the MS-DOS FORMAT utility to do

it.  The process required to low-level format a hard drive is

hardware-dependent (different for each manufacturer and model), and

is built into the BIOS of the hard drive.  The MS-DOS FORMAT utility

does not know how to access the BIOS low-level format function

because it is different for each disk.  For example, in the old MFM &

RLL drives that were used in early computers, you could usually start

the low-level formatting routine by loading the DOS DEBUG program and

typing "G=C800:5".  Unfortunately, that doesn't work on modern hard

disks.

With SCSI disks (which includes USB disks), there is a special SCSI

request that is issued to the disk BIOS that causes the disk to be

low-level formatted.  You can tell USBDRIVE to low-level format a USB

disk as follows:

USBDRIVE LowLevelFormat #

where the # is the physical disk number (INT 13h disk number) of the

Disk/LUN.  For most computers (which only have one "real" hard disk

numbered 80h), the first USB disk will probably be physical disk

number 81h (129 decimal).  You can determine the physical disk number

with the /Status option switch for USBDRIVE.

USBDRIVE simply issues a SCSI format request to the disk.  I have not

actually tried issuing a SCSI format request to an external USB hard

disk, but it should work (assuming the disk is really SCSI-compatible

like it's supposed to be).  The story is a little bit different for

USB flash drives, however.  Remember that the main purpose of a low-

level format is to re-align the heads and tracks of the magnetic

media.  Flash drives are solid-state and don't actually have heads or

tracks or magnetic media, so there is nothing to align.  Trying to

low-level format a flash drive could either do nothing, or it could

simply destroy the data on the disk (overwrite the entire disk with

zeroes or some random data pattern).

I have tried issuing a SCSI format request to a few different USB

flash drives just to see what would happen.  On the flash drives I've

tried, some have simply outright rejected the SCSI format request as

an invalid request, while others have accepted the SCSI request but

did not actually do anything to the disk (did not erase or change any

data).  If you want to experiment with formatting your flash drives,

you certainly can.  Just be aware that it could destroy any data that

is on the disk, so make sure there's nothing on the disk that you're

not willing to lose forever.

DOS USB DRIVER INTRODUCTION            Page 154 of 186

Resetting Disks

���������������

There are times when it is necessary to perform a reset on a USB

device, including USB disk drives.  A reset can either be performed

by physically unplugging and re-inserting the device into a hub port,

or by performing a "soft reset" on the device using software.

Normally, the way you issue a soft reset to a USB device is through

the host controller driver (USBUHCI{L}), but this requires you to

know the Host Index and Device Address of the device you want to

reset.

If you want to perform a soft reset on a particular USB disk,

USBDRIVE provides a way for you to do it without needing to

"translate" the disk information into a USB Host Index and Device

Address, as follows:

USBDRIVE Reset #

where the # is the Index (in USBDRIVE) of the USB device you want to

reset.  The Index is a 0-based number, and is listed in the first

column of the output of the /Status option switch in USBDRIVE.  The

first device has Index 0, the second has Index 1, etc.  If you only

have one USB disk device plugged in, it will usually be Index 0.  The

/Status option also shows the USB Host Index and Device Address if

you want to reset the disk using USBUHCI{L} instead of using

USBDRIVE.  They both accomplish the same result -- the switch option

is USBDRIVE is just there to make it more "convenient".

Troubleshooting

���������������

USB disks are the most unreliable, flaky USB Devices I have come

across so far.  I have FAR more trouble with them than I do with

printers, mice, keyboards, or joysticks.  The main problem with USB

disks is that they are not completely compatible with the SCSI

command set, even though they're supposed to be.  If you're having

problems getting a particular disk to work with USBDRIVE, there are a

few tools and tricks you can use that will sometimes fix the problem,

or at least help you to figure out what and where the problem might

be.

DOS USB DRIVER INTRODUCTION            Page 155 of 186

Bad Devices

�����������

When a USB drive does not configure correctly with USBDRIVE, the

Bad option switch is usually the first place to start

troubleshooting.  Simply do the following:

USBDRIVE Bad

This will give you details about exactly where during the complex

initialization process that the failure occurred.  Unfortunately,

the information provided is rather cryptic, and unless you're

familiar with the inner workings of USBDRIVE, it won't make any

sense.  You will need to download the source code for USBDRIVE to

do any significant troubleshooting.

Internal Buffers

����������������

As USBDRIVE is initializing a particular disk, it attempts to

download several separate groups of data from the disk.  These are

issued as SCSI requests to the disk drive (USB disks almost always

use the SCSI software protocol).  The SCSI Requests that USBDRIVE

issues are called Inquiry, Read Format Capacities, Read Capacity,

and Mode Sense (if you are familiar with SCSI, you should already

be familiar with what all of those are).  In addition, USBDRIVE

will download at least one sector's worth of data from the disk in

order to figure out if and how the disk is partitioned.

USBDRIVE downloads all of this data into buffers in the TSR, and

USBDRIVE has an option switch to let you see what's in the

buffers.  Again, what you see there is rather cryptic, but can be

very informative about how the disk is responding to various

requests (at least if you have basic understanding of the SCSI

protocols).  Note that USBDRIVE only has one set of buffers, which

are reset and reused each time it tries to initialize a new disk.

Therefore, the data in the buffers will only be valid for the last

disk that USBDRIVE tried to initialize.  To view the contents of

the buffers, you do the following:

USBDRIVE Buffers

DOS USB DRIVER INTRODUCTION            Page 156 of 186

SCSI Error Codes

����������������

The SCSI software protocols are fairly comprehensive, and when a

particular SCSI request fails for some reason the device returns a

detailed set of error codes that describe why the request did not

work.  USBDRIVE keeps track of the last 10 SCSI requests that

returned errors for each Disk/LUN, so that you can view them if

you want to.  When a USB disk doesn't appear to be working

correctly, viewing the last several errored SCSI requests may help

you to recognize some sort of pattern as to why things aren't

working correctly.  To view the SCSI error codes for a particular

disk, you do the following:

USBDRIVE ASCQHistory #

where the # is the physical disk number (INT 13h disk number) of

the Disk/LUN.  For most computers (which only have one "real" hard

disk numbered 80h), the first USB disk will probably be physical

disk number 81h (129 decimal).  You can determine the physical

disk number with the /Status option switch for USBDRIVE.

What USBDRIVE displays on the screen for each of the 10 entries

will be the SCSI Request Type, the Sense Key, the Additional Sense

Code, and the Additional Sense Code Qualifier.  Here is a typical

example:

SCSI Request 00h = Test Unit Ready

Sense Key    06h = Unit Attention (Target Reset)

ASC/Q Code 2800h = Not Ready to Ready Transition, Medium Change

Again, if you're already familiar with the SCSI protocols, this

probably makes sense to you.  If not, you'll probably need to

study up on SCSI if you want to do any effective troubleshooting

with USB disks.  Hopefully, everything will work just fine for you

and you'll never need to do any troubleshooting.

Delaying Initialization

�����������������������

Sometimes, USB drives (like SCSI drives) need a little bit of time

to "spin up" before they will initialize properly.  However, most

USB drives (especially flash drives) do not need any "spin up"

time.  Therefore, by default, USBDRIVE will not delay at all after

it detects a new disk before it tries to initialize it.  USBDRIVE

lets you adjust the Delay value to anything between 0 and 60

seconds.  For example, to set the delay to 5 seconds, you would do

the following:

USBDRIVE Delay 5

DOS USB DRIVER INTRODUCTION            Page 157 of 186

I leave the delay set at the default of 0 on all of my computers,

since none of the disks I use require extra initialization time.

If a disk doesn't initialize properly, try changing the delay

value (make it bigger) and see if it helps.  If leaving the

setting at 0 works (as it does for me), USBDRIVE will load things

much quicker for you.

Sector Transfer Size

��������������������

Some disks, for whatever reason, are "slow" and do not allow

USBDRIVE to transfer data at the maximum possible speed.  What

usually happens on these drives is that you will be in the middle

of transferring some files, and it will stop in the middle of the

data transfer with a "Drive Not Ready" error message.

When a program asks to transfer a large amount of data at one

time, USBDRIVE may need to break up the request into smaller

"chunks" -- small enough that each chunk can be transferred

without errors.  Decreasing the amount of data that is transferred

in each chunk usually increases the reliability, but also usually

decreases the overall data transfer speed.

You control the size of each data chunk as follows:

USBDRIVE MaximumSectors: #

where the # is the maximum number of sectors that USBDRIVE will

transfer in each data chunk.  The # can be any number between 1

and 64, with smaller numbers generally leading to increased

reliability but decreased data transfer speed.  The default value

that USBDRIVE uses is 1, which means that data transfers are slow,

but usually very reliable.  On my computers, I set the value to 4

since that is the is the largest value that works reliably on all

of my disks.  I have one particular disk (made by Sandisk) that

works with the maximum value of 64, but none of my other drives

do.  You will probably want to experiment with your particular

disks, see what happens, and configure USBDRIVE accordingly.  If

you discover any interesting patterns or relationships, please let

me know.

DOS USB DRIVER INTRODUCTION            Page 158 of 186

Following is a chart that shows how the data chunk size affects

transfer speed.  This data for this chart was created by making a

large file (26,762,776 Bytes) on the Sandisk drive I have that

works OK using a maximum transfer sector size of 64, formatted

with FAT32, and doing a "COPY FileName NUL" command at a DOS

command prompt.  There were no other devices on the USB host

controller except the Sandisk flash drive.  Copying to NUL instead

of another disk eliminates possible hardware effects of the other

disk, so gives a true indication of the speed of the USB disk

itself.  The chart should be used as a relative rather than

absolute speed comparison -- as the saying goes, "your mileage may

vary".

Maximum  Xfer Time  Xfer Speed

Sectors  (Seconds)    (Mbps)

-------  ---------  ----------

1     643.40        0.33

2     343.23        0.62

4     175.54        1.22

8     134.51        1.59

16     198.88        1.08

32     118.42        1.81

64     107.87        1.98

Note that increasing the data chunk size doesn't necessarily

always increase the speed.  In the chart above, increasing the

maximum transfer sector size from 8 to 16 actually decreased the

data transfer speed.  I'm not sure exactly why this happened, but

think it has something to do with the relationship between data

chunk sizes and the USB frame timing.  For each chunk of data that

gets transferred, there are actually several USB transactions that

take place on the bus.  If a piece of the transaction completes

near the beginning of a frame (which happens every millisecond)

instead of near the end, the process must wait nearly one full

millisecond (until the start of the next frame) before it can move

forward to the next step.  In addition, factors such as the host

controller manufacturer and model, disk manufacturer and model and

size and FAT type, number and type of hubs between the disk and

the root hub, number and type of other devices on the bus, etc.,

may all affect the data transfer speed.

DOS USB DRIVER INTRODUCTION            Page 159 of 186

Using USBASPI

�������������

The initialization process the USBDRIVE goes through appears to be

"incomplete" for some drives.  For drives that have this problem,

they "stall out" very early in the initialization process,

sometimes even while the Host Driver (USBUHCI{L}) is attempting to

do the "high level" initialization (before USBDRIVE even gets into

the picture).  Some of the drives that do this work with other DOS

USB driver programs, and in particular the popular ones that are

available from the Panasonic site in Japan (USBASPI.SYS &

DI1000DD.SYS).  As it turns out, you can sometimes use the

USBASPI.SYS file to "fix" USBDRIVE so that it works with these

particular drives.

The USBASPI.SYS file is actually a combination device driver

(.SYS) file and executable (.EXE) file.  When used as a .SYS file

(loaded in CONFIG.SYS), it installs itself into memory and starts

to manage the USB disks.  However, when used as an .EXE file, all

it does is initialize the USB drives that it finds, but does not

install itself into memory or manage anything.  What you need to

do is rename the USBASPI.SYS file to USBASPI.EXE, and then run it

immediately before you load these USB drivers into memory.  For

example:

USBASPI

USBUHCIL

USBDRIVE

I'm not sure exactly what USBASPI "resets" in the drives that lets

USBDRIVE work afterwards, but it obviously does something.  I have

contacted the Japanese company that wrote USBASPI (Novac)

regarding obtaining the source code for USBASPI, but they have not

yet responded to my request (and I suspect that they never will).

At some point in the future, I may try to reverse-engineer USBASPI

and try to figure out how they do it.  In the meantime, you may

need to use USBASPI to "fix" USBDRIVE as detailed above.  I

personally do not own any disk drives that need the USBASPI fix,

but some others have reported it helps in certain situations.

Sandisk U3 Software

�������������������

Most (if not all) USB flash drives made by Sandisk come with

special U3 software for MS Windows installed.  When the U3

software is installed, the disk is configured as a CD-ROM rather

than as a disk drive, and is not recognized by USBDRIVE.  You must

uninstall the U3 software (which you must do from within MS

Windows) before the disk will be recognized by USBDRIVE.

DOS USB DRIVER INTRODUCTION            Page 160 of 186

Security Software

�����������������

Some USB drives come with security protection on them, and they

cannot be accessed without first entering some sort of password.

I have never actually used one of these drives, and have no idea

how the security protection works.  From some of the information

I've seen, though, the security protection (at least on some of

the drives) only seems to work when you're using MS-Windows.  If

you're accessing the drives under any other operating system (Mac,

Linux, DOS, etc.), there is no security at all.  That makes the

security rather pointless, if you ask me.  If somebody has one of

those drives, they can let me know what they discover.

Wear Leveling

�������������

The solid-state media used to make flash drives can only be

written to a limited number of times (it does not last forever).

The number of times it can be written to is a lot (at least tens

of thousands of times, depending on the "quality" of the drive),

so under normal circumstances this is not a major concern.  Also

note that WRITING the data is what has a limited life span -- you

can READ the data as much as you want (at least that's what they

claim).

When storing data on a disk, certain areas of the disk get written

to a lot (like the File Allocation Table, which is near the start

of the disk), while others (like the data area near the end of the

disk) rarely change.  All modern USB flash drives include a

process called "wear leveling", which is designed to "spread out"

the disk writes across the entire disk rather than keeping them in

the same place all the time.  That is, which transistors in the

flash drive are used to store the File Allocation Table will move

around the disk to different places as you keep changing the data.

This makes the disk ultimately last thousands of times longer than

it otherwise would.

Wear leveling happens in the background and you normally never

need to worry about it.  There is one thing to keep in mind,

however.  A disk that uses wear leveling (and all modern ones do)

may not actually erase or overwrite data like it does on magnetic

media -- it may just move the data to a different set of

transistors.  If your data is "sensitive" and you never want

anyone to see it after you erase it, you probably shouldn't use

flash drives at all.  The data that you "erased", even if you did

it with special "disk wiping" software, may still be there stored

in the transistors.

DOS USB DRIVER INTRODUCTION            Page 161 of 186

DOS Sector Size

���������������

USBDRIVE simply provides physical access to the USB drives -- it

expects the operating system (DOS) and other software to

"interpret" and format the data that is stored there.  I have

heard of, but not actually seen, certain USB drives (such as

magneto-optical drives) that use a sector size larger than the

standard 512 bytes.  I do not know if these drives are actually

configured to look like hard drives (if they are in a standard

DOS-compatible FAT12/FAT16/FAT32 format).  If they are not, then

it will take special software to read and write to them.

In its base form, DOS will not work correctly with sector sizes

larger than 512 bytes.  There is a patch you can make to the DOS

kernel (MSDOS.SYS, IBMDOS.COM, or equivalent) that will allow them

to work with such disks, but it increases the size of the DOS

kernel (and will also increase the size of USBDRIVE) in memory.

If you run across such a disk, and want to patch the kernel,

contact me and I can tell you what to do.  This is definitely not

something you would want to do on every computer, since it will

simply waste memory unnecessarily.

Miscellaneous Notes

�������������������

To have USBDRIVE show you the current status of the program and the

drives it is managing, you would do the following:

USBDRIVE Status

This will display "general" information about the USBDRIVE program

(the drive initialization delay, whether it will assign drive letters

to FAT32 volumes, etc.).  In addition, it will also display details

for each of the individual USB disks you have installed on the

computer, including USB Host Index & Device Address, LUN, drive

letter, and capacity information.

USBDRIVE allows access to the drives at three different levels: BIOS

(INT 13h, including extended functions for large capacity disks), DOS

(through a device driver), and DOS direct functions (INT 25h & 26h).

This should allow virtually all programs (including "low-level"

programs like FDISK and FORMAT) to access and manipulate the USB

disks.

DOS USB DRIVER INTRODUCTION            Page 162 of 186

USBDRIVE provides a special extension to the INT 13h (Disk Drive)

functions in the BIOS.  The extension allows you to issue virtually

any SCSI request to a USB-attached disk.  This is conceptually

similar to ASPI (Advanced SCSI Programming Interface), which you may

already be familiar with.  Details of the INT 13h BIOS extension are

not provided here, but will be included with the final release (and

complete documentation) of USBDRIVE.  Until then, you can download

the source code for USBDRIVE if you want to study details about the

BIOS extension.  USBDRIVE itself uses the BIOS extension to perform

the /Format and /InquiryData option switches.

In the current state, the USB Host Drivers (USBUHCI{L}) only support

USB version 1.x compatible Hosts (maximum 12 Mbps).  Some of the

other DOS USB driver packages support USB version 2 (EHCI, maximum

480 Mbps).  Therefore, with these DOS Drivers, the access may be slow

compared to some other drivers.  Speed will be increased in the

future as other Host Controller drivers are written.

To reserve drive letters, USBDRIVE inserts some "fake" information

into the DOS Current Directory Structure (CDS) and Drive Parameter

Block (DPB) tables.  This is what all removable media (including

floppies) do.  Until a disk is actually inserted, USBDRIVE has

literally no idea what the drive will actually look like (how big,

how many partitions, what FAT type, etc.).  The "fake" information

inserted by USBDRIVE looks like a drive with no data on it (0 kB).

The information you see in the DRIVES program below (page 164) for

the USB drives will look like a bunch of 0 kB disks, until the USB

disks are actually plugged in and accessed by DOS.  Some people

mistakenly assume that USBDRIVE has "resized" a USB drive when they

see the size information in DRIVES, and that is not the case.  Until

DOS actually accesses a drive (downloads some data from it), the

information stored by DOS (and displayed by DRIVES) is totally bogus.

Be sure you do not confuse FAT32 (the ability to access disks up to 2

terabytes in size) with LFN (Long File Names).  They happened around

the same time (the Windows 95 era), so many people assume they are

related.  In reality, they have NOTHING to do with each other.  You

can have LFN without FAT32, and vice versa.  However, I will tell you

that Microsoft's design of LFN is one of the biggest kludges you have

ever seen in your life.  LFN is COMPLETELY incompatible with programs

like Microsoft's own DEFRAG program that came with several versions

of DOS.  If anybody other than Microsoft had designed LFN the way

they did, they would have been laughed right out of the programming

world.

DOS USB DRIVER INTRODUCTION            Page 163 of 186

In its original form, USB Legacy Support originally only addressed

keyboards and mice, and did not even address disk drives.  Legacy

Support in general is a kludge, and with disk drives it is even more

so.  That is at least partially why transferring control of disk

drives from Legacy Support to an Operating System driver does not

work very well.

DRIVES

������

Version:     0.01

Prog Type:   Support

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete, a few minor tweaks required

Environment: None

Redirection: No

Purpose:     Show details for all DOS disk drives

TSR Memory:  0 kB (not a TSR)

DRIVES is a program designed to let you view details about your disk

drives (drive letters) in the same way that DOS looks at them.  There is

a table that DOS uses to manage disks, called the Current Directory

Structure Table or CDS Table.  Each entry in the CDS Table is a single

CDS, and corresponds to a single drive letter (starting at A:).  As the

name implies, the CDS table keeps track of the current directory for

each drive, as well as a small amount of other information (such as

whether it is a physical drive or a network drive).  Other than the

current directory information, the most important thing stored in the

CDS is a pointer to another structure called the Drive Parameter Block

or DPB, which is where most of the critical information about the drive

is actually stored.  The DPB is discussed in the next paragraph.  The

total number of entries in the CDS Table (the total number of drive

letters available) is determined by the LASTDRIVE setting in your

CONFIG.SYS file, and cannot be changed after DOS has booted.

For each valid CDS, there is a corresponding Drive Parameter Block or

DPB.  The DPB contains most of the critical details about the drive,

including the FAT type, capacity, physical disk and partition

information, etc.  The DPB also contains a pointer to the device driver

for the drive, usually installed by the DOS kernel or by a special

driver in the CONFIG.SYS file.  DOS never accesses the drive directly --

it always does it through the device driver.

DOS USB DRIVER INTRODUCTION            Page 164 of 186

The Drive Parameter Block also contains a flag which becomes set when

the drive has been Accessed in some fashion by DOS (when DOS has read

some data from the drive).  This flag is very critical, because until a

drive (at least a removable drive, such as a floppy or USB disk) has

been accessed, none of the other details in the DPB can be trusted.  For

example, the default DPB entry for a 3-1/2 inch floppy drive usually is

set up to look like a low-capacity (720 kB) disk, but the Accessed flag

is cleared.  This tells DOS that there is a slot to plug in a disk, but

because it hasn't been Accessed, we don't know how big the drive

actually is.  If you insert a 1.44 MB floppy, but don't read any data

from it, the contents of the DPB won't change (it will still show an

unaccessed 720 kB floppy).  When you actually read some data from the

drive, DOS will change the DPB entry to correspond to an accessed 1.44

MB drive.  However, with floppy drives, DOS will regularly unset (clear)

the Accessed flag, since DOS can't actually tell if you've changed disks

(media) or not.  If the media has changed but the Accessed flag is not

cleared, DOS will probably corrupt the new disk (because it thinks it is

still the old disk).

DRIVES simply shows you some of the details that are in the DOS Current

Directory Structure (CDS) Table, and the corresponding Drive Parameter

Block (DPB) entries.  This is a "snapshot" view of all the disks you

have, as viewed from the perspective of DOS.  It is critical to keep in

mind that if the drive has not been accessed yet (if the "ACCESd" column

is not "Y" for a particular drive letter), none of the other details in

the output may be accurate.  That is simply how DOS handles removable

disks which can be of different formats and capacities.

USBPRINT

��������

Version:     0.08

Prog Type:   Device/Interface Driver

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete, a few minor tweaks required

Environment: USBPRINT

Redirection: Yes

Purpose:     Driver for up to 8 USB Printers

TSR Memory:  9.7 kB

USBPRINT is a Device/Interface Driver program for up to eight USB

printers.  USBPRINT emulates a regular printer (LPT) port in the

computer. This allows DOS programs to think that your printer is

attached to a regular parallel port instead of to a USB port.

DOS USB DRIVER INTRODUCTION            Page 165 of 186

Being unable to print to a USB printer from DOS (or from a DOS box

underneath Windows) has been a long-standing problem since USB came out

in the mid-1990's.  There have been several Windows programs that have

been developed to alleviate those problems.  Of course, because those

programs require Windows, the programs are not really fixing the

problem, they are just bypassing it.  USBPRINT is the only program I

know of that will actually allow you to print to a USB printer from

"real" DOS.

There are a couple of specific limitations that USBPRINT has, however,

that you need to know about.  The first limitation is that USBPRINT

emulates printer ports in the BIOS, and not at the hardware level.

Fortunately, nearly all DOS programs use the BIOS and do not interact

with the parallel port directly at the hardware level.  A program that

does interact at the hardware level will usually have an option to allow

it to use the BIOS instead (WordPerfect for DOS being an excellent

example of this).  If the program does not have an option to print using

the BIOS, it may have an option to print to a file (rather than sending

things directly to the printer), which is a "work-around" that will also

allow you to use USBPRINT to print from the program to a USB printer.

The other limitation of USBPRINT is that it simply emulates parallel

ports -- it does not actually emulate printers.  That is, the program

that is printing and the printer that it is printing to must use the

same protocol.  USBPRINT does not provide any protocol conversion

(unlike many of the Windows solutions, which can provide protocol

conversion).  For example, if you have an old (parallel port) printer

and a new computer with no parallel port, you can simply obtain one of

the USB-to-parallel-port converter cables, and with USBPRINT you should

be able to use the old printer just like you did on your old computer.

If you buy a new USB printer, and want to use it in "real" DOS, you will

usually want to buy a printer that uses or emulates the Hewlett Packard

PCL (Printer Control Language) protocol, which many DOS programs

understand.  Some of the more sophisticated DOS programs also understand

the PostScript protocol, though "simple" DOS programs generally do not.

Also keep in mind that even if a printer (especially a multi-function

printer) is made by Hewlett-Packard, it does not necessarily mean it

understands the PCL protocol.  I made this mistake once, and very much

regretted it.  Many of the less expensive Hewlett-Packard multi-function

devices (printer, copier, scanner, fax machine) use the LIDIL

(Lightweight Imaging Device Interface Language) protocol instead of PCL,

and you cannot print to them directly from DOS even with USBPRINT.

DOS USB DRIVER INTRODUCTION            Page 166 of 186

In addition to simply allowing you to print to USB printers in DOS,

USBPRINT also "enhances" the printer BIOS to allow you to do things with

printers (even those that are hooked up to "real" parallel ports) that

you could not do before.  These enhancements include having a total of 9

available printer ports (LPT1-LPT9) instead of just 3, two-way

communication (receiving data from the printer instead of just sending

data to the printer), and being able send blocks of data to the printer

instead of just sending one character at a time.  In fact, the INKLEVEL

program described below (page 176) takes advantage of these special

functions to display the ink levels of certain ink-jet printers.  Exact

details of the enhanced printer BIOS functions will not be discussed in

this document.  You can either download the source code for USBPRINT or

wait for the official USBPRINT program and documentation if you want

more information about it.

Unlike most of the other Device/Interface Drivers included in this

package, a USB Host Driver (USBUHCI{L}) does NOT need to be installed

into memory before installing USBPRINT.  USBPRINT provides enhanced

functionality to the regular (parallel port) printers that are useful

even if you don't have any USB printers installed.  To install USBPRINT

into memory, you can simply run it with no option switches:

USBPRINT

I want to explain something here that could be a source of confusion.

The standard BIOS has three parallel ports, which are associated with

three hardware I/O addresses.  At the BIOS level, the parallel ports

don't have a "name" -- they are simply the first, second, and third

parallel ports.  The terms LPT1, LPT2, and LPT3 are the names of DOS

Devices, and only have meaning when you're at the DOS level.  If you're

not using a DOS-based operating system, the terms LPT1, LPT2, and LPT3

don't mean anything and may not even exist on the computer at all.  The

BIOS parallel ports are always there no matter what the operating system

is.  In this document, we will use the terms somewhat interchangeably

(DOS' LPT1 is the same thing as the BIOS' first parallel port), though

technically that is not always true.

Printer BIOS

������������

The BIOS on a modern computer supports three parallel (printer)

ports, which DOS calls LPT1, LPT2, and LPT3.  Most modern computers

usually only have one physical parallel port, if they have any at all

(laptops usually don't).  It's getting harder and harder to find

printers that will even work with a parallel port -- most all of them

are going strictly to USB interfaces.

DOS USB DRIVER INTRODUCTION            Page 167 of 186

Most DOS programs, by default, assume that if you have a printer, it

is attached to LPT1.  Some programs, in fact, will not even let you

print to any printer port except LPT1 (they will not let you print to

LPT2 or to a serial-attached printer, for example).  For this and

other reasons, if you only have one printer, you will usually want it

to be mapped as LPT1.

USBPRINT supports up to 9 parallel ports (LPT1-LPT9), instead of just

3 like the normal BIOS.  As you attach and detach USB printers,

USBPRINT automatically assigns them to the LPT ports.  USBPRINT

automatically detects which "real" parallel ports (LPT1-LPT3) you

have on the computer, and by default will not assign them to the USB

printers.  In other words, by default, USBPRINT will never assign a

USB printer to be LPT1 if your computer has at least one parallel

port.  It doesn't matter whether you actually have a printer attached

to the parallel port or not -- if USBPRINT doesn't see a printer, it

doesn't necessarily mean you don't have a printer -- it could be that

you have one and it is simply turned off or unplugged at the moment.

If you want USBPRINT to assign a USB printer to LPT1, even if your

computer has a "real" parallel port, you need to tell USBPRINT to

override its default behavior and to "replace" the parallel ports

with the USB printer ports, as follows:

USBPRINT ReplaceParallel:Yes

When you do this, the first USB printer will be assigned as LPT1, the

second as LPT2, etc.

The BIOS uses a particular section of memory, called the BIOS Data

Area (BDA), to store various information about the computer hardware.

Information about the three parallel ports (LPT1-LPT3) is stored in a

specific location in the BDA, and what is stored is the base I/O

address of the parallel port (LPT1, for example, is usually I/O

address 370h).  Many programs, to determine whether or not a printer

is attached to an LPT port, look to see if there is a valid number

(non-zero I/O port address) in the BDA.  Therefore, in order for

USBPRINT to "fake" some programs into believing that a USB printer is

actually a parallel printer, USBPRINT must put a "fake" I/O port

address number in the BDA.  Note that most programs do not actually

use the parallel port I/O address stored in the BDA to interact with

the printer at the hardware level -- they just look to see if the

number in the BDA is zero or not.

DOS USB DRIVER INTRODUCTION            Page 168 of 186

USBPRINT needs a 32-byte range of I/O addresses to be able to "fake"

the three BIOS parallel ports (LPT1-LPT3).  The 32-byte range should

really be "fake", and should not be associated with any type of real

hardware at all (parallel port or otherwise).  By default, USBPRINT

will scan for a range of fake I/O addresses when it is first

installing itself into memory.  To perform the scan, USBPRINT must

actually send data back and forth to the I/O addresses it is trying

to test to see if there appears to be any real hardware attached

there.  On rare occasions, if there is some sort of hardware actually

attached to the address being tested, USBPRINT will inadvertently

screw something up or reconfigure the hardware that is attached

there.  USBPRINT is very careful about how it scans the I/O

addresses, but can still screw something up if the hardware reacts in

an unusual way.  If you want to specify a particular I/O address

range for USBPRINT to use, instead of having it automatically try to

find one by scanning, you would do the following:

USBPRINT IOAddress: #

where # is the starting address of a 32-byte range of I/O address to

use. # must be a number between 400h and FFE0h, and must be evenly

divisible by 32 (20h).  Again, if you use this option, it is critical

that the I/O address range is not associated with any real hardware

at all, or you can get yourself into trouble.

Even though many programs will look in the BDA to see if there is a

non-zero I/O address, almost no programs will actually use the I/O

address to do anything.  Programs that actually use the I/O address

are directly interacting at the hardware level, and are not using the

BIOS, and therefore will not work with USBPRINT.

To have USBPRINT show you a current status of the parallel ports, you

would do the following:

USBPRINT PortStatus

This will show you the status of all of the parallel ports, including

I/O addresses, printer status, and printer type.  The PortStatus

option can be used whether USBPRINT is installed in memory or not

(you can use it just to see the status of your parallel port

printers).

DOS USB DRIVER INTRODUCTION            Page 169 of 186

Reassigning LPT Ports

���������������������

One of the BIOS extensions that USBPRINT provides is the ability to

remap (redirect) printer ports to each other.  For instance, let's

say you have a printer attached to the third parallel port (LPT3),

but want to be able to print to it from a program that only

understands how to print to LPT1.  To do this, you would use USBPRINT

to remap LPT1 to LPT3, causing anything that got sent to LPT1 to

actually go the third parallel port, as follows:

USBPRINT Map LPT1 LPT3

To restore the original mapping (sending LPT1 to the first parallel

port again), you would do the following:

USBPRINT Map LPT1 LPT1

USBPRINT must be installed in memory as a TSR in order to be able to

remap parallel ports to each other.  However, there do not need to be

any USB printers attached to the computer for this to work.  You can

simply use USBPRINT to remap "real" parallel ports to each other.

Printer Device ID's

�������������������

In the mid 1990's, the Institute or Electrical and Electronics

Engineers (IEEE) developed a standard for parallel ports, which they

called 1284.  Since that time, nearly all printers have followed the

standard.  Part of the standard includes something called a Device ID

String, which is stored in the printer.  Using a special protocol,

the computer can download the Device ID String from the printer.  The

Device ID string MUST contain the manufacturer and model number, but

can also contain any other items that the printer manufacturer may

see fit to include (such as protocols and serial numbers).  The

Device ID String is how Windows (and other operating systems)

automatically identify what kind of printer you have.

USBPRINT allows you to download and view the Device ID Strings of all

the printers attached to your computer, whether they are attached

using parallel ports or USB ports.  To view the Device ID Strings of

all the printers attached to your computer, you would do the

following:

USBPRINT DeviceIDStrings

USBPRINT does not need to be installed in memory as a TSR to be able

to download the Device ID Strings.  Also note that if you have a

really old printer (manufactured before the IEEE 1284 standards were

created), your printer will not have a Device ID String.

DOS USB DRIVER INTRODUCTION            Page 170 of 186

Resetting a Printer

�������������������

There are times when a printer gets "stuck" in the middle of print

job, or for some other reason needs to be reset (reinitialize the

protocols, margins, fonts, etc.).  There is a "soft reset" function

that all printers are supposed to respond to that is sent through the

control port (parallel or USB).  To issue a soft reset through the

port, you must provide to USBPRINT an LPT port number.  For example,

to reset LPT1, you would do the following:

USBPRINT Reset LPT1

In addition to the soft reset signal sent through the control port,

nearly all printers also have some sort of reset that can be sent by

sending a special series of characters to the printer.  However,

exactly what the special series of characters is depends on the

printer (it is protocol dependent).  USBPRINT cannot reset a printer

using any type of protocol-dependent methods -- you (or the program

you are using) will need to do that yourself.

Epson Printer Initialization

����������������������������

At least some USB printers made by Epson require a special

initialization process.  After they are first turned on or reset,

they come up in something called "packet mode".  From what I've been

able to gather, nobody outside of Epson seems to know exactly what

packet mode is or what it is good for -- all that anybody seems to

know about packet mode is that you can't print while you're in it.

You must send a special series of characters to the printer to exit

packet mode before you can actually print anything.

USBPRINT will automatically recognize if a printer is made by Epson

or not, and if it is, USBPRINT will (by default) send the special

"exit packet mode" sequence at the appropriate times.  If you want to

override this default, and either send the sequence manually or do it

from some other program, you would do the following:

USBPRINT AutoEpson: No

The AutoEpson option only applies to Epson printers attached to USB

ports, not those attached to parallel ports or USB-to-parallel-port

converter cables.  For example, I used to have an Epson printer that

had both a USB and a parallel port.  When it was hooked up through

the parallel port, I did not ever need to issue the "exit packet

mode" sequence.  I only needed to do it when it was attached through

the USB port.

DOS USB DRIVER INTRODUCTION            Page 171 of 186

You can also have USBPRINT send the "exit packet mode" sequence to a

printer at any time, by specifying a particular LPT port.  For

example:

USBPRINT SendEpson: LPT1

will send the sequence to the printer attached to LPT1.

Copying Files to a Printer

��������������������������

One way you can print something to a printer (if it is a text file,

or a file that already has the special escape sequences needed to set

the printer into graphics mode), is to simply use the DOS COPY

command.  For example:

COPY FILENAME.TXT LPT1

However, one limitation of the DOS copy command is that you can only

print one file at a time -- you cannot use the DOS * or ? wildcards

to print multiple files at the same time.  USBPRINT provides a "copy"

function similar to DOS, but which will let you use wildcards.  For

example, to print all files in the C:\BATCH directory with the .BAT

extension to LPT9, you would do the following:

USBPRINT COPY C:\BATCH\*.BAT LPT9

As long as the underlying DOS version supports Long File Names, the

USBPRINT copy function also supports them, so you can do something

like this:

USBPRINT COPY "C:\Long File Names\Test.P??" LPT1

You can also perform multiple copies with one USBPRINT command line,

for example:

USBPRINT COPY *.PRN LPT1 COPY \AUTOEXEC.BAT LPT2

By default, at the end of each file, USBPRINT will send a new page

sequence (carriage return/form feed combination) to the printer so

that each different file will get start on a new page.

There may be times when you do not want USBPRINT to send the new page

sequence, either because the files being printed are not simple text

files (they already contain the appropriate escape codes and new page

sequence codes) or because you just do not want each file to be

separated onto a distinct page.  If you do not want USBPRINT to send

the new page sequence between each file, you would do the following:

USBPRINT FormFeed: No

DOS USB DRIVER INTRODUCTION            Page 172 of 186

You can use USBPRINT to print multiple files like this at the same

time even if USBPRINT is not installed in memory as a TSR.  That is,

you can use this option to send files to "real" parallel printers

even if you do not have any USB printers installed.

The CPU BOUND Bug

�����������������

There is a special instruction in the CPU called BOUND, which has a

"bug" associated with it.  Actually, what happens is not truly a bug,

since it works exactly like it's designed to work.  IBM screwed up

when they originally designed the PC BIOS, and used some of the

Interrupts for BIOS functions that were officially listed by Intel as

"reserved for future hardware use".  The Interrupts were not used by

Intel in the first CPU's used on IBM-compatible computers (8088's).

As Intel upgraded the processors (80286, 80386, etc.), the "reserved"

Interrupts started to get used, which created conflicts with what IBM

had done in the BIOS.

One such conflict occurred with the BIOS PrintScreen function, which

uses Interrupt (INT) 5.  When Intel introduced the 80286 CPU, it

added a new CPU instruction called BOUND, which also uses INT 5.  If

the BOUND CPU instruction is ever issued by a program accidentally

(which can happen with a program that has bugs or insufficient memory

or some other problem), the computer will go into an infinite

PrintScreen loop.  That is, the computer will act like you are

hitting the PrintScreen key over and over and over again without

stopping.  If the printer attached to LPT1 is turned on at the time,

this will send an endless sequence of screen copies to the printer,

possibly wasting reams of paper and a lot of ink.  If the printer is

turned off when the accidental BOUND instruction is issued, the

computer will simply lock up and stop responding.  The only way to

stop this infinite PrintScreen loop is to turn off the computer --

the CPU is "stuck" in the accidental BOUND instruction with no way to

exit.

Because the "bug" is associated with printer ports, USBPRINT has a

"patch" for the BOUND bug.  Unfortunately, it is not possible for

USBPRINT to actually "repair" the bug.  If the BOUND instruction is

ever issued accidentally, it means that something has gone very wrong

with the memory of the computer (it is performing instructions that

it shouldn't be).  Therefore, the only safe thing for USBPRINT to do

when it senses an errant BOUND instruction is to shut down (reboot)

the computer, which is exactly what it does.

DOS USB DRIVER INTRODUCTION            Page 173 of 186

By default, the BOUND bug patch is enabled whenever USBPRINT is

installed in memory as a TSR.  If you want to turn the BOUND bug

patch off, you would do the following:

USBPRINT BoundPatch: No

To re-enable the BOUND patch again, you would do the following:

USBPRINT BoundPatch: Yes

USBPRINT must be installed in memory as a TSR in order for the BOUND

bug patch to be installed.  However, USBPRINT does not require any

USB printers to be installed.

The BOUNDTST program discussed below (page 175) will let you test the

BOUND bug patch that is installed with USBPRINT to make sure it is

working properly.

Miscellaneous Notes

�������������������

To have USBPRINT show you the current status of the program and the

printers it is managing, you would do the following:

USBPRINT Status

This will display "general" information about the USBPRINT program

(whether "overwrite" parallel ports with USB ports, whether the BOUND

bug patch is enabled, etc.).  In addition, it will also display

details for each of the individual USB printers you have installed on

the computer, including USB Host Index & Device Address, whether the

printer is made by Epson or not (and consequently will be issued the

special Epson setup strings when appropriate), and a description of

the printer as contained in the Device ID string.

USB is a packet-based architecture which is inherently designed to

send several bytes of data at a time.  Sending only one byte of data

at a time in USB is EXTREMELY inefficient and slow, and defeats one

of the main purposes of USB.  However, printers (parallel ports) are

designed to only transfer one byte at a time.  USBPRINT internally

buffers the data being sent by the parallel port BIOS (one character

at a time), and only sends the data to the USB printer when the

buffer gets full or enough time has elapsed since the last character

was sent that USBPRINT thinks you might be done printing.  However,

unlike true print spooling programs, USBPRINT does not buffer an

entire print job before it starts to print.  USBPRINT is sort of

half-way between a real-time printing program and a print spooler,

which can create problems in certain situations.

DOS USB DRIVER INTRODUCTION            Page 174 of 186

USBPRINT seems to work well with programs that print in the

foreground (programs that wait for the print job to finish before

they continue doing the next thing).  However, programs that attempt

to print in the background (such as WordPerfect for DOS) can have

problems.  The problems will generally arise if you are printing

large documents, if the printer has a limited amount of memory, or if

the printer prints very slowly.  You will need to experiment to see

what happens with your particular programs and printers.  If it turns

out that USBPRINT will not work correctly with your programs, you may

need to set them up to print to files instead of printing directly to

the printer, and then use DOS or USBPRINT to COPY the files to the

printer after you exit or shell out of the program.  It is usually

possible to automate this process using appropriate configuration

options in the program and well-written batch files (I will leave it

up to you to figure out how to do that).  Hopefully, a future

revision of USBPRINT will have this problem resolved and you won't

need to resort to "work-arounds".

BOUNDTST

��������

Version:     1.00

Prog Type:   Test

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete

Environment: None

Redirection: No

Purpose:     Tests CPU BOUND instruction bug fix

TSR Memory:  0 kB (not a TSR)

There is a special CPU instruction called BOUND that works in a very

unusual way, and consequently has a "bug" associated with it.

Technically, it is not a true "bug", because it works exactly like it's

designed to work, but can cause you problems at certain times anyway.

More details on exactly what problems you may have with the BOUND

instruction are given above in "The CPU BOUND Bug" (page 173), and will

not be repeated again here.

What BOUNDTST does is issue a BOUND instruction, on purpose, to the CPU.

If you do NOT have an appropriate patch installed in memory (such as the

one included in USBPRINT), your computer will either lock up or print an

endless series of PrintScreen's to your printer.  If you DO have an

appropriate patch installed, your computer should simply reboot.  Again,

the details are discussed above, so read "The CPU BOUND Bug" (page 173)

if you want to know more.

DOS USB DRIVER INTRODUCTION            Page 175 of 186

INKLEVEL

��������

Version:     0.01

Prog Type:   Test

Source Code: C++ (Borland Turbo C++ for DOS 3.0)

Status:      In progress, several major items to address

Environment: None

Redirection: No

Purpose:     Show amount of Ink remaining in HP & Epson Printers

TSR Memory:  0 kB (not a TSR)

INKLEVEL is a program designed to show you the amount of ink remaining

in your printer cartridges.  Many modern ink jet and laser printers have

a method of "downloading" this data from the printer, though the printer

manufacturers almost never provide a way for you to see the data without

using a special Windows program.  As far as I know, INKLEVEL is the only

program available anywhere that will let you view your printer ink

levels in DOS.

In order for INKLEVEL to work properly, however, you must have USBPRINT

installed in memory first.  USBPRINT provides certain extended printer

BIOS functions that INKLEVEL needs to communicate with the printer

properly.  Without USBPRINT (or some equivalent program developed in the

future) installed in memory, INKLEVEL will not function.

Currently, INKLEVEL only works with some Hewlett Packard and Epson

printers, because those are the only ones I have ever had access to for

testing.  I strongly encourage someone to take INKLEVEL and "run with

it", adding support for various other models and manufacturers of

printers.  I will not be doing this myself unless I personally come

across a printer that I want to support for some reason, and that is not

likely to happen any time soon.  Unlike most of my other programs

(written in ASM), INKLEVEL is written in C++ so it should be relatively

easy for someone to update and improve it.  My purpose for writing

INKLEVEL was to relieve some of the frustration I had with not being

able to view this information from DOS, and to "show off" some of the

capabilities of USBPRINT.

By default, INKLEVEL will show you the ink level status of all printers

(USB and parallel) that it can find.  For example:

INKLEVEL

If you have more than one printer, but only want to see the status of a

particular one, you must provide an LPT number, between LPT1 and LPT9.

For example:

INKLEVEL LPT1

DOS USB DRIVER INTRODUCTION            Page 176 of 186

If you have several printers but only want to see the status of one or

two of them, you can also provide more than one LPT number, for example:

INKLEVEL LPT1 LPT4

INKLEVEL requires USBPRINT to be installed in memory before it will

function.  If you do not need or want USBPRINT to be installed in memory

permanently (for instance, if you have no USB printers), you can set

yourself up a batch file to show you the ink levels that looks something

like this:

USBPRINT               Install USBPRINT into memory

INKLEVEL               Show ink levels for all printers

USBPRINT Uninstall     Uninstall USBPRINT from memory

What INKLEVEL displays will be an identification of the port and printer

type, followed by a cryptic string, followed by the ink levels of all

the different ink cartridges both in simple percentages (numbers) and as

a graph.  The "cryptic string" is the data that INKLEVEL downloads from

the printer that contains the ink level percentages embedded in it.  It

would be nice if the printer simply said something like, "Red: 20%", but

none of them ever do that.  The ink percentages are encoded in some

special way, unique to the manufacturer, and sometimes to the model, of

the printer.  INKLEVEL displays the string so that I could see it as I

was troubleshooting INKLEVEL during development, and I left it in there

just as an "added feature" so that you could see what's going on behind

the scenes.

Every printer I've ever seen that tells you what the ink level is lies

about it.  As you use the printer, the ink level will go down slowly

until it reaches 0%, which should mean the printer won't work any more.

I've been able to print hundreds of pages on every printer I've had,

even after it told me it didn't have any ink left.  I know part (if not

all) of this is a marketing ploy to get you to buy more ink than you

really need (Epson has been sued over this).  The bottom line is no

matter what the printer tells you (through INKLEVEL), you shouldn't

necessarily believe it.

DOS USB DRIVER INTRODUCTION            Page 177 of 186

USBSUPT1

��������

Version:     0.07

Prog Type:   Support

Source Code: ASM (A386 by Eric Isaacson)

Status:      In progress, several major items to address

Environment: None

Redirection: No

Purpose:     Translate various USB codes (numbers) to words

TSR Memory:  0 kB (not a TSR)

USBSUPT1 is the main support program for this USB software package.

It's main purpose is to "translate" the various cryptic USB code numbers

and data structures (of which there are many) into a format that you (as

a human being) can understand.  While USBSUPT1 can be used as a

standalone program, it's actually designed to be called by other

programs, such as the various DOS USB drivers.  USBSUPT1 is able to

translate many of the things that are common to several USB programs.

By putting the "common" things that several programs use into one place,

it makes each individual program smaller and makes it easier to update

the entire architecture.

USBSUPT1 does not actually provide all of the support functions itself,

however.  USBSUPT1 at times itself calls other "sub-support" programs,

which may be specific to a certain class of USB Device or Interface or

Vendor, or some specialized function that is complicated or data-

intensive enough to warrant a separate program.  For example, HIDSUPT1

(described below, page 180) is a support program specifically dedicated

to USB Human Interface Devices (mice, keyboard, joysticks, etc.), and

UNI2ASCI (described below, page 181) translates Unicode strings into

ASCII.

USBSUPT1, and the entire set of USB support programs in general, is

incomplete.  There are many kinds of USB devices that will need

specialized support programs, assuming DOS device drivers are ever

developed for them.  For instance, USBSUPT1 currently contains some of

the translation data it needs for USB Audio Device/Interfaces

(microphones, speakers, equalizers, etc.), but does not contain all of

them.  I was actually working on this several months ago, but decided it

would be better to distribute the USB package as it is (incomplete but

functional).

DOS USB DRIVER INTRODUCTION            Page 178 of 186

Because USBSUPT1 is designed to be called from other programs, it does

not contain many of the "nice" features that many of the other programs

have (lengthy lists of Aliases, ErrorLevel tables, environment variable

support, etc.).  Nonetheless, you can use it as a standalone program to

translate specific codes or display lengthy tables of data for various

USB related items.  Playing around with USBSUPT1 should provide lots of

interesting information to you (assuming you like to play around with

that sort of thing), but if you're not already pretty familiar with the

USB architecture and terminology and structure of these DOS USB driver

programs, it may confuse you more than it helps you.

Since the main purpose of USBSUPT1 is to translate USB codes (numbers)

into something understandable, you many times need to provide some sort

of number (or numbers) to USBSUPT1.  USBSUPT1 will generally accept

numbers in either decimal or hexadecimal format, with hexadecimal

numbers indicated by an "h" at the end.  For example, the following are

equivalent:

USBSUPT1 Country 10

USBSUPT1 Country Ah

Usually, if you don't provide a number at all, USBSUPT1 will display the

entire list of appropriate codes.  For example, to display the entire

list of Country codes (rather than a single one), you would do:

USBSUPT1 Country

There are certain options where USBSUPT1 does not translate a simple

code number, but rather translates an entire structure or descriptor or

set of descriptors that are stored somewhere in memory.  What must be

provided to USBSUPT1 in this case is a memory address, in Segment:Offset

format.  When providing a memory address, however, the numbers are

ALWAYS assumed to be in hexadecimal format, even if they are not

followed by an "h".  For example, the following are equivalent:

USBSUPT1 ControlPacket 1234:5678

USBSUPT1 ControlPacket 1234h:5678h

We will not list here all of the different types of things that USBSUPT1

will display.  To display the list, you can simply type "USBSUPT1" at

the command line with no options at all:

USBSUPT1

DOS USB DRIVER INTRODUCTION            Page 179 of 186

HIDSUPT1

��������

Version:     0.06

Prog Type:   Support

Source Code: ASM (A386 by Eric Isaacson)

Status:      Complete, a few minor tweaks required

Environment: None

Redirection: No

Purpose:     Translate various HID codes to words

TSR Memory:  0 kB (not a TSR)

HIDSUPT1 is very similar in concept to USBSUPT1, described directly

above (page 178).  However, while USBSUPT1 is directed at the "high

level" USB codes and structures, HIDSUPT1 is specifically directed at

Human Interface Devices or HID's.  HID's are actually a large class of

"Devices", which includes mice, keyboards, joysticks, and several other

items (pretty much anything that has buttons or controls or displays or

lights that a human being might interact with).

The USB HID specifications define two types of descriptors: Report

Descriptors and Physical Descriptors.  All HID's must contain at least

one Report Descriptor, which enumerates details about how many controls

and lights and other things that the Human Interface Device has.  The

Report Descriptor also describes the format of the data packets that are

sent back and forth between the computer and the HID (which bits and

bytes correspond to which buttons and keys and LED's and etc.).

While Report Descriptors are required, Physical Descriptors are optional

(I have personally never seen a Device with a Physical Descriptor).  The

intent of a Physical Descriptor is to "describe" in detail exactly where

each button or LED or whatever is located.  For instance, let's say a

joystick has 4 buttons on it.  In the Report Descriptor, the buttons

would simply be numbered from 1 to 4.  Without a Physical Descriptor,

the only way to find out which button is which is to experiment (with a

test program like THRUST described above on page 139, for example).  If

the joystick had a Physical Descriptor, it could tell you that button 1

is the one you press with your index finger, button 2 with your thumb,

etc.  HIDSUPT1 is able to decode both Report Descriptors and Physical

Descriptors.  However, because I have never actually seen a Device with

a "real" physical descriptor, I do not know if it actually works like it

should.  If anybody is ever able to test this for me, I would appreciate

some feedback.

DOS USB DRIVER INTRODUCTION            Page 180 of 186

Pretty much all of the cautions and warnings and directions that are

described above for USBSUPT1 also apply to HIDSUPT1 (numbers can be

either decimal or hexadecimal, not entering a number displays the whole

list, etc.).  Therefore, that information will not be repeated again

here.  Also, like USBSUPT1, you can simply run the program with no

option switches at all to display the list of exactly what the program

can show you, as follows:

HIDSUPT1

UNI2ASCI

��������

Version:     0.01

Prog Type:   Support

Source Code: ASM (A86 by Eric Isaacson)

Status:      In progress, several major items to address

Environment: None

Redirection: No

Purpose:     Convert Unicode characters to ASCII

TSR Memory:  0 kB (not a TSR)

UNI2ASCI is a program that converts Unicode strings into ASCII strings,

so that they can be viewed and printed in regular DOS programs.  When

the screen of the computer is in text mode (which is the case for most

DOS programs), each character that is viewed on the screen must be

selected from a set of 256 possibilities (8-bit characters).  The entire

set of 256 possibilities is called a Code Page.  The default Code Page

that DOS uses is the one for United States English, and is called Code

Page 437.  There are also several other Code Pages available for DOS,

and there are various DOS utilities related to changing and controlling

the Code Pages (NLSFUNC, COUNTRY, CHCP, etc.).

The ASCII character set is actually limited to 128 characters (7 bits).

For each Code Page, the first 128 (of 256 possible) characters

correspond directly to the ASCII characters.  These include various

control characters (escape, carriage return, form feed, etc.), upper-

and lower-case letters (a-z), numbers (0-9), and miscellaneous

punctuation (periods, commas, parentheses, etc.).  The second 128

characters of each Code Page are different for each Code Page, but

generally include box-drawing characters, accented letters and numbers

(characters with acute, grave, diereses, etc.), and miscellaneous other

characters.  For instance, Code Page 869 is specifically designed for

the Greek language, and contains the entire upper- and lower-case Greek

alphabet (Code Page 437 contains a few Greek characters, but not the

entire Greek alphabet).

DOS USB DRIVER INTRODUCTION            Page 181 of 186

The Unicode character set allows 20 bits for each character, for a total

of 1,048,576 possibilities.  This includes support for all kinds of

languages that there are no DOS Code Pages for, such as Arabic, Chinese,

and Cherokee.  20 bits is an unusual number in the programming world, so

the Unicode organization has come up with three different ways of

"encoding" Unicode characters: UTF-32, UTF-16, and UTF-8.  UTF-32

encodes a 20-bit Unicode character into a 32-bit space by simply

ignoring the upper 12 bits (making them 0).  UTF-16 encodes 20-bit

characters into 16 bit words by making certain Unicode characters

require 2 16-bit words instead of just one (that is, the length of a

single Unicode character is variable).  UTF-8 works in a similar

fashion, encoding a single 20-bit Unicode character as a variable-length

string of 8-bit bytes.

In addition to the UTF-8, UTF-16, and UTF-32 encodings, Unicode

characters can be encoded either Little-Endian or Big-Endian.  Endian-

ness comes about because of the way different processors store numbers.

Intel-compatible CPU's always store things Little-Endian, while certain

other CPU's (such as Motorola) store them Big-Endian.  With UTF-8,

Endian-ness does not matter.  With UTF-16 and UTF-32, there are "sub-

categories": UTF-16 LE, UTF-16 BE, UTF-32 LE, and UTF-32 BE.

In order to correctly decode a Unicode string, the decoding program (in

this case, UNI2ASCI) needs to know how the string is encoded (UTF-8,

UTF-16 LE, UTF-16 BE, UTF-32 LE, or UTF-32 BE), or it will not be able

to decode it properly.  It is usually not possible for a decoding

program to determine the encoding scheme automatically.  However, if a

decoding program knows the number of bits per character in the encoding

scheme (UTF-8, UTF-16, or UTF-32), the first character in the string can

(but does not need to) contain a special character that will tell the

decoding program whether it is Big-Endian or Little-Endian.

UNI2ASCI takes a Unicode string that is stored in memory somewhere,

decodes it, and displays it on the screen as well as it can.  That is,

it will take each Unicode character, and if it is similar to a character

that can be displayed on the screen using the current DOS Code Page,

UNI2ASCI will display it on the screen as that character.  If there is

no Code Page character that is similar to the Unicode character,

UNI2ASCI will display the character as "{U+####}", where the #### is the

hexadecimal number of the Unicode character ("U+####" is a standard way

of expressing Unicode characters in general).  If the Unicode character

is a Control Character, the hexadecimal number will be displayed along

with the common acronym used to describe the control (for example,

"{1B=ESC}" for the Escape character).  The Unicode character set is

divided into "regions", where certain characters (numbers) are Reserved,

Private, Illegal, etc.  When UNI2ASCI comes across one of these

"special" characters, it will display the hexadecimal number of the

character along with an appropriate description (e.g.,

"{F603=Private}").  You should review the Unicode specification if you

want further details on all of this.

DOS USB DRIVER INTRODUCTION            Page 182 of 186

There are direct relationships between some of the Unicode characters

and the Code Page characters.  However, there are also many (in some

cases, very many) Unicode characters that appear very similar to one of

the Code Page characters, even though they are technically not the same.

In such cases, UNI2ASCI will still display the character using the Code

Page character that it is similar to, rather than simply displaying a

"{U+####}".  Of course, similarity is in the eye of the beholder, and

characters that some people believe are "similar" other people may not.

I have gone through all of the Unicode version 5.0 characters and found

the Code Page characters from Code Page 437 (United States English) that

I personally believe they are similar to.  It took many, many days to go

through all of them, so things that I thought looked similar on one day

versus another will not necessarily be consistent (depending on how I

felt at the time, the weather, the lighting in the room, and who knows

what else).

In spite of the inconsistencies, I think UNI2ASCI is a pretty good start

to a "universal" Unicode-to-Code-Page conversion program for DOS.

However, UNI2ASCI currently only supports UTF-16 LE (which is how all

USB-related strings are encoded), and will only decode to Code Page 437

(the Code Page that I use).  Unless I find myself with a LOT of spare

time on my hands (hah!), I will probably not update UNI2ASCI in any

significant way, such as supporting other UTF encodings or Code Pages.

I would strongly encourage someone who is interested to take UNI2ASCI

and "run with it", updating it to include all sorts of new items, and

enabling it to be used in all sorts of ways other than USB.

To use UNI2ASCI, you must simply provide the memory address (hexadecimal

Segment:Offset format) where the Unicode string (UTF-16 LE) is located.

The string must end in Unicode character 0.  For example:

UNI2ASCI 1234:5678

The USBSUPT1 program (page 178 above) calls UNI2ASCI to display all

Unicode strings that are downloaded from the USB Devices.

DOS USB DRIVER INTRODUCTION            Page 183 of 186

VENDORID

��������

Version:     0.07

Prog Type:   Support

Source Code: ASM (A86 by Eric Isaacson)

Status:      Complete

Environment: None

Redirection: No

Purpose:     Translate PCI & USB Vendor ID codes to words

TSR Memory:  0 kB (not a TSR)

VENDORID is a program that searches for and displays the names of all of

the known PCI, PNP, or USB Vendor ID's.  PNP (Plug-N-Play) ID's are

three letter acronyms (for example, ICO = Intel Corporation) assigned by

Microsoft, and will receive limited discussion here since they do not

directly apply to USB devices.  In the PCI (Peripheral Component

Interface) and USB (Universal Serial Bus) architectures, each vendor

(manufacturer) registers with the appropriate organization, and receives

a Vendor ID (a number).  Every device that the Vendor manufacturers is

required to have the ID stored in the firmware of the device, so that

the device can be identified automatically (using software).  In

addition to the Vendor ID, PCI and USB Devices also contain a field for

the Device ID.  While the Vendor ID is assigned by the USB or PCI

organization (and requires a registration fee), the Device ID's are

assigned and maintained by the individual Vendor.

Even though the PCI and USB organizations are the only ones who can

officially assign Vendor ID's, some Vendors make up their own, "borrow"

ID's from each other, change their names, get taken over by other

companies, go out of business, etc.  Therefore, it is very difficult to

maintain an accurate list of names, since it is a "moving target".  On

top of that, the PCI and USB organizations are not always willing to

release their official information freely (even for something as simple

as the list of Vendor ID's).  Over the years, there have been several

volunteers that have gathered information about the Vendors and Devices

for both USB and PCI.  However, you cannot always believe all of the

information that you find, even if it comes directly from the

organization itself.

I have taken the information from several different locations (including

the USB and PCI organizations where possible), and combined them into

two sets data, one set for PCI and one for USB.  The VENDORID executable

file itself (VENDORID.COM) does not actually contain the data, but

simply allows you to look and search through the data that is in the

data files (VENDORID.USB, VENDORID.PCI, and VENDORID.PNP).  The data

files must be in the same directory as the executable file

(VENDORID.COM).  The data files are in a simple ASCII text format, so

you can easily look at and modify the data yourself with a simple text

editor if you are so inclined.

DOS USB DRIVER INTRODUCTION            Page 184 of 186

There are four basic functions that VENDORID provides for the data.  The

first function will tell you where the data came from (which web sites

the data came from, and the date that the data was downloaded).  For

example, to see where I got the PCI data from, you would do the

following:

VENDORID PCI Source

Similarly, to see where I got the PNP data from, you would do the

following:

VENDORID PNP Source

The second function that VENDORID provides allows you to discover the

name of the company associated with a specific Vendor ID (number or

acronym).  To do this, you must provide VENDORID with the Vendor ID

number (either in decimal or hexadecimal format), or three-letter

acronym (case-insensitive).  To indicate a hexadecimal number, you can

either precede the number with a "0x" (C-style), follow the number with

an "h", or both.  For example, the following lines will all provide the

same result:

VENDORID PCI 0x8086

VENDORID PCI   8086h

VENDORID PCI 0x8086h

VENDORID PCI 32902

The third function that VENDORID provides allows you to search for

Vendor ID's by knowing all or part of the vendor's company name.  To do

this, you provide a string to search for, surrounded by quotes (double

quotes, left single quotes, or right single quotes).  The search is

case-insensitive.  For example, the following lines will provide the

same result:

VENDORID USB "Intel"

VENDORID USB 'intel'

VENDORID USB 'INTEL'

The results in this case would of course include "Intel Corp", along

with several other vendors that included "intel" somewhere in their name

(e.g., "Intelligent", "Intellix", etc.).

If you want to see the entire list of USB vendors, you would do the

following:

VENDORID USB AllVendors

DOS USB DRIVER INTRODUCTION            Page 185 of 186

The VENDORID.USB, VENDORID.PCI, and VENDORID.PNP data files store the

information sorted by Vendor ID (number or acronym) rather than by

vendor name.  VENDORID does not offer any kind of sorting other than by

Vendor ID.  If you want the display sorted in some fashion other than by

Vendor ID (say, by vendor name), or want to filter it some other way

(say, do a case-sensitive vendor name search), you will need to use an

external filter.  The standard filters that come with DOS are FIND,

SORT, and MORE, and if you're very familiar with DOS you should already

understand them.  There are also several "after market" filters that you

can obtain.  For example, if you want to sort the entire PCI vendor list

by vendor name, and pause the screen after each page has been displayed,

you would do the following:

VENDORID PCI All | FIND "h " | SORT /+8 | MORE

The company names as stored and displayed in VENDORID are limited to 40

characters, so are almost always abbreviated in some fashion (Company is

always "Co", Incorporated is always "Inc", etc.).  If you edit the data

files yourself, and make a vendor name longer than 40 characters, the

name will be truncated to 40 characters when it is displayed by

VENDORID.COM.

The format of the data files is fairly self explanatory if you look at

them.  They can have blank lines and comment lines -- comment lines

always start with a semicolon (";").  The lines containing the data

sources start with "Source =".  A line containing a Vendor ID and name

will start with the Vendor ID (number or acronym), followed by at least

one space, followed by the vendor name.  VENDORID does not do any

testing to verify sort order, duplicates, etc., in the data file, but

rather simply assumes the data file is correct.

DOS USB DRIVER INTRODUCTION            Page 186 of 186