--------------------------- PCTIMER 1.3 --------------------------

<URL: ftp://ftp.coast.net/SimTel/msdos/c/pctime13.zip>

------------------------------------------------------------------

Program written by: Chih-Hao Tsai (c-tsai4@uiuc.edu)      11/26/95

------------------------------------------------------------------

This  program  is  FREE. Feel free to use, re-distribute or modify

it.  However, you should recognize that this program comes without

any  warranty.  It  is  recommended that you examine and test this

program  before  actually use it. If you find this program useful,

or if you find a bug, please send an e-mail to me to let me know.

------------------------------------------------------------------

1. PCTIMER

PCTIMER v1.3 is a set of millisecond resolution timing routines

written  in  C. It can be used in programs in which millisecond

(or higher) resolution timing is required.

History

11/26/95 Version 1.3

(1) There are now two sets of PCTIMER. The first (original) one

is  based  on IRQ 0 (int 8h), and the second (new) one is based

on IRQ 8 (int 70h).

(2)  MSC  source of PCTIMER has been removed. This is because I

do  not have a Microsoft C compiler now, and therefore will not

be able to maintain MSC programs.

1/29/95 Version 1.2

Revision  (and  correction)  of  gccint8.c. PCTIMER for GCC was

first  developed  under  gcc  2.6.0,  and it was found that the

following  instructions:  "r.x.cs  = rm_old_handler.rm_segment;

r.x.ip   =  rm_old_handler.rm_offset;  r.x.ss  =  r.x.sp  =  0;

_go32_dpmi_simulate_fcall_iret(&r);"     could     not     call

rm_old_handler.rm_segment:rm_old_handler.rm_offset   correctly.

Therefore,  my  solution  in PCTIMER 1.1 was to call f000:fea5,

which  is  the  default  handler  of int 8h in IBM PC and fully

compatible  BIOSes.  Since the bug has been fixed in gcc 2.6.3,

the  real  mode interrupt handler "rm_new8h" in gccint8.c could

now  be  revised. As a result, PCTIMER for GCC now requires GCC

version 2.6.3 or higher.

1/16/95 Version 1.1

(1)  In  version  1.0, gccint8.c only intercepts protected mode

int  8h,  which may cause loss of ticks when programs switch to

real  mode.  I  revised  gccint8.c that it would intercept both

protected mode and real mode int8h's. (I would like to thank DJ

Delorie,  Eli  Zaretskii,  and  Long  Doan  for  their  helpful

suggestions and discussions.)

(2) I added source for Turbo C/C++ compiler.

(3) The delay8h function had been slightly modified.

11/5/94 Version 1.0

The  first  freeware  release of PCTIMER. With source codes for

GNU C/C++ and Microsoft C/C++ compilers.

2. RATIONALE FOR WRITING AND USING PCTIMER

Although  the  timing  routines  of  most programming languages

always  report times to centisecond, the resolution of standard

timer  of  IBM-PCs is actually only 1/18.2 seconds. That is, if

you   use   the   standard  timing  routine  provided  by  your

programming  language,  the  precision  of  your measurement is

actually  55  milliseconds  with  a  plus/minus 27.5 seconds of

measurement  error!  I  wrote  PCTIMER  to provide IBM-PCs with

high-resolution timing ability.

3. FEATURES

(1) Precise: Basically, PCTIMER is designed to provide a timing

routine  with millisecond resolution. If such a resolution does

not  satisfy  you,  you  can  also  modify  it  to  get  higher

resolutions.  However, in most cases, millisecond resolution is

precise enough.

(2)  Stable:  PCTIMER/IRQ0  sets  new parameters to the 8254 IC

(Programmable  Interrupt Timer) to get high frequency of SYSTEM

TIMER  Interrupt (IRQ 0, or int 8h) whose default is 18.2 times

per  second.  However,  many hardware routines depend on IRQ 0.

Abnormal  IRQ  0  frequency  may  cause the system unstable. To

maintain  the  PC's  stability,  PCTIMER intercepts the int 8h,

only  calls  the original int 8h at the frequency of 18.2 times

per  second.  Consequently,  high-resolution  timing and system

stability  are  simultaneously achieved. PCTIMER/IRQ8 makes use

of  the  default 1024 Hz IRQ 8 (int 70h) generated by Real Time

Clock  (RTC, or CMOS) to do the timing job. Since the interrupt

frequency   is  default  and  the  frequency  of  system  timer

interrupt  (int  8h)  is  not changed, the machine can be quite

stable.

(3)  Flexible:  Originally  I  wrote  PCTIMER to do millisecond

resolution  timing.  If millisecond precision in unnecessary to

you,  you  can  also  set  PCTIMER  to  do  centisecond timing.

Resolution  higher  than  millisecond is also possible, but you

have  to  modify  a  few  parts  of the program by yourself. It

should  be  an  easy task. (Only PCTIMER/IRQ0 can adjust timing

resolution. The resolution of PCTIMER/IRQ8 is fixed.)

(4) Portable: Most public-domain timing routines are written in

assembly  codes, or in C codes with in-line assembly codes. The

assembly  codes  make those routine difficult to understand and

use  if  one  does  not  have  good  background of the assembly

language.  And  the  format of in-line assembly varies from one

compiler  to  another. Worst of all, several C compilers do not

support in-line assembly. PCTIMER does not have these problems,

because  all  functions  are  written  in C codes. There are no

assembly codes at all.

4. SYSTEM AND COMPILER REQUIREMENTS

(1)  PCTIMER  is  a  tiny  timing  routine  that  does not have

specific memory and CPU speed limitations. The only requirement

is  that  it  only  runs on IBM-AT compatible PCs. An XT is not

appropriate   because   an   XT  does  not  have  an  CMOS  IC.

PCTIMER/IRQ0  needs the CMOS to obtain correct system time, and

PCTIMER/IRQ8 depends on the CMOS to do millisecond timing.

(2) PCTIMER comes with two versions, one for GNU C/C++, and one

for Turbo C/C++. The GCC version was developed under GCC 2.6.3,

and  the  Turbo  C version was developed under Turbo C/C++ 2.0.

There  should  be  no  problems  if  you  are using a different

version  of Turbo C. However, the version of GCC *does* matter.

Please do not use GCC older than 2.6.3.

5. USING PCTIMER/IRQ0

(1)  At  the beginning of your program, include the header file

of PCTIMER:

#include "gccint8.h"

/* "tcint8.h" for Turbo C */

(2)  Declare  three  unsigned  long  integers (of course, their

names are arbitrary):

unsigned long int tick_1, tick_2, elapsedtime;

(3)  In  your program, use the following function and parameter

to initiate the timer:

init8h(1000); /* init8h(100) for centisecond resolution */

ticks_8h = 0; /* reset the main counter */

(4) Timing

tick_1 = ticks_8h;

/* .... do some stuff here .... */

tick_2 = ticks_8h;

elapsedtime = time8h(tick_1, tick_2);

/*  The  function  "time8h" always returns millisecond unit

digits.  That  is,  if you use init8h(1000), then the value

returned  by  time8h  is  exactly  the time elapsed between

tick_1  and  tick_2.  However,  if you use init8h(100), the

value   returned   by  time8h  is  still  in  the  unit  of

millisecond.  The  difference is that the value returned by

time8h  in  the  init8h(1000)  condition is measured in the

precision  of  one millisecond, while the value returned by

time8h  in  the  init8h(100)  condition  is measured in the

precision of 10 milliseconds. */

(5) How to turn off the timer:

quit8h();

/*  Be  sure  to  turn  off  the  timer before your program

terminates.  Also  turn  off the timer before you set a new

parameter for init8h. */

(6) delay8h and sound8h

/*  The  "delay8h" is a high-resolution delay function. The

parameter  is  always in the unit of millisecond, no matter

what you put in the "init8h". However, the precision of the

delay  duration  depends on the parameter of "init8h". Here

is an example for a 150 milliseconds delay: */

delay8h(150);

/* the "sound8h" takes advantage of "delay8h" to generate a

sound  with  a  specific duration. Here is an example for a

800 Hz sound with 200 ms duration: */

sound(800, 200);

6. USING PCTIMER/IRQ8

The  procedures  are  mostly  the same as those in the previous

section. The differences are:

(1) The main counter is ticks_70h.

(2) Use init70h() without any parameters to initiate timer, and

use quit70h() to turn off the timer.

(3) Use time70h(tick_1, tick_2) to get elapsed time.

(4) Use delay70h(duration) and sound70h(frequency, duration) to

generate a delay or sound.

7. WHY TWO VERSIONS?

The method that PCTIMER/IRQ0 employed is to speed up the system

timer  interrupt (IRQ 0), which is a kind of standard technique

of  doing  high resolution  timing.  This  method  is  the most

popular  one  because  8253/8254  has  been a standard hardware

component since the age of the first IBM PC.

The  method  that PCTIMER/IRQ8 employed is not as well-known as

does  the  speed  up system timer interrupt method. Part of the

reason  is  because  the  CMOS has not been a standard hardware

component  until IBM PC/AT. One of the advantages of using CMOS

to  do  millisecond  resolution timing is that the CMOS it self

can  generate  1024  interrupts  (IRQ 8) per second. Therefore,

there is no need to speed up the system timer.

The above two methods are the two most general methods of doing

high  resolution  timing  on IBM PCs. I am not going to explain

the details here. Readers are referred to this good document:

Heidenstrom,  K.  (1995). FAQ / Application Note: Timing on the

PC under DOS. Available

ftp://oak.oakland.edu/SimTel/msdos/info/pctim002.zip.

When  I  was  developing PCTIMER, I did not know this document.

Therefore,  my  implementation may be very different from those

in  Heidenstrom's  FAQ. However, the basic principles should be

the same.

8. LIMITATION

PCTIMER/IRQ0  uses  an  unsigned  long  integer  to  count  the

cumulative  frequency of int 8h interrupts. Assume that you set

init8h(1000), ticks_8h = 0, PCTIMER will work 4294967296/1000 =

4294967.296  seconds, or 1193.05 hours without encountering the

problem  of  overflow. I hope the 1193.05 hours would be pretty

enough. If you do need to continuously use PCTIMER for a period

longer  than  1193.05 hours, just reset the ticks_8h to zero at

appropriate  location  of  your  program.  PCTIMER/IRQ8  has  a

similar limitation.

9. FILES

pctimer.doc    This file

gccint8.h      PCTIMER/IRQ0: header file for GCC

gccint8.c      PCTIMER/IRQ0: source code for GCC

tcint8.h      PCTIMER/IRQ0: header file for Turbo C

tcint8.c      PCTIMER/IRQ0: source code for Turbo C

test8.c      PCTIMER/IRQ0 test program: source

test8.exe    PCTIMER/IRQ0 test program: executable

gccint70.h      PCTIMER/IRQ8: header file for GCC

gccint70.c      PCTIMER/IRQ8: source code for GCC

tcint70.h      PCTIMER/IRQ8: header file for Turbo C

tcint70.c      PCTIMER/IRQ8: source code for Turbo C

test70.c      PCTIMER/IRQ8 test program: source

test70.exe    PCTIMER/IRQ8 test program: executable

------------------------------------------------------------------