What this document is all about

-------------------------------

This is the documentation for the YAPCBR, Yet Another  PC

BRidge.  This  software  has  been developed by the Department of

Computer Science, IIT, Bombay, India.

Copyright

---------

For the copyright, see COPYING.DOC.

General Information

-------------------

This  bridge  belongs  to  the  class  of  Self  Learning

Transparent bridges.  You can use any Ethernet cards (or a mix of

cards from different vendors) for this bridge. All  you  need  to

have  is  the  appropriate packet drivers for these cards. Packet

drivers for most of the ethernet cards are in public  domain  and

can  be obtained from crynwr.com. This bridge can be set up using

a PC or AT (hard  disk  is  not  necessary,  a  floppy  drive  is

sufficient)  running  MSDOS  ver  3.0  or  higher.  This is not a

spanning tree bridge and hence cannot  detect  loops  in  network

paths.

Installation Procedure:

-----------------------

1. Put the ethernet cards in the PC expansion slots  with  proper

jumper/switch settings.

2. You can have up to four network segments  (so  correspondingly

four ethernet cards). Load the packet drivers for these cards. Be

careful with IRQs, I/O Base address and Memory  Base  address  if

any.   See  to it that the addresses and interrupts you have used

do not clash.

Choose  the  software  interrupts  from   0x60   onwards,   these

interrupts  have  to  be consecutive. e.g., 0x60 and 0x61 for two

cards, 0x60, 0x61, 0x62 for a three card bridge, etc.

3. The bridge maintains the  table  of  node  addresses  and  the

corresponding  segments.  This  table is built up dynamically. By

default this table is cleared every 60 minutes.  If  you  have  a

network  where  the cards are shifted from one segment to another

frequently, then set this parameter to a small value (minimum one

minute) and vice versa (the maximum is 1000 minutes).

Usage

------

A:> yapcbr <no of interfaces> [Hash Table Flush Time In Minutes]

e.g. A batch file in our department looks like this

rem   A sample batch file which loads the packet drivers and starts the bridge

rem   Load the packet driver for the network interfaces

de100pd 0x60 3 0xd000 0x300

de100pd 0x61 2 0xd200 0x320

de100pd 0x62 5 0xc000 0x340

rem This bridge has three interfaces, and hash table flush time is 40 mins

yapcbr 3 40

Runtime Commands available:

---------------------------

S : Packet Statistics

C : Clear the hash table

R : Reset this bridge

Compiling :

-----------

This software has been compiled using TurboC++ Ver1.0. It

has the following files. Select large model for compiling.

BRIDGE   BAT       305  /* an example batch file */

BRIDGE   DOC      3814  /* this very file */

COPYING  DOC     12375  /* GNU's General Public License */

TURBOC   CFG        26

TCCONFIG TC       1808  /* config files needed for TurboC */

MAIN     C        9252  /* the main program */

YAPCBR   H        5630  /* the header file */

YAPCBR   C        8298  /* the packet driver - C interface */

YAPCBR   EXE     35025  /* the standalone executable file */

PKVEC    ASM      6604  /* pkt driver interrupt handler */

TIMER    C        1810  /* timer interrupt handler */

YAPCBR   PRJ      6572  /* tc++ project file for this software */

SCREEN   C        3946  /* some screen management routines */

Acknowledgments

----------------

We thank all our colleagues, especially K.V. Ramani  for  helping

us  out  with  the  code and Vinod Kulkarni for testing.  We also

thank Prof. S.S.S.P.Rao for initial inspiration.

Final Comment

-------------

No flames please. This may not be the fastest  and  the  best  PC

bridge available but it works :-) We want more and more people to

test this software, and send  us  bug  reports  (along  with  bug

fixes,  if  possible),  suggestions  and  more  importantly, some

PERFORMANCE/BENCHMARK figures.

Our Postal address is

Nitin Kaulavkar, <nitin@cse.iitb.ernet.in>

Department of Computer Science and Engg,

Indian Institute of Technology,

Powai, Bombay, 400 076

India.

Our email address is

Internet : yapcbr@cse.iitb.ernet.in

Thanks and enjoy. :-) :-)